| Permit Number: | PAN-011755 |
| --- | --- |
| Applicant name: | Mr Anthony Deakins |
| Site Address: | Mill Race Lodges, Llangunllo, Knighton, LD7 1SP |
| Discharge Point NGR: | SO 20895 71576 |
| Respond by date: | 18/11/2020 (10 working days) |
| Permitting Officer: | Alex Garratt |

Brief Description of proposal:

The operator will be discharging primary treated sewage from a septic tank into a drainage field infiltration system, both of which meet the current British standards. The effluent is derived from 3 domestic properties and will not exceed 2.7 cubic metres per day, the receiving environment is ground water. This is an existing discharge, which was started before 2010 and there is no change in the treatment level since it was installed in 2006.

Habitats Screening Report – Designations or protected species/habitats:

The HSR brought up one protected species migratory route, Atlantic Salmon.

It also raised one SSSI being the river Lugg, for biological reasons.

We will be completing an appendix 4 assessment for the SSSI as part of the determination process.

Please find the habitats screening report here on the DMS.

Risk Assessment Results (modelling or L1 Risk Assessment):

No river modelling will be carried out for this application because the discharge is below 5 cubic metres.

Miscellaneous:

.

NRM Responses:

Environment Management comments: No concerns, the site is outside of the sewage area and is 60m from the nearest surface water so going to ground seems like the best option. Going to ground also has the advantage of reducing loading on the river Lugg which is identified as having elevated nurturance.

Officers name: Tim Arlett
Date: 5/11/2020

Biodiversity comments:

Officers name:
Date:

Fisheries comments: no concerns as its going to ground

Officers name: tim arlett
Date: 5/11/2020

Conservation/Ecology Comments:

Officers name:
Date:

Internal Consultation Responses from other NRW Teams:

Water Quality Planning:

N/A

Officers name:

Date:

Geoscience Response:

Existing discharge, outside sewered area (140 m from edge of existing drainage field as submitted).

Level 1 risk assessment amended.  Mapping and BGS borehole record in same geology 150 m distant used to support assessment.

Risk acceptable. No objection to granting permit

Officers name: Alan Price
Date: 30/11/2020

Flood Risk Management:

N/A

Officers name:
Date:

| External Consultations sent to: | Yes/No |
| --- | --- |
| National Parks |  |
| Water Company regarding Drinking Water Protected Zone / SPZ1 |  |
| Environmental Heath |  |

Any Permitting Responses:

Date: