| Permit Number: | PAN-011211 |
| --- | --- |
| Applicant name: | Mr Robert Wyn & Rachel Elizabeth Davies |
| Site Address: | Yscoedreddfyn Farm, Trecastle, Brecon, LD3 8RH |
| Discharge Point NGR: | SN 84170 31712 |
| Respond by date: | 30/09/2020 (10 working days) |
| Permitting Officer: | Thomas Dauncey |

Brief Description of proposal:

The applicant proposes a new discharge of sheep dip to their 2.59 hectare field.

The proposed discharge will be 1 part used working strength dip with 4 parts slurry or water for a total disposal of 5m3/day. Discharge is planned for twice a year; once in August and again in October. A vacuum tanker will discharge with an application rate of 20m3/hectare.

Calculated disposal area: 0.5ha.

The plans/maps provided with the application were unclear but the site pan here shows the confirmed true extent of the field. The applicant would like to discharge in the northern section of this field which has less than 11o gradient.

Habitats Screening Report – Designations or protected species/habitats:

The Risk Screening Report detected a SSSI approximately 650 metres away.

I have not completed an Appendix 4 because the SSSI is a geological site that does not consider the proposed activity to be a risk to its management objectives.

Risk Assessment Results (modelling or L1 Risk Assessment):

The score on the L1 Assessment was 48. Although this is below the 50 threshold for Geoscience’s input, the score is very close and have therefore asked for their consultation.

Miscellaneous:

NRM Responses:

Environment Management comments:

The land (SN 8431 1673) is noted to be improved grassland used for grazing. Please note the southern section of this land appears to have steep ground as per MyMap and the map does not identify the direction of the slope. Additionally, the northern end of the parcel of land is a woodland area. Spreading plots have not been identified on this section of land. The competent person must ensure that COGAP is always followed, checking that the land is in a suitable state to receive used dip and that the conditions are suitable for spreading.

The nearby watercourses and spring are a suitable distance from the allocated parcel of land.

I note that Part B2, 2c is selected as a waste operation rather than a water discharge activity. Additionally, some other sections have been filled out that are not required.

Officers name: Ffion Roberts 
Date: 21/09/2020

Biodiversity comments:

N/A

Officers name:
Date:

Fisheries comments:

N/A

Officers name:
Date:

Conservation/Ecology Comments:

N/A

Officers name:
Date:

Internal Consultation Responses from other NRW Teams:

Water Quality Planning:

N/A

Officers name:

Date:

Geoscience Response:

Some changes made to the L1 assessment form (in red text), summarised as follows:

- Distance to receptors. measured from edge of field identified.  GW receptor taken as spring identified in application form.

- Aquifer/unsaturated zone.  based on mapping available.  Secondary bedrock aquifer with no superficial deposits. Worst case scenario used for unsaturated zone as no info on depth to groundwater available.

- Number of applications changed to 1.  We can only allow one discharge of spent dip on any given area of land per year.  Two disposal events per year requires two distinct areas (which can be adjacent to each other in the same field, but need to make the distinction). Please make this clear in the permit.

Revised score of 49 is acceptable.

The northern part of the field identified appears relatively flatter and further away from receptors, which is preferable to reduce risk to the water environment.  There appears to be enough room to accommodate two areas, one for each of the two disposals per year.

Officers name: Alan Price
Date: 29/09/2020

Flood Risk Management:

N/A

Officers name:
Date:

| External Consultations sent to: | Yes/No |
| --- | --- |
| National Parks | No |
| Water Company regarding Drinking Water Protected Zone / SPZ1 | No |
| Environmental Heath | No |

Any Permitting Responses:

Date: