PAN-023087  My Map Report:

Application map

Deployment Boundaries

Surface Water Courses

All watercourses shown with buffers on application maps

Lagoon storage see RFI

Groundwater source protection zones

None

Any well, spring (or issue) or borehole used / not used for the supply of water for human consumption including from a private water supply or supplies within 50m.

Spring within 8302A shown with buffer on application map.

East of 2319B, issue shown on application map but pond not shown on application map. Potentially insufficient buffer RFI.

Issue N of 2A shown but insufficient buffer. Issue and pond SE of 1A shown on application map but insufficient buffer RFI

Any well, spring (or issue) or borehole used / not used for the supply of water for human consumption including from a private water supply or supplies within 250m.

Pond NE of 8302A, Issue SE of 8302 shown on application map.

Multiple springs, issues, wells and drains all shown on application map with the exception of the Well N of 5044A RFI

Wells NW and W of 2A not shown on application map RFI

Issue, collects and 3x ponds/lagoons within range, shown on application maps

European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m

None

Other protected sites within 500m:

- Protected Species-

- Protected Habitats-

- Scheduled Ancient Monument-

- Local Nature Reserve-

- National Nature Reserve-

- Ancient Woodlands-

- Local Wildlife Sites (SINC)-

- National Park-

- Nitrate Vulnerable Zone (NVZ)- ALL OF WALES

- Groundwater Safeguard Zone-

- Hedges-

- Ditches-

Lowland dry acid grassland and Lowland calcareous grassland NE of 8302B, not shown on application Map

LWS (SINC) Wern-hennaidd to E of fields 5044A and 5044B. Lowland dry acid grassland and Lowland calcareous grassland within field and N of 5044A. ancient woodland NE and E of 5044A none acknowledged on application maps

LWS Cae Coed N of 2A, ancient woodland W, N and NE of 2A

Purple moor grass and rush pastures E of 1A

Public rights of way

All PRoW shown with no spread areas on application maps

Distance from Main Farm Address

Farm Address

differs from application.

CEFN Y WERN,LLANELIDAN,RUTHIN,LL15 2RN shown in MyMap RFI

Slope Map

8302A is predominantly 0-7, however; the W corner is 7-12 and 12+ sloping away from the field and towards the watercourse issuing from within the field to the SW. Not identified on appliciation maps. Risk of run off is high. RFI

8302B is predominantly 0-7. With some 7-12. Risk of run off low.

5044A is predominantly 12+. The slope grades to the NE towards two watercourses. Risk of run off is high and not fully identified on application maps. Remove from the deployment. RFI

5044B, central part of field slopes from 7-12 to 12+ towards the centre of the field risk of run of is low. Not indicated on application maps as no spread area. Adding no spread buffers would eliminate all but a fraction of the field. Remove from the deployment. RFI

1929A, the majority of this field is 12+ and 7-12 grading N toward a watercourse. Not shown on application maps as no spread area. Run off risk med. Adding no spread buffers would eliminate all but a fraction of the field. Remove from the deployment. RFI

1929B, the N sector of the field slopes 12+ grading N to a watercourse flowing W-E. and other area 7-12. Risk of run off is high without suitable no spread buffers. Not indicated on application maps as no spread area. Adding no spread buffers would half the size of the field.field to potentially be removed from the deployment. A 10metre buffer is in place for the watercourse to the N. RFI

2391A and B, mainly 0-7 with minor 7-12, run off risk low

1A and 2A, 0-7

END