| Permit Number: | BP0366401 (PAN-003982) |
| --- | --- |
| Applicant name: | Dwr Cymru Cyfyngedig |
| Site Address: | Llys Y Fran Dam STW, Llys Y Fran, Clarbeston Road |
| Discharge Point NGR: | SN 03787 24195 |
| Respond by date: | 01/02/2019 (10 working days) |
| Permitting Officer: | Elliot Burge |

Brief Description of proposal:

We have received an application to vary an existing permit at Llys Y Fran Dam Sewage Treatment Works. The variation is to increase the daily discharge limit from a DWF of 9.9 to 14.5, this is to accommodate for growth in the catchment (expansion of existing visitor centre and a new outdoor activity centre) and to move the discharge point from SN 03963 24241 to SN 03787 24195.

The proposed solution consists of a Rotating Biological Contractor Package Treatment Plant to receive and treat all flows from the catchment. The system will be designed with sufficient capacity to ensure that no emergency discharges will take place.

Further details on the proposal can be found in the solutions document on the DMS here.

Habitats Screening Report – Designations or protected species/habitats:

The habitats screening report can be found on the DMS here.

There are many SAC and SSSI designations due to the large screening distance used for this application. A Form 1 and Appendix 4 will be completed and sent to the relevant Conservation teams for consultation.

There are also Brown Trout and Salmon within 500m of the proposed discharge point.

Risk Assessment Results (modelling or L1 Risk Assessment):

Water Quality Planning has undertaking the modelling for the site, the modelling report can be found on the DMS here.

The limits proposed are 40:60:20 (BOD:Suspended Solids:Ammonia), this allows 10% deterioration.

Miscellaneous:

Currently the treatment works receives and treats flows for the catchment and the visitor centre has a separate discharge via a septic tank and soakaway. The proposal to increase the flows at the treatment works is based on the fact the current septic tank and soakaway at the visitor centre will not be able to handle the increased flows from the outdoor activity centre, therefore, the variation will mean the works is permitted to treat and discharge the sewage from the catchment and the expanded visitor centre.

NRM Responses:

Environment Management comments:

No adverse comments providing the modelling is acceptable

Officers name: Rod Thomas
Date: 21 Jan 2019

Biodiversity comments:

Officers name:
Date:

Fisheries comments:

Officers name:
Date:

Conservation/Ecology Comments:

Officers name:
Date:

Internal Consultation Responses from other NRW Teams:

Water Quality Planning:

N/A

Officers name:

Date:

Geoscience Response:

N/A

Officers name:
Date:

Flood Risk Management:

Officers name:
Date:

| External Consultations sent to: | Yes/No |
| --- | --- |
| National Parks | N |
| Water Company regarding Drinking Water Protected Zone / SPZ1 | N |
| Environmental Heath | N |

Any Permitting Responses:

Date: