[image: image3.jpg]Cyfoeth
Naturiol
Cymru

Natural
Resources
Wales





Permit with introductory note

The Environmental Permitting (England & Wales) Regulations 2016
Mona Precast (Anglesey) Ltd
Site drainage at 

Mona Precast (Anglesey) Ltd
Gaerwen Industrial Estate
Isle of Anglesey
LL60 6HR
Permit number

EPR/DB3093ZD
Site Drainage at 
Mona Precast (Anglesey) Ltd
Permit Number EPR/DB3093ZD
Introductory note

This introductory note does not form a part of the permit

The main features of the permit are as follows.  
This permit regulates an existing discharge of storm/rainwater from a concrete yard to the edge of the site during periods of heavy rain. The applicant has installed a new treatment system to treat the discharge.
The discharge will be treated by drawing water from the collection ditch by the existing pump. It will then be fed by into a flocculation tank with a flocculant: Anionic polyacrimide and coagulant: poly Aluminium Chloride to separate the solids and liquids. The amount of flocculant and coagulant used will be pH dependent but will not exceed 10mg/l. This mixture will then pass into the settlement tank where solids settle out and the clear supernatant passes through the storage tanks for use in the concrete process. When there is overflow of the storage tanks due to heavy rain conditions it flows back into the ditch. As the contaminated water will have already been put though the Waste Water Treatment system, any overflow will be rainwater only with no chemical addition. The maximum discharge rate will be 2.8 litres/second.
The status log of the permit sets out the permitting history, including any changes to the permit reference number.
	Status log of the permit

	Description
	Date
	Comments

	Application PAN-019716
	Duly made 28/03/2023
	Application for discharge of trade effluent consisting of site drainage.

	Additional information received
	14/06/2023
	Schedule 5 response: additional fee, management plan, site plan, flow rate and details of treatment.

	Permit determined
	25/10/2023
	Permit issued to Mona Precast Limited


End of introductory note

	Permit


The Environmental Permitting (England and Wales) Regulations 2016
Permit number 

EPR/DB3093ZD
The Natural Resources Body for Wales (“Natural Resources Wales”) authorises, under regulation 13 of the Environmental Permitting (England and Wales) Regulations 2016 
Mona Precast (Anglesey) Ltd (“the operator”),

of

Mona Precast (Anglesey) Ltd
Gaerwen Industrial Estate
Gaerwen

Anglesey

LL60 6HR
company registration number 00926654
to operate a water discharge activity at

Mona Precast (Anglesey) Ltd
Gaerwen Industrial Estate
Gaerwen

Anglesey

LL60 6HR
to the extent authorised by and subject to the conditions of this permit.

	Name
	Date

	Emma Jones
	25/10/2023


Authorised on behalf of Natural Resources Wales
Conditions 

1 Management
1.1 General management

1.1.1 The operator shall manage and operate the activities:

(a) in accordance with a written management system that identifies and minimises risks of pollution, including those arising from operations, maintenance, accidents, incidents, non-conformances and those drawn to the attention of the operator as a result of complaints; and

(b) using sufficient competent persons and resources.
1.1.2 Records demonstrating compliance with condition 1.1.1 shall be maintained. 

1.1.3 Any person having duties that are or may be affected by the matters set out in this permit shall have convenient access to a copy of it kept at or near the place where those duties are carried out.

2
Operations
2.1
Permitted activities

2.1.1
The operator is only authorised to carry out the activities specified in schedule 1 table S1.1 (the “activities”). 

2.2
The site 

2.2.1
The activities shall not extend beyond the site, being the land shown edged in green and the discharge shall be made at the point marked on the site plan at schedule 7 to this permit and as listed in table S3.2 (discharge points). 
2.3
Operating techniques

2.3.1
(a)
The activities shall, subject to the conditions of this permit, be operated using the techniques and in the manner described in the documentation specified in schedule 1, table S1.2, unless otherwise agreed in writing by Natural Resources Wales.


(b)
If notified by Natural Resources Wales that the activities are giving rise to pollution, the operator shall submit to Natural Resources Wales for approval within the period specified, a revision of any plan or other documentation (“plan”) specified in schedule 1, table S1.2 or otherwise required under this permit which identifies and minimises the risks of pollution relevant to that plan, and shall implement the approved revised plan in place of the original from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.
3
Emissions and monitoring
3.1
Emissions to water

3.1.1
There shall be no point source emissions to water except from the sources and emission points listed in schedule 3.

3.1.2
The limits given in schedule 3 shall not be exceeded.

3.2 Emissions of substances not controlled by emission limits

3.2.1 Emissions of substances not controlled by emission limits (excluding odour) shall not cause pollution. The operator shall not be taken to have breached this condition if appropriate measures, including, but not limited to, those specified in any approved emissions management plan, have been taken to prevent or where that is not practicable, to minimise, those emissions.

3.2.2
The operator shall:

(a)
if notified by Natural Resources Wales that the activities are giving rise to pollution, submit to Natural Resources Wales for approval within the period specified, an emissions management plan which identifies and minimises the risks of pollution from emissions of substances not controlled by emission limits;

(b)
implement the approved emissions management plan, from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

3.2.3
All liquids in containers, whose emission to water or land could cause pollution, shall be provided with secondary containment, unless the operator has used other appropriate measures to prevent or where that is not practicable, to minimise, leakage and spillage from the primary container.

3.3
Monitoring 

3.3.1
Permanent means of access shall be provided to enable sampling/monitoring to be carried out in relation to the emission points specified in schedule 3 tables S3.1, S3.2 and S3.3 unless otherwise agreed in writing by Natural Resources Wales.
4
Information

4.1
Records
4.1.1 All records required to be made by this permit shall:

(a) be legible;

(b) be made as soon as reasonably practicable;

(c) if amended, be amended in such a way that the original and any subsequent amendments remain legible, or are capable of retrieval; and

(d) be retained, unless otherwise agreed in writing by Natural Resources Wales, for at least 6 years from the date when the records were made.
4.1.2
The operator shall keep on site all records, plans and the management system required to be maintained by this permit, unless otherwise agreed in writing by Natural Resources Wales. 

4.2
Reporting
4.2.1
The operator shall send all reports and notifications required by the permit to Natural Resources Wales using the contact details supplied in writing by Natural Resources Wales.

4.3
Notifications
4.3.1
Natural Resources Wales shall be notified without delay following the detection of:

(a) any malfunction, breakdown or failure of equipment or techniques, accident, or  emission of a substance not controlled by an emission limit which has caused, is causing or may cause significant pollution;

(b) the breach of a limit specified in the permit; or 

(c) any significant adverse environmental effects.  

4.3.2
Any information provided under condition 4.3.1 shall be confirmed by sending the information listed in schedule 5 to this permit within the time period specified in that schedule.
4.3.3
Where Natural Resources Wales has requested in writing that it shall be notified when the operator is to undertake monitoring and/or spot sampling, the operator shall inform Natural Resources Wales when the relevant monitoring and/or spot sampling is to take place. The operator shall provide this information to Natural Resources Wales at least 14 days before the date the monitoring is to be undertaken.

4.3.4
Natural Resources Wales shall be notified within 14 days of the occurrence of the following matters, except where such disclosure is prohibited by Stock Exchange rules: 

Where the operator is a registered company:
(a) any change in the operator’s trading name, registered name or registered office address; and

(b)
any steps taken with a view to the operator going into administration, entering into a company voluntary arrangement or being wound up.

Where the operator is a corporate body other than a registered company:
(a) any change in the operator’s name or address; and

(b) any steps taken with a view to the dissolution of the operator.

4.3.5
Where the operator proposes to make a change in the nature or functioning, or an extension of the activities, which may have consequences for the environment and the change is not otherwise the subject of an application for approval under the Regulations or this permit:

(a)
Natural Resources Wales shall be notified at least 14 days before making the change; and


(b)
the notification shall contain a description of the proposed change in operation. 

4.4
Interpretation
4.4.1
In this permit the expressions listed in schedule 6 shall have the meaning given in that schedule.

4.4.2
In this permit references to reports and notifications mean written reports and notifications, except where reference is made to notification being made “without delay”, in which case it may be provided by telephone. 

Schedule 1 - Operations 
	Table S1.1 Activities

	Description of activity
	Limits of specified activity

	Discharge of trade effluent consisting of site drainage via Outlet 1
	N/A


	Table S1.2 Operating techniques

	Description of documentation
	Parts
	Date Received

	“PAN-019716 Schedule 5 notice reply management system”
	All
	07/06/2023


Schedule 2 - Waste types, raw materials and fuels

Wastes are not accepted as part of the permitted activities and there are no restrictions on raw materials or fuels under this schedule.

Schedule 3 – Emissions and monitoring

	Table S3.1 Point Source emissions to water (other than sewer) – emission limits and monitoring requirements

	Discharge source and discharge point ref. & location
	Parameter  
	Limit (including unit)
	Reference Period
	Limit of effective range
	Monitoring frequency
	Compliance Statistic

	Trade effluent consisting of site drainage via Outlet 1
	Maximum rate of discharge
	2.8  litres per second
	Instantaneous (spot sample)
	N/A
	N/A
	Maximum

	
	Suspended solids (measured after drying at 105o C)
	40 mg/l
	Instantaneous (spot sample)
	N/A
	N/A
	Maximum 

	
	pH
	6 to 9
	Instantaneous (spot sample)
	N/A
	N/A
	Minimum and maximum

	
	Total Aluminium as Al 
	1,000 µg/l 
	Instantaneous (spot sample)
	N/A
	N/A
	Maximum

	
	Visible oil or grease
	No significant trace present
	Instantaneous (spot sample)
	N/A
	N/A
	No significant trace


	Table S3.2 Discharge points

	Effluent Name
	Discharge Point
	Discharge point NGR
	Receiving water



	Trade effluent consisting of site drainage 
	Outlet 1
	SH 47366 71625
	Ditch leading to unnamed tributary of Afon Cefni


	Table S3.3 Monitoring points

	Effluent and discharge point
	Monitoring type
	Monitoring point NGR
	Monitoring point reference

	Trade effluent consisting of site drainage via Outlet 1
	Effluent sample point
	SH 47488 71542
	9B Effluent sample point


Schedule 4 – Reporting

There is no reporting under this schedule.

Schedule 5 - Notification 

These pages outline the information that the operator must provide. 

Units of measurement used in information supplied under Part A and B requirements shall be appropriate to the circumstances of the emission. Where appropriate, a comparison should be made of actual emissions and authorised emission limits.

If any information is considered commercially confidential, it should be separated from non-confidential information, supplied on a separate sheet and accompanied by an application for commercial confidentiality under the provisions of the EP Regulations.

Part A 
	Permit Number
	EPR/DB3093ZD

	Name of operator
	

	Location of Facility
	Gaerwen Industrial Estate, Isle of Anglesey

	Time and date of the detection 
	


	(a) Notification requirements for any malfunction, breakdown or failure of equipment or techniques, accident, or emission of a substance not controlled by an emission limit which has caused, is causing or may cause significant pollution

	To be notified within 24 hours of detection

	Date and time of the event
	

	Reference or description of the location of the event 
	

	Description of where any release into the environment took place
	

	Substances(s) potentially released
	

	Best estimate of the quantity or rate of  release of substances
	

	Measures taken, or intended to be taken, to stop any emission
	

	Description of the failure or accident.
	


	(b) Notification requirements for the breach of a limit

	To be notified within 24 hours of detection unless otherwise specified below

	Emission point reference/ source
	

	Parameter(s)
	

	Limit
	

	Measured value and uncertainty
	

	Date and time of monitoring
	

	Measures taken, or intended to be taken, to stop the emission
	


	Time periods for notification following detection of a breach of a limit

	Parameter
	Notification period

	
	

	
	

	
	


	(c) Notification requirements for the detection of any significant adverse environmental effect

	To be notified within 24 hours of detection

	Description of where the effect on the environment was detected
	

	Substances(s) detected
	

	Concentrations of substances detected
	

	Date of monitoring/sampling
	


Part B - to be submitted as soon as practicable
	Any more accurate information on the matters for notification under Part A.
	

	Measures taken, or intended to be taken, to prevent a recurrence of the incident
	

	Measures taken, or intended to be taken, to rectify, limit or prevent any pollution of the environment which has been or may be caused by the emission
	

	The dates of any unauthorised emissions from the facility in the preceding 24 months.
	


	Name*
	

	Post
	

	Signature
	

	Date
	


* authorised to sign on behalf of the operator
Schedule 6 - Interpretation 

“accident” means an accident that may result in pollution.

“annually” means once every year.
“application” means the application for this permit, together with any additional information supplied by the operator as part of the application and any response to a notice served under Schedule 5 to the EP Regulations.

“emissions of substances not controlled by emission limits” means emissions of substances to air, water or land from the activities, which are not controlled by an emission or background concentration limit.
“EP Regulations” means The Environmental Permitting (England and Wales) Regulations SI 2016 No.1154 and words and expressions used in this permit which are also used in the Regulations have the same meanings as in those Regulations.

“groundwater” means all water, which is below the surface of the ground in the saturation zone and in direct contact with the ground or subsoil.
“Natural Resources Wales” means the Natural Resources Body for Wales established by article 3 of the Natural Resources Body for Wales (Establishment) Order 2012. The Natural Resources Body for Wales (Functions) Order 2013 transferred the relevant functions of the Countryside Council for Wales, and functions of the Environment Agency and the Forestry Commission in Wales to the Natural Resources Body for Wales.  
“quarter” means a calendar year quarter commencing on 1 January, 1 April, 1 July or 1 October.

“year” means calendar year ending 31 December.
Schedule 7 - Site plan
[image: image1.png]



[image: image2.png]



© Crown Copyright and database right 2023. Ordnance Survey licence number 100019741.
END OF PERMIT

ii
	Permit Number EPR/DB3093ZD
	  Page ii
-  -
- ii -
- ii -ii 
	



[image: image3.jpg]