| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-023328 |
| Permit Reference Number: | EPR-GP3792SK |
| Permit Holder: | Byproduct Recovery Limited |
| Site Address: | Abergelli Farm, Felindre, Swansea, SA5 7NN |
| Grid Reference: | Farm location: SN 65102 01649 Fields: 1294 265134,201933 2225 265225,202221 3732 265374,202310 5867 265586,201657 1808 265199,201825 25 264889,202061 26 263984,201309 27 264131,201267 35 265504,201168 36a 265311,201537 37 265266,201697 Storage: n/a |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: wasteregulation.swwales@cyfoethnaturiolcymru.gov.uk Environment team: swanseaenvironmentteam@cyfoethnaturiolcymru.gov.uk NLO: Don.Rother@environment-agency.gov.uk |
| Date due: | 10/10/2023 |
| Are fields on Anglesey? If yes, then send application. | N/A |
| Permit compliance rating and multiplier | Rating B- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | EA public register |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and deployment fee- Charging Scheme |
| Fee on MyNRW | Y | Transaction reference: 2118248 Date: 04/08/2023 Ref: BS/040823/08 BY PRODUCT RECOVERY PSCAPPBYPRO1321 Amount: 1,561.00 |
| Location map | Y | 2. Location Plan_Abergelli Farm 1 (2023) |
| Benefit statement | Y | 1.1 LPD1 Supplement_Parcles of Land (2023) 1.2 LPD1 Supplement_Previous Spreading (2023) 3. ABS_ Abergelli Farm 1 (2023) 6. Landowner consent to spread_Abergell Farm 1 (2023) BPR Approval to sign on behalf (2023) |
| Waste Analysis | Y | 4.1 Felindre liquid waste analysis May23 4.2 Waste interpretation dry matter |
| Soil Analysis | Y | 5.1 Soils_Fields 25, 26 & 27 5.2 Soils_Fields 35, 36a, 37, 1294 & 2225 5.3 Soils_Fields 3732, 5867, 1808 |
| EIA Agri Regulations evidence | Y | 7. Seminatural assessment (Photo evidence) |
| Site Specific Risk Assessment | n/a |  |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| - About the Permit |  |  |
| ## Has there been any pre-application discussions? | N |  |
| ## Has the permit number been provided? | Y |  |
| ## Has the permit holder name been provided? | Y |  |
| ## What permit is being deployed under? | SR2010 No4 |  |
| - Contact details |  |  |
| ## Has contact information been provided? | Y | Esther Koroma |
| ## Has the technically competent manager details been provided? | Y | Richard Evans |
| ## Has the nominated competent person details been provided? | Y | Richard Evans |
| - About the land you want to treat |  |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | LPD1 form: 59/156/0050 MyMap: 59/156/0050 WB AND SAM LLEWELLYN ABERGELLI FARM |
| ## Has all of parcels of land table been completed? | Y | See document 1.1 LPD1 Supplement_Parcles of Land (2023) Contains all required information |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y | 25.9 ha |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | Y |  |
| ## Has written consent of all owners(s) or occupiers(s) been given? | Y | See document 6. Landowner consent to spread_Abergell Farm 1 (2023) Contains all required information |
| - Previous land treatment |  |  |
| Has all of the previous land treatment table been completed? | Y | PAN-018217 expired 01/08/2023 See document 1.2 LPD1 Supplement_Previous Spreading (2023) |
| - Which risk band does the activity fall within? - SR2010No4 List A wastes (Lower risk) - Location risk - Low risk deployment - Medium risk (1) deployment - Medium risk (2) deployment - High risk deployment |  |  |
| ## Which risk band does the activity fall within? | Medium (1) |  |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) or High risk deployment has a SSRA been provided? | n/a |  |
| - Additional information on sensitive receptors |  |  |
| ## What type of risk assessment has been submitted? | benefit statement |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | n/a |  |
| If no have details of the assessment evidence to support this conclusion been provided? | Y | See document 7. Seminatural assessment (Photo evidence) Some of the field labels are illegible, this need to be clear- RFI Resolved 27/10/23 |
| - About the waste |  |  |
| Have all parts of the waste-types table been completed? - List of waste code (6 digit) - Description of waste (this must be an accurate description specific to this waste that refers to the List of Waste written description) - Physical form of waste/Is it stackable or non-stackable - Is the waste high in readily available nitrogen? - Producer of waste & permit number (if applicable) - Total amount being spread/used (tonnes) | Y | All information provided on the table |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y | DCWW waste |
| - About the storage |  |  |
| Has all of waste storage table been completed? | n/a | No storage proposed |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | n/a | No storage proposed |
| 18. Supporting documents |  |  |
| Has checklist been completed? | Y |  |
| 20. Confidentiality and national security |  |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| 21. Declaration |  |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | Y |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has the operator confirmed they will comply with the NRW pre-notification requirements. | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | n/a | No storage proposed |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no storage or spreading within 10 meters) (buffer zones needed) | Y | All watercourses buffered |
| groundwater source protection zones (permit condition: no storage or spreading in SPZ1) (high risk deployment if in SPZ2) | n/a | None in vicinity |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no storage or spreading within 50 meters) | N | Abstraction in field 1808 buffered Issue between fields 2225 and 1294 has been buffered but issues alongside field 1808 and to the east of field 26 have not been buffered- RFI Resolved 27/10/23 |
| location of all storage sites for the waste to be spread | n/a | No storage proposed |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | N | See above Resolved 27/10/23 |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | N | See above Resolved 27/10/23 |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | n/a | None within 500 meters |
| location of any public rights of way (shown on map) | N | Between fields 1294 and 1808. Not shown on site plan Resolved 27/10/23 |
| any buildings or houses within 250m (shown on map) | Y |  |
| Within Groundwater Safeguard Zone | n/a | None |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Section 7 habitats - Section 7 species - Scheduled Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Hedges - Ditches | N | See PAN-023328 MyMap Report Number of LWS border fields but have not been accurately shown on the spreading maps. Field 3732 needs a 10 meter no spread zone Resolved 27/10/23 |
| What is the slope of the fields? | 0-7 degrees (transparent)- all fields 7 -12 degrees (yellow)- areas of field 2225 and 5867 only 12 degrees plus (red)- no fields |  |

| Waste Storage | Y N n/a | Comments |
| --- | --- | --- |
| Is waste storage occurring at the place where waste is being deployed? | n/a | No storage proposed |
| Is only waste as specified in this deployment being stored in the proposed storage locations? | n/a | No storage proposed |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in the same place one constructed within the last two years? or in different location? | n/a | No storage proposed |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in a field: - liable to flooding or becoming waterlogged, or - within 50 metres of a borehole, spring or well or - within 10 metres of surface water or a land drain - within 30 meters of a watercourse on steeply sloping land | n/a | No storage proposed |
| If the waste is non-stackable is the waste stored within either a vessel, a covered building, or on an impermeable surface | n/a | No storage proposed |
| If the waste is high RAN and it is being stored within 200 metres of a European Site (SAC/SPA), Ramsar Site or a Site of Special Scientific Interest (SSSI) is the waste being covered to prevent loss of ammonia? (condition 3.2.4 of the permit) | n/a | No storage proposed |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | n/a | Not relevant to waste |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | Y | Aluminium and Iron |
| - If yes; have they been tested for? | Y | Both tested for |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a | PTEs in waste are not high, but some fields are high in arsenic, zinc and Molybdenum - See comments section |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y | Application rates have been adjusted for fields where P is higher than target |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y | “Targeted periods of spreading on grass fields include spring 2024 (late Feb- Aril) and summer and autumn 2024 (July - October). The waste may also be spread in Autumn 2023 (October).” |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | n/a |  |
| - Soil physical properties (e.g. organic matter and soil structure) | Y | Organic matter |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| Timing of application- if any of the wastes are high RAN, are they being spread OUTSIDE of the closed periods | n/a | Waste not high RAN |
| Groundwater Safeguard Zone for nitrate- if any of the wastes are high RAN, are the application rates less than 50 m3/ha, with a minimum 3 week gap between applications | n/a | Not spreading in a Groundwater Safeguard Zone for nitrate/ Waste not high RAN |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | n/a |  |
| - Included the nutrient level in their environmental risk assessment | n/a |  |
| - Stated the source of information that they used to set the nutrient level. | n/a |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y | See comments |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need, as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If the cropping is any of the following, is the maximum nitrogen limit in line with that given in The Water Resources (Control of Agricultural Pollution) (Wales) Regulations 2021: - 0 kg/ha- Field beans, Peas - 120 kg/ha- Sugar beet - 150 kg/ha- Asparagus, Carrots, Forage maize. Radish, Spring barley, Swedes - 180 kg/ha- Spring-sown wheat, Winter barley - 220 kg/ha- Autumn or early winter sown wheat | n/a | Cropping is grass |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | n/a | Only one waste is being spread |
| - Is it only for operational purposes? | n/a |  |
| - Have individual application rates for each waste been given? | n/a |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | n/a |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y | low trajectory splash plate |
| - Is the timing of application appropriate? | Y | “Targeted periods of spreading on grass fields include spring 2024 (late Feb- Aril) and summer and autumn 2024 (July - October). The waste may also be spread in Autumn 2023 (October).” |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | Y | See document 4.2 Waste interpretation dry matter |
| Does the total nutrients given by the applicant match NRW calculations? | Y |  |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y | PAN-018217 expired 01/08/2023 See document 1.2 LPD1 Supplement_Previous Spreading (2023) |
| - How much material was spread? | Y |  |
| - At what application rate material was spread? | Y |  |
| - The nutrients supplied by the material? | Y |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | n/a |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | n/a |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | n/a |  |
| - Pathways | n/a |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | n/a |  |
| - Measures that will minimise or mitigate the impact will be put in place | /a |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | N |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | n/a |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | N |

Comments

Consultation responses: - No actions required following consultation

MyMap/location and slope assessments: MyMap/location assessment See Comments above and PAN-023328 MyMap Report on DMS Slope assessment Field number: areas of field 2225 and 5867 only Slope of the land: 7 -12 degrees (yellow) Soil type: Medium Wastes types applied (solid, liquid, sludge etc.): non-stackable liquid sludge Cropping type: grass Method of application: low trajectory splash plate Weather conditions before during and after spreading: Spreading will only take place subject to ground conditions and following the Code of Good Agricultural Practice (Defra, 2011), NVZ regulations where applicable and the permit holder’s Environmental Management System (EMS). Spreading activities will also comply with The Water Resources (Control of Agricultural Pollution) (Wales) Regulations 2021. Other risk factors not mentioned above: None Conclusions (is run off risk high?): low risk of run off due to small areas of slope and low application rates

CPH details: - LPD1 form: 59/156/0050 - MyMap: 59/156/0050 WB AND SAM LLEWELLYN ABERGELLI FARM

Protection of habitats and ecosystems - Evidence provided, but the field labels on some photographs are illegible- RFI Resolved 27/10/23

Agricultural benefit calculations: PTEs Soil analysis shows that some fields are high (At or above % of maximum permissible concentration of PTE in arable/grasssland soil) in arsenic, zinc and Molybdenum. Waste analysis show that the DCWW Felindre is not high in these PTEs and do not exceed their limits- see PAN-023328 NRW Nutrient Calculator for PTE additions. Phosphate additions P index 2- 25, 27, 36a- Application rate of 97 t/ha P additions are fractionally higher (0.77kg/ha) that offtake. P index 3- 1294, 2225, 3732, 1808, 37- Application rate of 30 t/ha P offtake is higher than additions and will reduce overall P levels P index 4- 5867, 35- Application rate of 15 t/ha P offtake is higher than additions and will reduce overall P levels

Request for information: Sent 26/10/23- deadline of 10/11/23 - Protection of habitats and ecosystems evidence labels are illegible - Issues not shown or buffered - Public right of way not shown - Local life sites not shown or buffered Response received 27/10/23- all points resolved.

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date | 30/10/23 |
| ## Effective date | 30/10/23 |
| ## Permitting Officer Date | Louise Bailey 30/10/23 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 30/10/2023 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 440 |
| Sign off | 10 |
| Issuing | 10 |
| Total | 520 |