# Schedule 1 - Notification of abnormal emissions

This page outlines the information that the Operator must provide to satisfy conditions 5.1.1 and 5.1.2 of this Permit.

Units of measurement used in information supplied under Part A and B requirements shall be appropriate to the circumstances of the emission. Where appropriate, a comparison should be made of actual emissions and authorised emission limits.

#### If any information is considered commercially confidential, it should be separated from non-confidential information, supplied on a separate sheet and accompanied by an application for commercial confidentiality under the provisions of the PPC Regulations.

Part A

| Permit Number | BU8002 |
| --- | --- |
| Name of Operator | Dunbia Cross Hands |
| Location of Installation | Cross Hands Food Park, Llanelli, Carmarthenshire SA14 6RF |
| Location of the emission | W5 |
| Time and date of the emission | 20/10/23 at 09:00 |

| Substance(s) emitted | Media | Best estimate of the quantity or the rate of emission | Time during which the emission took place |
| --- | --- | --- | --- |
| Ammoniacal Nitrogen | Surface water W5 | 0.8mg/l – Limit 0.25mg/L | 09:00 |

| Measures taken, or intended to be taken, to stop the emission | Following receipt of this result an investigation was carried out. All chemical and waste areas have been checked and there has been no evidence of spillage or leakage in bunded and contained areas. There is also no evidence of spillage outside of these areas. The site drainage channel and roof areas have been recently fully cleaned removing build-up of algae or leaves as a root cause. When skips are changed over a small volume of liquid seepage can occur when the skip is disconnected from the compactor, however when reviewed this was found to be contained within the foul drainage channel in this area. Due to the age of the compactors we would like to upgrade these compactors to ensure more effective compaction. As part of this equipment upgrade we will review the feasibility of upgrading the compactor for a skip that has a built-in compactor eliminating the need for a skip to be disconnected from the compactor so minimising risk from this activity further. Feasibility assessment to be completed by 30/12/2023. If feasible change will be completed, with timeline to be verified by supplier dependent on lead time for new equipment. |
| --- | --- |

Part B

| Any more accurate information on the matters for notification under Part A. | As above. |
| --- | --- |
| Measures taken, or intended to be taken, to prevent a recurrence of the incident | As above. |
| Measures taken, or intended to be taken, to rectify, limit or prevent any pollution of the environment or harm which has been or may be caused by the emission | As above. |
| The dates of any unauthorised emissions from the installation in the preceding 24 months. | 14/01/21, 21/08/21, 30/11/21, 29/12/21, 27/05/22, 09/09/22, 30/12/22, 30/06/23, 13/07/23, 24/08/23 |

| Name* | Chris Dodd |
| --- | --- |
| Post | HSE Manager |
| Signature | C. Dodd |
| Date | 21/11/2023 |

* authorised to sign on behalf of Dunbia (UK) Ltd