# Natural Resources Wales permitting decisions

# Bespoke permit

We have decided to grant the permit for Cas Blaidd operated by Penylan Country Lodge Kennels & Cattery Limited.

The application number is PAN-022499.

The permit number is [insert permit reference]

The permit regulates an existing discharge from a septic tank: a maximum of 1 cubic metre per day discharging to ground via an infiltration system that was installed pre 1 April 2010. The system serves one residential apartment and a grooming room with two baths.

This discharge was previously regulated under a registration. Following a review of the Source Protection Zones, the discharge is now within Source Protection Zone 1 and a bespoke permit is required for the discharge.  An improvement condition that requires the installation of a new 20 micron filter has been added as a condition to this permit.

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

Purpose of this document

This decision document:

- explains how the application has been determined

- provides a record of the decision-making process

- shows how all relevant factors have been taken into account

- justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

Structure of this document

- Annex 1 the decision checklist

- Annex 2 the consultation responses

# Annex 1: decision checklist

This checklist should be read in conjunction with the Duly Making checklist.

| ### Aspect | ### Justification / Detail | Criteria met |
| --- | --- | --- |
|  |  | Yes |
| Consultation |  |  |  |
| Responses to consultation, | The web publicising, consultation and newspaper advertising responses (Annex 2) were taken into account in the decision. The decision was taken in accordance with our guidance. No responses were received. | ü |
| Operator |  |  |  |
| Control of the facility | We are satisfied that the applicant (now the operator) is the person who will have control over the operation of the facility after the grant of the permit. The decision was taken in accordance with EPR RGN 1 Understanding the meaning of operator. | ü |
| European Directives |  |  |  |
| Applicable directives | All applicable European directives have been considered in the determination of the application. | ü |
| The site |  |  |  |
| Extent of the site of the facility | The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility including discharge points. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary. | ü |
| Habitat Regulation Assessment for a designated site under the EU Habitats Directive | No FORM 1 completed: HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site. This proposal has been screened to a radius of 500m from the proposed discharge and is outside of any Natura 2000/Ramsar sites. | ü |
| Environmental Risk Assessment and operating techniques |  |  |  |
| Environmental risk | We have carried out a risk assessment on behalf of the operator. The operator considers this risk assessment is satisfactory. | ü |
| Operating techniques | We have reviewed the techniques used by the operator and compared these with the relevant guidance notes according to TGN 7.01 . The proposed techniques / emission levels for priorities for control are in line with the benchmark levels contained in the TGN and we consider them to represent appropriate techniques for the facility. | ü |
| The permit conditions |  |  |  |
| Improvement conditions | Based on the information on the application, we consider that we need to impose improvement conditions. We have imposed improvement conditions to ensure that a 20 micron filter is installed as part of the system within six months of the issue of permit. | ü |
| Considerations of foul sewer | We agree with the operators justification for not connecting to foul sewer. | ü |
| Operator Competence |  |  |  |
| Environment Management System | There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. | ü |
| Sustainable Management of Natural Resources (SMNR) |  |  |
| Considerations of SMNR - Compliance with our General Purpose | We are satisfied that this decision is compatible with our general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources” | ü |
| Environment Wales Act 2016 |  |  |
| Biodiversity and resilience of ecosystems duty | Section 6 of the Environment Wales Act 2016 requires that we seek to maintain and enhance biodiversity in the exercise of our functions, and in so doing promote the resilience of ecosystems, in a manner that is consistent with the proper exercise of our functions. NRW is satisfied that in this case, we have taken into account and had due regard to this duty in so far as it is consistent with the function of determining an application for a groundwater discharge activity permit under EPR2016. | ü |

# Annex 2: Consultation responses

Response received from

DCWW

Brief summary of issues raised

From the information provided we have determined that the disposal location of the proposed discharge falls outside any Dŵr Cymru Welsh Water (DCWW) drinking water catchment. We therefore have no further comments on this application.

Summary of actions taken or show how this has been covered

No further action