| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-023587 |
| Permit Reference Number: | EPR-GP3792SK |
| Permit Holder: | 4R Byproduct Recovery Limited incorrect |
| Site Address: | Abergelli Farm 2, Felindre, Swansea, SA5 7NN |
| Grid Reference: | Farm location: SN 65117 01649 Fields: 415 SN 65089 02237 3907 SN 65439 02101 17 SN 64579 01629 16 SN 64634 01489 18 SN 64473 01537 6201 SN 65623 00929 1163 SN 66015 00733 0173 SN 66110 00671 0455 SN 66032 00569 Storage: No Storage |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: wasteregulation.swwales@cyfoethnaturiolcymru.gov.uk Environment team: swanseaenvironmentteam@cyfoethnaturiolcymru.gov.uk NLO: Don.Rother@environment-agency.gov.uk |
| Date due: | 07/11/2023 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating B- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | Variations to change address only |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and deployment fee- Charging Scheme |
| Fee on MyNRW | Y | Amount:1561 Date cleared on MyNRW:13/10/23 Transaction number: 212275 |
| Location map | Y |  |
| Benefit statement | Y |  |
| Waste Analysis | Y |  |
| Soil Analysis | Y |  |
| EIA Agri Regulations evidence | Y |  |
| Site Specific Risk Assessment | n/a | High risk location only |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| - About the Permit |  |  |
| ## Has there been any pre-application discussions? | N |  |
| ## Has the permit number been provided? | Y | EPR/GP3792SK |
| ## Has the permit holder name been provided? | Y | 4R BYPRODUCT RECOVERY LIMITED given in application but is BYPRODUCT RECOVERY LIMITED in permit |
| ## What permit is being deployed under? | SR2010 No4 |  |
| - Contact details |  |  |
| ## Has contact information been provided? | Y |  |
| ## Has the technically competent manager details been provided? | Y |  |
| ## Has the nominated competent person details been provided? | Y |  |
| - About the land you want to treat |  |  |
| ## Has the address and grid reference of the main farm address been given? | Y | Abergelli Farm 2, Felindre, Swansea. |
| ## Does the farm address match the grid reference? | Y | SN 65117 01649 |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | See comments |
| ## Has all of parcels of land table been completed? | Y |  |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y |  |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | Y | Ticked yes but this is wrong as the farm and fields are not owned by permit holder |
| ## Have details of the land owner or occupier been provided? | N | Not answered as previous question answered “yes” but landowner declarations provided also see comment re CPH |
| ## Has written consent of all owners(s) or occupiers(s) been given? | Y | Unclear, documents don’t hold all relevant info, farm addreses don’t all match, names don’t match |
| - Previous land treatment |  |  |
| Has all of the previous land treatment table been completed? | Y | PAN not provided PAN-018834 found on DMS expired Oct 2023 |
| - Which risk band does the activity fall within? - SR2010No4 List A wastes (Lower risk) - Location risk - Low risk deployment - Medium risk (1) deployment - Medium risk (2) deployment - High risk deployment |  |  |
| ## Which risk band does the activity fall within? | Medium (1) | States list A waste low risk on application, the waste is 19 09 02 which is a list B waste in the permit therefore the application is med risk |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) or High risk deployment has a SSRA been provided? | n/a |  |
| - Additional information on sensitive receptors |  |  |
| ## What type of risk assessment has been submitted? | benefit statement |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | n/a |  |
| If no have details of the assessment evidence to support this conclusion been provided? | Y | See comments re field 0415 |
| - About the waste |  |  |
| Have all parts of the waste-types table been completed? - List of waste code (6 digit) - Description of waste (this must be an accurate description specific to this waste that refers to the List of Waste written description) - Physical form of waste/Is it stackable or non-stackable - Is the waste high in readily available nitrogen? - Producer of waste & permit number (if applicable) - Total amount being spread/used (tonnes) | Y |  |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y |  |
| - About the storage |  |  |
| Has all of waste storage table been completed? | n/a |  |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | n/a |  |
| 18. Supporting documents |  |  |
| Has checklist been completed? | Y |  |
| 20. Confidentiality and national security |  |  |
| Has confidentially been claimed? | n/a |  |
| Has national security been claimed? | n/a |  |
| 21. Declaration |  |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | Y |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has the operator confirmed they will comply with the NRW pre-notification requirements. | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | n/a |  |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no storage or spreading within 10 meters) (buffer zones needed) | Y | See comments |
| groundwater source protection zones (permit condition: no storage or spreading in SPZ1) (high risk deployment if in SPZ2) | n/a |  |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no storage or spreading within 50 meters) | Y | See comments |
| location of all storage sites for the waste to be spread | n/a |  |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | Y | See comments |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | Y |  |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | n/a |  |
| location of any public rights of way (shown on map) | Y |  |
| any buildings or houses within 250m (shown on map) | Y |  |
| Within Groundwater Safeguard Zone | n/a |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Section 7 habitats - Section 7 species - Scheduled Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Hedges - Ditches | Y | See comments |
| What is the slope of the fields? | See comments |  |

| Waste Storage | Y N n/a | Comments |
| --- | --- | --- |
| Is waste storage occurring at the place where waste is being deployed? | n/a |  |
| Is only waste as specified in this deployment being stored in the proposed storage locations? i.e. slurry in lagoons Note: There may be multiple deployments using one storage location | n/a |  |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in the same place one constructed within the last two years? or in different location? | n/a |  |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in a field: - liable to flooding or becoming waterlogged, or - within 50 metres of a borehole, spring or well or - within 10 metres of surface water or a land drain - within 30 meters of a watercourse on steeply sloping land | n/a | No storage |
| If the waste is non-stackable is the waste stored within either a vessel, a covered building, or on an impermeable surface | n/a | No storage |
| If the waste is high RAN and it is being stored within 200 metres of a European Site (SAC/SPA), Ramsar Site or a Site of Special Scientific Interest (SSSI) is the waste being covered to prevent loss of ammonia? (condition 3.2.4 of the permit) | n/a | No storage |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | n/a |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | n/a |  |
| - If yes; have they been tested for? | n/a |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y |  |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y | Table 10 refers to not in NVZ |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y |  |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | Y |  |
| - Soil physical properties (e.g. organic matter and soil structure) | Y |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| Timing of application- if any of the wastes are high RAN, are they being spread OUTSIDE of the closed periods | n/a | Not high RAN |
| Groundwater Safeguard Zone for nitrate- if any of the wastes are high RAN, are the application rates less than 50 m3/ha, with a minimum 3 week gap between applications | n/a | Not in GSZ but consideration is given in AgBen |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | n/a |  |
| - Included the nutrient level in their environmental risk assessment | n/a |  |
| - Stated the source of information that they used to set the nutrient level. | n/a |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y |  |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need, as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If the cropping is any of the following, is the maximum nitrogen limit in line with that given in The Water Resources (Control of Agricultural Pollution) (Wales) Regulations 2021: - 0 kg/ha- Field beans, Peas - 120 kg/ha- Sugar beet - 150 kg/ha- Asparagus, Carrots, Forage maize. Radish, Spring barley, Swedes - 180 kg/ha- Spring-sown wheat, Winter barley - 220 kg/ha- Autumn or early winter sown wheat | n/a | Grass only |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | n/a |  |
| - Is it only for operational purposes? | n/a |  |
| - Have individual application rates for each waste been given? | n/a |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | n/a |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y | States feb – sept not accurate |
| - Have they stated how often the waste will be spread? | Y | As above, vague |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients given by the applicant match NRW calculations? | N | See comments |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y |  |
| - How much material was spread? | Y |  |
| - At what application rate material was spread? | Y | PAN -018834 includes the relevant information |
| - The nutrients supplied by the material? | Y | PAN -018834 includes the relevant information |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | n/a |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | n/a |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | n/a |  |
| - Pathways | n/a |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | n/a |  |
| - Measures that will minimise or mitigate the impact will be put in place | n/a |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | N |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | n/a |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | N |

Comments

Consultation responses: - Nil responses

MyMap/location and slope assessments: MyMap/location assessment Detailed river network: Watercourse SE of 415, not shown clearly shown on application map, no buffer applied Wastercourse SW of 3907, not shown clearly on application map but buffer is in place Watercourse S of 16 shown on application map with buffer, see comments re Spring below. None at 17 or 18. Watercourses SE, SW and W of 6201 shown on application maps with buffer None at 1163, 0173, 0455 Groundwater protection zones none Any well, spring (or issue) or borehole used / not used for the supply of water for human consumption including from a private water supply or supplies within 50m. Issue SE of 415 not shown on application map, no buffer Issue W of 3907, not shown on application maps, map does show a buffer but this may not be sufficient RFI Spring SW of 16 not shown on application map, no buffer, Issue N of 6201 not shown on application map but a buffer is in place, dimensions for the buffer not given. RFI None at 16, 17, 18 Any well, spring (or issue) or borehole used / not used for the supply of water for human consumption including from a private water supply or supplies within 250m. Issue S of 415 not shown on application map 4xIssues E and 1x Issue SW of 3907 not shown on application maps Issue and Spring SE of 16 not shown on application map Deregualted abstraction S of 6201, shown on application map, 2xIssue N of 6201 not shown on application map Issue SW of 0455 not shown on application map. None at 0173, 1163 European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m None in range Other protected sites within 500m See MyMap screening report for additional details - Ancient Woodland N and SW of 415 not shown on application map - Local Wildlfe Sites (SINCs) at Waun Garn Wen SW, Rhyd-Y-Pandy Valley and Grasslands E, Rhos Fawr N and Lower Lliw Reservoir N of 415 not shown on application map - Ancient Woodland S of 3907 not shown on application map - Local Wildlife Sites Waun Garn Wen E, Llety-Morfil S, Rhyd-Y-Pandy Valley and Grasslands E, Rhos Fawr N of 3907 not shown on application map - Ancient woodland, N, S and SE of 16, 17 and 19 not shown on application map - Local Wildlife Sites (SINCs) Waun Garn Wen in which the fields are placed, Llety-Morfil SE, Cilfaen NW of 16, 17, 18 not shown on application maps - Ancient Woodland Sites N, S and W of 6201 not shown on application maps - Local Wildife Sites Llety-Morfil N and W, Pant Lasau SW, Middle Llan E and S of 6201 not shown on application maps - Ancient Woodland E of 0455 partially shown on application map but not identified as AW. RFI - Local Wildlife Sites (SINCs) Pant Lasau E of 0455, Rhyd-Y-Pandy Valley and Grasslands NE, Middle Llan N of 1163 not shown on application maps - Protected species SW of 6201 migratory fish route for Brown Trout: Ferox, Trout. Sea Trout: Peal, Salmon Trout, Sewen, White Trout, Eel, Silver eel, Yellow Eel, Millers Thumb not shown on application maps Public rights of way Nearest is adjacent NW of 3907, buffer in place Slope Map Fields 415, 0455,0173,1163 – 0-7 degrees 16, 18, 0-7 6201, 0-7 0455, 0173, 0-7 17 mainly 0-7 with some 7-12 falls to a landrain but buffer is in place not shown on maps 1163 0-7, 7-12. No buffer at boundary and no consideration in agben Slope assessment Field number:17 Slope of the land: mainly 0-7 with some 7-12 Soil type: medium Wastes types applied (solid, liquid, sludge etc.): liquid sludge Cropping type: grass Method of application:dribble bar Weather conditions before during and after spreading:IAW COGAP Other risk factors not mentioned above: falls to a land drain but buffer is in place not shown on maps Conclusions (is run off risk high?):low Field number:1163 Slope of the land:0-7, 7-12 Soil type: medium Wastes types applied (solid, liquid, sludge etc.): liquid sludge Cropping type: grass Method of application:dribble bar Weather conditions before during and after spreading:IAW COGAP Other risk factors not mentioned above: No buffer at boundary and no consideration in agben Conclusions (is run off risk high?):medium but low if buffer applied

CPH details: CPH provided 550020122 Abergelli Farm 2, Felindre, Swansea SA5 7NN The details provided do not fully match any of the details available to NRW 455, 1163, 0173 show as 55/002/0122 HT AND MM THOMAS GLYN Y MEIRCH WERNDDU ROAD AMMANFORD DYFED SA18 2US 415, 3907, 18, 17, 16 shows as 59/156/0050 WB AND SAM LLEWELLYN ABERGELLI FARM FELINDRE ABERTAWE SA5 7NN 6201 shows as 59/156/0050 WB AND SAM LLEWELLYN ABERGELLI FARM FELINDRE ABERTAWE SA5 7NN

Protection of habitats and ecosystems EIA evidence provided Field 0415 - previously determined in application PAN-018834 that field 0415 falls within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat), and therefore cannot be included within this spreading area. It was agreed that this land could not be spread on in the previous deployment but has been included within this application

Agricultural benefit calculations: not all of the calculations match the nutrient calculator including Nitrogen as 11.1k/ha, our calculations make this 111.33k/ha, Magnesium as 27.7k/ha, our calculations make this 28.67k/ha. Waste pH 6.9 Phosphate addition - increase within limits for all fields with the exception of 455 which shows a minimal decrease. Potash addition – minimal addition for all fields.

Request for information: n/a application returned

Other information to note: nil

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | No |
| ## Issue date | n/a application returned |
| ## Effective date | n/a application returned |
| ## Permitting Officer Date | Bryan Burn 06/12/2023 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 06/12/2023 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 660 |
| Sign off | 10 |
| Issuing | 10 |
| Total | 740 |