|  | SAFE WORKING PROCEDURE | Document Number |
| --- | --- | --- |
|  |  | CRUK-SWP-CRM-004 |
|  | Grab attachment inspection and maintenance. | Review Period: 2 Years |
|  |  | Review Date Due: June 2025 |
|  |  | Owner: Maintenance Manager |

Revision History

| Review Team Date of Review | Name & Position | Name & Position | Name & Position | Name & Position |
| --- | --- | --- | --- | --- |

| Rev Number | Reason for Revision | Revision Author | Reviewed By | Authorised By | Date |
| --- | --- | --- | --- | --- | --- |
| 00 | Draft for implementation | C Griffiths |  |  |  |

- Purpose and Scope

The purpose of this safe working procedure is to define the method for the inspection and general maintenance on a grab attachment to a 360° Material Handler. It applies to all locations, managers and operators where Celsa Recycling UK have the operating responsibility of the material handlers on site.

- Risk Assessment Reference

Area Risk Assessment: CRUK-CCH-ARA-001-00

CUK-GRA-004-04 General lifting operations

CUK-GRA 011-03 Operating mobile plant

CUK-GRA-013 Traffic movements

Glossary of Terms

| PPE | Personal Protective Equipment |
| --- | --- |
| Grab | Claw attachment for 360° material handler |

- Personal Protective Equipment and Tools required

| PPE Required | Tools Required |
| --- | --- |
| Minimum Site PPE Applies - Hard Hat with Chin Strap - Safety Glasses - Coveralls - Safety Boots - Gloves Hard hat, gloves and safety glasses can be removed within the confines of the cab, but must be worn at all times outside the cab. | - 360° Material Handler - Grease gun - Rags |

- Preconditions

| Satisfy the Preconditions |  |  |
| --- | --- | --- |
| No | Check | Tick |
| 1. | Daily check to have been completed and machine in a serviceable condition |  |
| 2. | Licensed operator only to operate equipment. |  |
| 3. | Machine is located in Safe location outside of yard operating area. |  |

- step by Step Procedure

| Step | Description |  |
| --- | --- | --- |
| 1 | Step 1: Drive machine to maintenance area of yard - Maneuver machine to maintenance area of yard. - Park machine. - Switch off engine. - Open grab tines and place on the ground in a safe area - Remove keys from machine. Collision hazard, check parking area is clear of pedestrians and obstacles before maneuvering in to the maintenance area. |  |
| 2 | Step 2: Visual inspection of the material handler grab attachment - Turn off machine, remove keys and exit the cab - Visually inspect each tine and hydraulic cylinder for any defects or fluid leaks. - Grease each tine in-line with the OEM manual - Inspect tips for wear and replace in-line with OEM manual - Re-enter cab and start machine - Operate grab open and closed to ensure operation is acceptable - Hand machine over back into operations. Hazardous materials –Oil, Grease, Fuel. END OF PROCEDURE |  |