|  | …. SAFE WORKING PROCEDURE | Document Number |
| --- | --- | --- |
|  |  | 01 |
|  | Shear Maintenance (DECAP, Blade Change and Daily Maintenance Checks) | Review Period: 19.05.2022 |
|  |  | Review Date Due: 19.05.2024 |
|  |  | Owner: Scrap Yards UK |

Revision History

| Review Team Date of Review | Name & Position | Name & Position | Name & Position | Name & Position |
| --- | --- | --- | --- | --- |
| 001 |  |  |  |  |

| Rev Number | Reason for Revision | Revision Author | Reviewed By | Authorised By | Date |
| --- | --- | --- | --- | --- | --- |

- Purpose and Scope

To provide instructions & guidance on the Shear Maintenance (including DECAP and Blade Change) (maintenance tasks, daily checks & greasing)

- Risk Assessment Reference

Area risk assessment

Generic activity risk assessment

Task risk assessment

- Glossary of Terms

| PPE | Personal Protective Equipment |
| --- | --- |
| Shear Maintenance | DECAP, Blade Changes and Daily Maintenance checks |

- Personal Protective Equipment and Tools required

| PPE Required | Tools Required |
| --- | --- |
| - Overalls - Welding Leathers - Welding Gloves - Safety Glasses - Welding Masks - Hard hat with chin strap - Cut resistant gloves (WG0347- Kutlass PU500 Gloves) - High visibility vest or jacket - Ear plugs - Steel toe & sole boots (UVEX U8415-2 BOOT QUATRO GORETEX - Welding Boots - Comply with CELSA UK Jewelry Policy 4C7130 - Dust masks | - Hand Tools - Welding Equipment - Burning Equipment - Grinders - Grease Gun - DECAP Locks |

- Preconditions

1 or more trained competent person for the task. Workers must familiarized with the working area and the conditions of the yards, Workers must have a Work Permit to carry out any work within the yard (RAPNET) also presented the Celsa RA for the present task as well as present their own, DECAP in place before any work is carried out, All Hazards, near misses and accidents must be reported immediately the  Yard Manager.

| Satisfy the Preconditions |  |  |
| --- | --- | --- |
| No | Check | Tick |
| 1. | Hot works permit to be issued and R.A.M.S checked |  |
| 2. | Clear communication between Yard Manager |  |
| 3. | DECAP in place on machine |  |
| 4. | PPE for Daily maintenance and PPE for Hot works |  |
| 5 | Sufficient lighting and Good housekeeping in working area |  |
| 6 | Radios |  |

- Significant Hazards & Precautions

| Hazard | Risk Precautions |
| --- | --- |
|  | MAN MACHINE INTERACTION Beacons, claxons & cameras fitted in machinery Blind spot training for floor worker Radio communication to be maintained between floor worker machine operators at all times during operation 'Stop! Thumbs up' rule to apply to floor worker and plant operators All plant operators to be trained and competent High Vis clothing |
|  | Injury from moving machinery Isolate the shear before carrying out any work. |
|  | Burns from hot steel Wear welding gloves, jacket (closed, collar up, arms covered), and face visor |
| Manual handling | Manual Handling of Equipment and Maintenance parts Persons Manual Handling trained and competent of lifting and carrying any equipment and parts for maintenance tasks. |
| H.A.V.S | All persons should have undertaken H.A.V.S training Persons will be using Pneumatic tools. |
| Abrasive Wheels | All persons should have undertaken Abrasive wheel training Persons will be using disc cutter/grinder |
|  | Slips, trips and falls |
|  | - Noise from machines and movement of scrap - Dust - Vibrations |
|  | No pedestrian access to operational areas |
|  | Fork lift trucks and tele handler operating |
|  | No smoking anywhere on Celsa sites |

- Step by Step Procedure

| Step | Description | Done? |
| --- | --- | --- |
| 1 | Step 1 On arrival to the Yard all contractors report to site office for induction/sign in. - work permit to be issued by Site Manager before any work is carried out - Celsa site manager to show contractors where to isolate (DECAP) shear. - Make sure all contractors are wearing all the correct PPE before entering the yard. - Celsa staff to make sure contractors have all the correct equipment/parts for the job. - Escort contractors to the working area Make sure working area is clear of debris and the positioning of contractors vans/equipment is positioned correctly so there is no interference with anything else in yard. . |  |
| 2 | Step 2 Setting up working area and platform for accessing Shear. 2.1 Celsa staff to make sure everything in the working area which can be removed (blast walls) moved to a different location. 2.2 Celsa staff to attach the working platform (Blade changing platform) using Tele handler to the front of the shear for access and working platform. 2.3 Contractor to assemble there tools ready for work. Make sure access ladder to the platform is secure correctly Verb, pictures optional, brief and to the point, command not a story. |  |

- Appendices

…..