Natural Resources Wales permitting decisions

Decision Document – Volac International Limited, The Old Creamery, Felinfach

Contents

Variation	3

The variation number is: EPR/BV4177IS/V008	3

The applicant /operator is: Volac International Limited	3

The Installation is located at: The Old Creamery, Felinfach, Lampeter.	3

Purpose of this document	4

Key issues of the decision	5

Receipt of application	5

European Directives	5

The site	5

Site condition report	5

Biodiversity, Heritage, Landscape and Nature Conservation	5

Environmental Risk Assessment	5

Air	5

Water	6

Soil	6

Odour	6

Noise	6

Fugitive emissions	7

Monitoring	7

Reporting	7

Operating techniques	8

The permit conditions	8

Incorporating the application	8

Environment management system	8

# Variation

## The variation number is: EPR/BV4177IS/V008

## The applicant /operator is: Volac International Limited

## The Installation is located at: The Old Creamery, Felinfach, Lampeter.

We have decided to issue the variation for Volac Felinfach operated by Volac International Limited.

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

## Purpose of this document

This decision document:

explains how the application has been determined

provides a record of the decision-making process

shows how all relevant factors have been taken into account

justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

# Key issues of the decision

# Receipt of application

## European Directives

All applicable European directives have been considered in the determination of the application.

## The site

The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility including discharge points.

A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary.

The site perimeter has been adjusted to correct a change made in error in 2016 and to add a small area for the storage of wood.

### Site condition report

The operator has provided a description of the condition of the new part of the site.

We consider this description is satisfactory.  The decision was taken in accordance with our guidance on site condition reports – guidance and templates (H5).

### Biodiversity, Heritage, Landscape and Nature Conservation

There are no proposed changes to the composition or quantity of the emissions in this variation. Therefore a review of the impacts of emissions on these sites was not necessary.

## Environmental Risk Assessment

### Air

No increase in air emissions, some emission points are raised.

#### Emission limits

No change to emission limits. Emission point A2 has been removed at the request of the operator.

### Water

Based upon the information in the application we are satisfied that the appropriate measures will be in place to prevent pollution of ground and surface water.

#### Emission limits

The only emission limit that is altered by this variation for the discharge of effluent from the site to the adjacent effluent treatment works. The variation removes the concentration limit on phosphate, although the phosphate monitoring will continue. As the discharge from the adjacent effluent treatment plant is direct to the sea, the concentration of phosphate is less relevant than the phosphate load. The adjacent effluent treatment plant has its own permit.

We have decided that emission limits should be set for the parameters listed in the permit.

### Soil

The operator has provided a description of the condition of the extension to the site.

We consider this description is satisfactory. The decision was taken in accordance with our guidance on site condition reports – guidance and templates (H5).

### Odour

This variation does not alter the odour emissions from the site.

### Noise

The noise modelling report has been assessed. The changes to the noise characteristics, brought about by raising the height of an emission point, in line with increasing the height of the building, are minimal.

### Fugitive emissions

Based upon the information in the application we are satisfied that the appropriate measures will be in place to prevent or where that is not practicable to minimise fugitive emissions and to prevent pollution from fugitive emissions.

### Monitoring

There are no changes to the monitoring requirements.

### Reporting

There are no changes to the reporting requirements.

## Operating techniques

We have reviewed the techniques used by the operator and compared these with the relevant guidance notes.

The proposed techniques/ emission levels for priorities for control are in line with the benchmark levels contained in the TGN and we consider them to represent appropriate techniques for the facility.

## The permit conditions

### Incorporating the application

We have specified that the applicant must operate the permit in accordance with descriptions in the application, including all additional information received as part of the determination process.

These descriptions are specified in the Operating Techniques table in the permit.

### Environment management system

There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions.  The decision was taken in accordance with RGN 5 on Operator Competence.