| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-023888 |
| Permit Reference Number: | EPR/ GP3792SK |
| Permit Holder: | BYPRODUCT RECOVERY LIMITED |
| Site Address: | Cassandra Farm, Caerleon, Newport, NP18 1LR |
| Grid Reference: | Farm location: 334950 191821 Fields: 1 1 ST 35261 91855 2 2 ST 35627 91831 3 3 ST 35509 91756 4 4 ST 35321 91674 5 5 ST 35166 91675 6 6 ST 35015 91765 7 7 ST 35103 91921 8 8 ST 34765 93095 9 9 ST 34495 92897 10 10 ST 34573 93104 12 13 ST 34975 91581 13 15 ST 34589 91431 14 16 ST 34474 91560 15 20 ST 34563 92561 16 22 ST 34724 92149 Storage: A 335216 191859 B 335469 191819 C 335157 191927 D 335079 191695 E 334950 191821 F 335095 191827 G 334583 191706 H 334485 191457 I 334679 192177 J 334746 192104 K 334581 192578 L 334464 192952 M 334805 193123 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: SE.waste.queries@cyfoethnaturiolcymru.gov.uk Environment team: ET.NewportCaerphilly&BlaenauGwent@cyfoethnaturiolcymru.gov.uk environmentteammonmouthshireandtorfaen@cyfoethnaturiolcymru.gov.uk NLO: Don.Rother@environment-agency.gov.uk |
| Date due: | 05/12/2023 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating B- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | English- NLO |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and deployment fee- Charging Scheme |
| Fee on MyNRW | Y | Amount: £1837 Date cleared on MyNRW:10/11/2023 Transaction number: 2123705 |
| Location map | Y |  |
| Benefit statement | Y |  |
| Waste Analysis | Y |  |
| Soil Analysis | Y |  |
| EIA Agri Regulations evidence | Y |  |
| Site Specific Risk Assessment | Y | High risk location only |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| - About the Permit |  |  |
| ## Has there been any pre-application discussions? | Y |  |
| ## Has the permit number been provided? | Y |  |
| ## Has the permit holder name been provided? | Y |  |
| ## What permit is being deployed under? | SR2010 No4 |  |
| - Contact details |  |  |
| ## Has contact information been provided? | Y |  |
| ## Has the technically competent manager details been provided? | Y |  |
| ## Has the nominated competent person details been provided? | Y |  |
| - About the land you want to treat |  |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | Applicant has stated there is no CPH available RFI. resolved via RFI. fields 1-13 60/034/0021- J Mayho 15-16 none available by applicant or on GIS 20-22- 60/030/0079 |
| ## Has all of parcels of land table been completed? | Y |  |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y |  |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | Y | Details of one landowner/occupier have been provided however there are other occupiers/landowners who have not been mentioned Resolved via RFI |
| ## Has written consent of all owners(s) or occupiers(s) been given? | Y | Consent does not meet the standards required for consent. As above landowner/occupier for someth fields has not provided consent. Resolved via RFI. |
| - Previous land treatment |  |  |
| Has all of the previous land treatment table been completed? | Y | PAN-019349 |
| - Which risk band does the activity fall within? - SR2010No4 List A wastes (Lower risk) - Location risk - Low risk deployment - Medium risk (1) deployment - Medium risk (2) deployment - High risk deployment |  |  |
| ## Which risk band does the activity fall within? | High |  |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) or High risk deployment has a SSRA been provided? | Y | Does not address all risks. Resolved via rfi, amended risk assessment provided. |
| - Additional information on sensitive receptors |  |  |
| ## What type of risk assessment has been submitted? | site‐specific risk assessment benefit statement |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | N |  |
| If no have details of the assessment evidence to support this conclusion been provided? | Y | EIA images provided for field 1 and 5 and 6 and 7 appear to be the same fields. In application you have stated the all fields current and previous cropping is grass however the EIA image evidence provided shows fields 1, 5, 6, 7, 8, 10 appear to contain arable crops. Following RFI applicant confirmed previous crops in these fields were either forage maize or spring barley. Fields 1, 5, 6 and 7 are growing Forage maize this year. |
| - About the waste |  |  |
| Have all parts of the waste-types table been completed? - List of waste code (6 digit) - Description of waste (this must be an accurate description specific to this waste that refers to the List of Waste written description) - Physical form of waste/Is it stackable or non-stackable - Is the waste high in readily available nitrogen? - Producer of waste & permit number (if applicable) - Total amount being spread/used (tonnes) | Y | Digestate is high RAN |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y | Checked on google confirmed producers |
| - About the storage |  |  |
| Has all of waste storage table been completed? | Y |  |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y | Applicant has proposed 13, 3000 ton field heaps/stockpiles several in areas which are at risk of flooding. RFI Resolved via RFI Applicant will store no more than 3000 tons overall in field heap storage. Field heaps in flood risk locations have been removed. |
| 18. Supporting documents |  |  |
| Has checklist been completed? | Y |  |
| 20. Confidentiality and national security |  |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| 21. Declaration |  |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | Y |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has the operator confirmed they will comply with the NRW pre-notification requirements. | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y |  |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no storage or spreading within 10 meters) (buffer zones needed) | Y |  |
| groundwater source protection zones (permit condition: no storage or spreading in SPZ1) (high risk deployment if in SPZ2) | n/a |  |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no storage or spreading within 50 meters) | n/a |  |
| location of all storage sites for the waste to be spread | Y | See above note on storage. Resolved via RFI |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | Y | Correctly buffered |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | n/a |  |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | Y | SSS/SAC River usk is adjacent to field 13, a 20 no spread zone has been placed between the spread area and SSSI/SAC |
| location of any public rights of way (shown on map) | Y |  |
| any buildings or houses within 250m (shown on map) | Y | Site specific odour management plan is required. Resolved via RFI |
| Within Groundwater Safeguard Zone | n/a |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Section 7 habitats - Section 7 species - Scheduled Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Hedges - Ditches | Y |  |
| What is the slope of the fields? | 0-7 degrees (transparent)- fields: all fields 7 -12 degrees (yellow)- fields: 1, 5, 6, 16, 15, 12 degrees plus (red)- fields: all red areas have been covered by a no spread zone. |  |

| Waste Storage | Y N n/a | Comments |
| --- | --- | --- |
| Is waste storage occurring at the place where waste is being deployed? | Y |  |
| Is only waste as specified in this deployment being stored in the proposed storage locations? i.e. slurry in lagoons Note: There may be multiple deployments using one storage location | Y | See previous comments- Resolved via RFI. |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in the same place one constructed within the last two years? or in different location? | Different |  |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in a field: - liable to flooding or becoming waterlogged, or - within 50 metres of a borehole, spring or well or - within 10 metres of surface water or a land drain - within 30 meters of a watercourse on steeply sloping land | Y | Several field heaps were in storage in areas liable to flood and/or become waterlogged. Resolved via RFI. |
| If the waste is non-stackable is the waste stored within either a vessel, a covered building, or on an impermeable surface | n/a |  |
| If the waste is high RAN and it is being stored within 200 metres of a European Site (SAC/SPA), Ramsar Site or a Site of Special Scientific Interest (SSSI) is the waste being covered to prevent loss of ammonia? (condition 3.2.4 of the permit) | n/a |  |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | Y |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | Y | 19 09 02- Iron and Aluminium 19 06 06- plastics, glass, odour |
| - If yes; have they been tested for? | Y |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y |  |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y |  |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | Y |  |
| - Soil physical properties (e.g. organic matter and soil structure) | Y |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| Timing of application- if any of the wastes are high RAN, are they being spread OUTSIDE of the closed periods | Y |  |
| Groundwater Safeguard Zone for nitrate- if any of the wastes are high RAN, are the application rates less than 50 m3/ha, with a minimum 3 week gap between applications | n/a |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | Y |  |
| - Included the nutrient level in their environmental risk assessment | Y |  |
| - Stated the source of information that they used to set the nutrient level. | Y |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y | Odour and flooding issues need to be addressed. Resolved following RFI |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need, as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If the cropping is any of the following, is the maximum nitrogen limit in line with that given in The Water Resources (Control of Agricultural Pollution) (Wales) Regulations 2021: - 0 kg/ha- Field beans, Peas - 120 kg/ha- Sugar beet - 150 kg/ha- Asparagus, Carrots, Forage maize. Radish, Spring barley, Swedes - 180 kg/ha- Spring-sown wheat, Winter barley - 220 kg/ha- Autumn or early winter sown wheat | Y | Following RFI 2: Fields 1,5,6,7 are growing forage maize. nitrogen additions are too high. Resolved with reduced spread rates in RFI 3. |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | n/a |  |
| - Is it only for operational purposes? | n/a |  |
| - Have individual application rates for each waste been given? | n/a |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | n/a |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients given by the applicant match NRW calculations? | Y |  |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y | PAN-019349 |
| - How much material was spread? | Y |  |
| - At what application rate material was spread? | Y |  |
| - The nutrients supplied by the material? | Y |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | Y |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | Y | River Usk |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | Y | Flooding and odour. Flooding risk and odour risk resolved via RFI, risk assessment amended and a site specific odour management plan have been provided. |
| - Pathways | Y | Flooding and odour. Flooding risk and odour risk resolved via RFI, risk assessment amended and a site specific odour management plan have been provided. |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | Y | Flooding and odour. Flooding risk and odour risk resolved via RFI, risk assessment amended and a site specific odour management plan have been provided. |
| - Measures that will minimise or mitigate the impact will be put in place | Y | Flooding and odour Flooding risk and odour risk resolved via RFI, risk assessment amended and a site specific odour management plan have been provided. |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | Y | PAN-023888 Appendix 4 |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | N |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | Y |
| No FORM 1 completed: HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site. No impact pathway to note. Applicant has inserted buffer zones around field edges and slopes to prevent runoff issues. A 20m no spread area is in place near The SSSI Fields which are at risk of flooding have had field heap storage removed and spreading will only take place in acceptable conditions. CoGAP will be observed. A site specific Odour management plan and risk assessment is in place. All springs and water courses have been correctly buffered. |  |

Comments

Consultation responses: Consultees confirmed buffer zone between spread areas and the SSSI/SAC are appropriate.

MyMap/location and slope assessments: MyMap/location assessment: PAN-023888 Mapping Report Slope assessment 0-7 degrees (transparent)- fields: all fields 7 -12 degrees (yellow)- fields: 1, 5, 6, 16, 15, 12 degrees plus (red)- fields: all red areas have been covered by a no spread zone. Field number: 1, 5, 6, 16, 15 Slope of the land: 7-12 Soil type: medium Wastes types applied (solid, liquid, sludge etc.): liquid, sludge cake Cropping type: grass (under RFI) Method of application: rear discharge spread for sludge cake, trailing shoe spreader Weather conditions before during and after spreading: CoGAP will be followed, spreading will only take place Other risk factors not mentioned above: n/a Conclusions (is run off risk high?): low

CPH details: fields 1-13: 60/034/0021- J Mayho 15-16: none available by applicant or on GIS- Sl Jones 20-22- 60/030/0079: GK & RJ underwood.

Protection of habitats and ecosystems: Fields appear to be of established agricultural use and contain no semi natural habitat.

Agricultural benefit calculations: Calculations match those of the applicant save for rounding errors.

Request for information: - Consent landowner/occupier - Consent waste codes - CPH - Storage amount of waste - Field floods - SSRA flood risk - Odour risk - Cropping - Eia images RFI Response 1: 09/01/24 Resolved all points except 1, 3, and 8 RFI 2: sent 12/01/24 1. Landowner consent for fields 15 and 16 2. CPH provided was incorrect for all fields 3. Response regarding cropping was unclear and did not match the crop which appear in fields on EIA evidence RFI Response 2: 18/01/24 1. Corrected land owner consent 2. Provided acceptable CPH numbers 3. Corrected cropping, fields 1, 5, 6 and 7 are growing forage maize. However nutrient additions for nitrogen are now too high. RFI 3: 19/01/24 1. Requested reduced application rates to bring nitrogen additions in line with control of agri pollution regs. RFI Response 3: 22/01/24 1. Provided amended agben and spread rates.

Other information to note:

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date | 23/01/24 |
| ## Effective date | 23/01/24 |
| ## Permitting Officer Date | Ethan Banfield 23/01/24 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 23/01/24 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 950 |
| Sign off | 10 |
| Issuing | 60 |
| Total | 1060 |