# Natural Resources Wales permitting decisions

# Bespoke permit

We have decided to grant the permit for Garth Construction Compound operated by Hochtief (UK) Construction Ltd.

The application number is PAN-020734

The permit number is EPR/DB3190FD

Trade discharge of 1,300m³/d derived through the drilling operations at located at Garth Construction Compound. Treatment is through settlement lagoons, usage of polyaluminium chloride hydroxide sulphate coagulant and anionic polymer flocculant and carbon dioxide to treat pH.

The discharge is comprised solely of settled process water derived from drilling operations and surface water drainage from Garth Construction Compound.

Abstraction Licence, with elver screen, for the purpose of dewatering – WA/065/0006/0023

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

Purpose of this document

This decision document:

- explains how the application has been determined

- provides a record of the decision-making process

- shows how all relevant factors have been taken into account

- justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

Structure of this document

- Annex 1 the decision checklist

- Annex 2 the external consultation and web advertising

# Annex 1: decision checklist

This checklist should be read in conjunction with the Duly Making checklist.

| ### Aspect | ### Justification / Detail | Criteria met |
| --- | --- | --- |
|  |  | Yes |
| Consultation |  |  |  |
| Scope of consultation | The consultation requirements were identified and implemented. The decision was taken in accordance with RGN 6 High Profile Sites, our Public Participation Statement and our Working Together Agreements. | ü |
| ### Responses to external consultations and web advertisement | No responses were received in relation to the web advert or external consultation. Therefore the decision was taken in accordance with our guidance. | ü |
| Operator |  |  |  |
| Control of the facility | We are satisfied that the applicant (now the operator) is the person who will have control over the operation of the facility after the grant of the permit. The decision was taken in accordance with EPR RGN 1 Understanding the meaning of operator. | ü |
| European Directives |  |  |  |
| Applicable directives | All applicable European directives have been considered in the determination of the application. | ü |
| WFD | In light of the conclusions of a detailed compliance assessment (Stage 3), and taking account of the advice received from technical specialist advisors, it has been established that the activity/project has no potential to cause deterioration of any water body or prevent a water body or WFD Protected Area from meeting its objectives, taking into account any legally binding avoidance measures or restrictions as applicable, either alone or in-combination with other activities. |  |
| The site |  |  |  |
| Extent of the site of the facility | The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility including discharge points. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary. | ü |
| Biodiversity, Heritage, Landscape and Nature Conservation and a designated site under the Wildlife and Countryside Act. | The application is within the relevant distance criteria of a site of heritage, landscape or nature conservation, and/or protected species or habitat . A full assessment of the application and its potential to affect the sites/species/habitat has been carried out as part of the permitting process. We consider that the application will affect the features of the site/species/habitat. Operational controls/emission limits have been placed on the permit to mitigate against these effects. Following internal consultation and discussions with local officers, we have decided that Outlet 2 should be used for all detections, which is direct to the Afon Glaslyn and will benefit from continuous flow and dilution as Outlet 1 is to a seasonally dry ditch | ü |
| Habitat Regulation Assessment for a designated site under the EU Habitats Directive | Assessment of Likely Significant Effect: The project has been screened for likelihood of significant effects and, taking account of the advice received from protected sites advisors, is considered not likely to have a significant effect on any Natura 2000/Ramsar site (As documented in section 3.2 of OGN 200 form 1, or section 5 if applicable) Appropriate assessment: In light of the conclusions of an appropriate assessment, and taking account of the advice received from protected sites advisors, it has been established that the project will not adversely affect the integrity of any Natura 2000/Ramsar site, taking into account any conditions or restrictions as applicable, either alone or in-combination with other plans and projects. (As documented in section 4 of OGN 200 form 1, and section 5 if applicable) |  |
| Environmental Risk Assessment and operating techniques |  |  |  |
| Environmental risk | We have reviewed the operator's assessment of the environmental risk from the facility. The operator’s risk assessment is satisfactory. The assessment shows that, applying the conservative criteria in our guidance on Environmental Risk Assessment, all emissions may be categorised as environmentally | ü |
| Operating techniques | We have reviewed the techniques used by the operator and compared these with the relevant guidance notes. The proposed techniques / emission levels for priorities for control are in line with the benchmark levels contained in the TGN and we consider them to represent appropriate techniques for the facility. | ü |
| The permit conditions |  |  |  |
| Emission limits | We have decided that emission limits should be set for the parameters listed in the permit. It is considered that the numeric limits described below will prevent significant deterioration of receiving waters. We have imposed numeric limits because either a relevant environmental quality or operational standard requires this. The proposed discharge is direct to the Afon Glaslyn SAC waterbody and Glaslyn SSSI. There is a sluice located approx. 2.3km downstream of the point of discharge and therefore not within tidal reach to provide dilution. Emission limits have been set to 60mg/l for SS and pH between 6-9. The discharge pipe will be weighted within the waterbody channel so risk of riparian scouring reduced, however flow rate limit set at maximum of 6l/s to reduce risk of scouring to waterbody bed. | ü |
| Monitoring | We have decided that monitoring should be carried out for the parameters listed in the permit, using the methods and to the frequencies specified. These monitoring requirements have been imposed in order to flow monitoring to the operators monthly reporting with table S3.2. We made these decisions in accordance with EPR 7.01. Based on the information in the application we are satisfied that the operator’s techniques, personnel and equipment have either MCERTS certification or MCERTS accreditation as appropriate. | ü |
| Reporting | We have specified reporting in the permit. We made these decisions in accordance with EPR 7.01 under Table 4.1 and Table 4.2 to include monitoring data to be provided monthly by the operator. | ü |
| Considerations of foul sewer | We agree with the operators justification for not connecting to foul sewer. | ü |
| Operator Competence |  |  |  |
| Environment Management System | There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. | ü |
| Sustainable Management of Natural Resources (SMNR) |  |  |
| Considerations of SMNR - Compliance with our General Purpose | We are satisfied that this decision is compatible with our general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources” | ü |
| Environment Wales Act 2016 |  |  |
| Biodiversity and resilience of ecosystems duty | Section 6 of the Environment Wales Act 2016 requires that we seek to maintain and enhance biodiversity in the exercise of our functions, and in so doing promote the resilience of ecosystems, in a manner that is consistent with the proper exercise of our functions. NRW is satisfied that in this case, we have taken into account and had due regard to this duty in so far as it is consistent with the function of determining an application for a standalone water discharge activity permit under EPR2016. | ü |