| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-023961 |
| Permit Reference Number: | EPR-GP3792SK |
| Permit Holder: | ByProduct Recovery Limited |
| Site Address: | Tylebrithos, Cantref, Brecon, LD3 8LR |
| Grid Reference: | Farm location: 225469 305368 RFI – incorrect NGR Fields: RFI – none of the grid references have enough digits and do not match those given on farmer consents Tylibrithos 2 SO05272524 3 SO05332534 4 SO05332534 Aber 2 SO07412695 Pentwyn 2 SO07642617 3 SO07442624 4 SO07512650 Pencelli Court 1 SO09232570 2 SO09252551 3 SO09182532 4 SO09312523 Storage: none provided on farmer consents RFI SO 05043 25672 SO 05116 25727 SO 05223 26135 SO 05542 25521 SO 05522 25525 SO 07706 25983 SO 07552 26561 SO 07038 26433 SO 06815 26309 SO 09129 25674 SO 09205 25200 SO 09088 25282 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: GwastraffaGorfodi.Canolbarth@cyfoethnaturiolcymru.gov.uk Environment team: environmentteam.southpowys@cyfoethnaturiolcymru.gov.uk NLO: Don.Rother@environment-agency.gov.uk |
| Date due: | 12/12/2023 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating B- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y |  |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and deployment fee- Charging Scheme |
| Fee on MyNRW | Y | Amount:1837 Date cleared on MyNRW:24/11/23 Transaction number: 2124420 |
| Location map | Y |  |
| Benefit statement | Y |  |
| Waste Analysis | Y |  |
| Soil Analysis | Y |  |
| EIA Agri Regulations evidence | Y |  |
| Site Specific Risk Assessment | Y | High risk location only |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| - About the Permit |  |  |
| ## Has there been any pre-application discussions? | N |  |
| ## Has the permit number been provided? | Y |  |
| ## Has the permit holder name been provided? | Y |  |
| ## What permit is being deployed under? | SR2010 No4 |  |
| - Contact details |  |  |
| ## Has contact information been provided? | Y |  |
| ## Has the technically competent manager details been provided? | Y |  |
| ## Has the nominated competent person details been provided? | Y |  |
| - About the land you want to treat |  |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | N | Incorrect RFI |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | For main farm fields only. No CPH provided far any other fields RFI |
| ## Has all of parcels of land table been completed? | Y | GR differ to those on farmer consents and are not 12 digit RFI |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y | 48.59 |
| Is the land to be treated within a 10 miles radius of the main address? | Y | NGR for main address incorrect RFI |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | Y |  |
| ## Has written consent of all owners(s) or occupiers(s) been given? | Y | Do not contain storage locations, 190606 waste RFI |
| - Previous land treatment |  |  |
| Has all of the previous land treatment table been completed? | Y | None in past 12 months |
| - Which risk band does the activity fall within? - SR2010No4 List A wastes (Lower risk) - Location risk - Low risk deployment - Medium risk (1) deployment - Medium risk (2) deployment - High risk deployment |  |  |
| ## Which risk band does the activity fall within? | High |  |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) or High risk deployment has a SSRA been provided? | Y |  |
| - Additional information on sensitive receptors |  |  |
| ## What type of risk assessment has been submitted? | site‐specific risk assessment benefit statement |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | n/a |  |
| If no have details of the assessment evidence to support this conclusion been provided? | Y |  |
| - About the waste |  |  |
| Have all parts of the waste-types table been completed? - List of waste code (6 digit) - Description of waste (this must be an accurate description specific to this waste that refers to the List of Waste written description) - Physical form of waste/Is it stackable or non-stackable - Is the waste high in readily available nitrogen? - Producer of waste & permit number (if applicable) - Total amount being spread/used (tonnes) | Y | 190606 is high RAN 190902 is not high RAN |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y |  |
| - About the storage |  |  |
| Has all of waste storage table been completed? | Y |  |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y |  |
| 18. Supporting documents |  |  |
| Has checklist been completed? | Y |  |
| 20. Confidentiality and national security |  |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| 21. Declaration |  |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | Y |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has the operator confirmed they will comply with the NRW pre-notification requirements. | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y | See comments section below for notes |
| Do the grid references match the site plan? | Y | Main address NGR incorrect. Field NGR not 12 digit and do not match farmer consents RFI |
| If there is storage; do the grid references match the site plan? | Y | Locations H and I not shown on application maps. 2x Lagoons not yet constructed. 1x marked as a pit. RFI |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no storage or spreading within 10 meters) (buffer zones needed) | Y | See comments |
| groundwater source protection zones (permit condition: no storage or spreading in SPZ1) (high risk deployment if in SPZ2) | n/a | None in range |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no storage or spreading within 50 meters) | n/a | None in range |
| location of all storage sites for the waste to be spread | Y | See comments RFI |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | n/a | None in range |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | Y | Issue North of ABER2 not shown RFI |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | Y |  |
| location of any public rights of way (shown on map) | Y |  |
| any buildings or houses within 250m (shown on map) | Y |  |
| Within Groundwater Safeguard Zone | n/a | None in range |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Section 7 habitats - Section 7 species - Scheduled Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Hedges - Ditches | Y | See Comments RFI |
| What is the slope of the fields? | See comments RFI |  |

| Waste Storage | Y N n/a | Comments |
| --- | --- | --- |
| Is waste storage occurring at the place where waste is being deployed? | Y | Benefit statement refers to a storage pit for liquid, question the construction. i.e. does it comply with requirements under TGN8.01 RFI A number of storage locations are within a reasonable distance although there are two proposed locations for new lagoons. Again ask for confirmation that construction will comply with TGN8.01 RFI |
| Is only waste as specified in this deployment being stored in the proposed storage locations? i.e. slurry in lagoons Note: There may be multiple deployments using one storage location | Y |  |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in the same place one constructed within the last two years? or in different location? | n/a | None in last two years |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in a field: - liable to flooding or becoming waterlogged, or - within 50 metres of a borehole, spring or well or - within 10 metres of surface water or a land drain - within 30 meters of a watercourse on steeply sloping land | n/a |  |
| If the waste is non-stackable is the waste stored within either a vessel, a covered building, or on an impermeable surface | Y | Nurse tanks, lagoons and a Waste pit See comments above re pit. |
| If the waste is high RAN and it is being stored within 200 metres of a European Site (SAC/SPA), Ramsar Site or a Site of Special Scientific Interest (SSSI) is the waste being covered to prevent loss of ammonia? (condition 3.2.4 of the permit) | Y | Andigestion waste is high RAN. Storage locations I and L are within 200m of SAC River Usk but are not open containers. Other storage locations for this waste are not within 200m |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | n/a |  |
| - Sulphur | Y | n/a for Andigestion |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | n/a |  |
| - If yes; have they been tested for? | n/a |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y |  |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y | States spring summer autumn. Too vague RFI |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | Y | pH data in table 6-14 is incorrect and does not align with the lab analysis RFI |
| - Soil physical properties (e.g. organic matter and soil structure) | Y |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| Timing of application- if any of the wastes are high RAN, are they being spread OUTSIDE of the closed periods | Y | Andigestion is High RAN |
| Groundwater Safeguard Zone for nitrate- if any of the wastes are high RAN, are the application rates less than 50 m3/ha, with a minimum 3 week gap between applications | Y | States not in NVZ RFI |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | n/a |  |
| - Included the nutrient level in their environmental risk assessment | n/a |  |
| - Stated the source of information that they used to set the nutrient level. | n/a |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y |  |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need, as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If the cropping is any of the following, is the maximum nitrogen limit in line with that given in The Water Resources (Control of Agricultural Pollution) (Wales) Regulations 2021: - 0 kg/ha- Field beans, Peas - 120 kg/ha- Sugar beet - 150 kg/ha- Asparagus, Carrots, Forage maize. Radish, Spring barley, Swedes - 180 kg/ha- Spring-sown wheat, Winter barley - 220 kg/ha- Autumn or early winter sown wheat | n/a | Grass |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | Y | Benefit statement table 5 |
| - Is it only for operational purposes? | Y |  |
| - Have individual application rates for each waste been given? | Y |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | Y |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y | See comments above re timing RFI |
| - Have they stated when the waste will be spread? | Y | See comments above re timing RFI |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients given by the applicant match NRW calculations? | Y |  |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | n/a |  |
| - How much material was spread? | n/a |  |
| - At what application rate material was spread? | n/a |  |
| - The nutrients supplied by the material? | n/a |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | Y |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | Y |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | Y |  |
| - Pathways | Y | Does not consider flooding RFI |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | Y |  |
| - Measures that will minimise or mitigate the impact will be put in place | Y |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | Y |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | N |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | Y |
| No FORM 1 completed: HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site. The activity management includes appropriate controls in accordance with TGN8.01, RB209, Site Specific Risk Assessment and the SR2010 No4 permit. The controls include; - No spread buffers zones of 20metres where the SSSI and SAC are adjacent to or in close proximity to the field boundaries. - Limiting factors on spreading rates. - Suitable spreading methods. - Restrictions on timing of the spread which includes not spreading during or prior to adverse weather conditions |  |

Comments

Consultation responses: Reviewed and closed. Comments either required no action or have been identified and added to the RFI where relevant.

MyMap/location and slope assessments: also see MyMap report for restricted information MyMap/location assessment Detailed river network, 10m Watercourse S of Ty2 and Ty3, shown on application map with buffer. None at storage locations D and E. None at storage locations A, B, C Watercourse W of Aber 2, shown on application map with buffer None at storage locations H and I. Watercourse flowing N between Pentwyn 2 and 3 shown on application map with buffer. Pond adjacent to Pentwyn 2, 3 and 4 shown on application map with buffer. None at storage locations F and G. Watercourse E of Pencelli 1 and 2 shown on application map with Buffer. Watercourse S of Pencelli 2 and N of pencelli 3 and 4 shown on application map with buffer Watercourse within Pencelli 4 shown on application map with buffer. None at storage locations J, K, L. Groundwater protection zones None Any well, spring (or issue) or borehole used / not used for the supply of water for human consumption including from a private water supply or supplies within 50m. None at Ty2 and Ty3. None at storage location A, B, C None at Aber 2 None at Pentwtn 2, 3, 4 None at storage locations H and I None at Pencelli 1,2,3,4 Any well, spring (or issue) or borehole used / not used for the supply of water for human consumption including from a private water supply or supplies within 250m. Spring E of Ty4 shown on application map. None at Ty2, 3 None at storage locations A, B, C. Issue N of Aber 2 which flows E to river Usk not shown on application maps RFI None at Pentwyn 2, 3, 4 2x NALD Impounds, 1x deregulated abstraction, 2x licenced abstractions – these include 4x water resource permit points NW of storage location I. Shown on application map. None at Pencelli 1,2,3,4 European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m None at Ty 2,3,4 None at storage locations A, B, C SSSI River Usk (upper Usk) N and River Usk (Tributaries) W of Aber2, both are Special Areas of Conservation. Shown on application map SSSI River Usk (upper Usk) N Pentwyn 4 and River Usk (Tributaries) S of Pentwyn 2, both are Special Areas of Conservation. Shown on application maps. SSSI River Usk (Tributaries) W of storage locations H and I, SSSI is also SAC. Shown on application maps. SSSI River Usk (upper Usk) E of Pencelli fields and River Usk (Tributaries) passing between Pencelli 2, 3, and 4, both are Special Areas of Conservation. Shown on application maps. High RAN storage within 200m of European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI. Andigestion waste is high RAN. Storage locations I and L are within 200m of SAC River Usk but are not open containers. Other storage locations for this waste are not within 200m. Other protected sites within 500m Entire deployment area is within Brecon Beacons National Park not mentioned in application RFI Ancient woodland S and NE of Ty2,3,4 and NE, NW of storage location C shown on application maps Scheduled Monument - Brynich Aqueduct (Brecknock & Abergavenny Canal), NE of Aber 2. Shown on application map. Ancient woodland N, SE and W of Aber2 shown on application maps Ancient woodland E of Pentwyn 4, within Pentwyn 2, 4 and S of Pentwyn 3 all shown on application maps with buffer within 2 and 4 Ancient woodland S of Pencelli 4 out of range on application map, risk low. Distance from Main Farm Address Main address Grid Reference (X/Y coordinates) is incorrect RFI Flood Risk No flood risk at Tylebrithos fields No risk at fields at Pentwyn. Low risk to a small area of Aber2 Pencelli fields 1 and 2, high flood risk to the E boundary, high risk to the S of 2. low flood risk to the majority of the fields. RFI Pencelli fields 3 and 4, high risk to the N boundary. RFI Temporary stockpiles are not in flood risk zones High RAN storage within 200m of European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI. Andigestion waste is high RAN. Storage locations I and L are within 200m of SAC River Usk but are not open containers. Other storage locations for this waste are not within 200m. Slope assessment Pentwyn 3 and 4 0-7 Field number: Tylebrithos 2 RFI Slope of the land: 0-7, 7-12, 12+ sloping to a watercourse to the S Soil type: Medium Wastes types applied (solid, liquid, sludge etc.): liquid, sludge, liquid sludge Cropping type: Grass Method of application: Rear discharge spreader for sludge cake, dribble bar for liquid sludges Weather conditions before during and after spreading: IAW COGAP Other risk factors not mentioned above: buffers not applied on application maps Conclusions (is run off risk high?): Yes if buffers not applied Field number: Tylebrithos 3 RFI Slope of the land: 0-7, 7-12, 12+to the S boundary sloping to a watercourse to the S Soil type: Medium Wastes types applied (solid, liquid, sludge etc.): liquid, sludge, liquid sludge Cropping type: Grass Method of application: Rear discharge spreader for sludge cake, dribble bar for liquid sludges Weather conditions before during and after spreading: IAW COGAP Other risk factors not mentioned above: buffers not applied on application maps Conclusions (is run off risk high?): Yes if buffers not applied Field number: Tylebrithos 4 RFI Slope of the land: 0-7, 7-12, 12+to the S boundary sloping to a watercourse to the SE Soil type: Medium Wastes types applied (solid, liquid, sludge etc.): liquid, sludge, liquid sludge Cropping type: Grass Method of application: Rear discharge spreader for sludge cake, dribble bar for liquid sludges Weather conditions before during and after spreading: IAW COGAP Other risk factors not mentioned above: buffers not applied on application maps Conclusions (is run off risk high?): Yes if buffers not applied Field number: Aber 2 RFI Slope of the land: 0-7 with some 7-12 12+ dissecting S-N falling to SSSI Soil type: Medium Wastes types applied (solid, liquid, sludge etc.): liquid, sludge, liquid sludge Cropping type: Grass Method of application: Rear discharge spreader for sludge cake, dribble bar for liquid sludges Weather conditions before during and after spreading: IAW COGAP Other risk factors not mentioned above: buffers not applied on application maps Conclusions (is run off risk high?): Yes if buffers not applied Field number: Pentwyn 2, no spread buffers in place. RFI Slope of the land: 0-7, 12+. Soil type: Medium Wastes types applied (solid, liquid, sludge etc.): liquid, sludge, liquid sludge Cropping type: Grass Method of application: Rear discharge spreader for sludge cake, dribble bar for liquid sludges Weather conditions before during and after spreading: IAW COGAP Other risk factors not mentioned above: buffers applied on application maps within ancient woodland Conclusions (is run off risk high?): No Field number: Pencelli fields. Buffer in place on application maps. Slope of the land: 0-7, minor 7-12, 12+ to E boundary Soil type: Medium Wastes types applied (solid, liquid, sludge etc.): liquid, sludge, liquid sludge Cropping type: Grass Method of application: Rear discharge spreader for sludge cake, dribble bar for liquid sludges Weather conditions before during and after spreading: IAW COGAP Other risk factors not mentioned above: buffers applied on application maps although also see notes on flood zones Conclusions (is run off risk high?): No, if buffers are applied and flood issues addressed in the RFI

CPH details: CPH provided 52/045/0010 A Phillips Tylebrithos, Cantref, Brecon. LD3. Shown on app form and farmer consent. Correct on MyMap for fields at Tylebrithos. ABER2 is split on MyMap in several parcels, some of which show as 52/045/0010 but with some parcels showing as 52/045/0020 EXECUTORS OF JRC LLOYD, ABERCYNRIG, LLANFRYNACH, BRECON, LD3 7AQ RFI for clarification CPH not provided for Pencelli Court, Pencelli, Brecon. LD3 7LX RFI MyMap shows as 52/051/0031 D MORGAN AND SONS, PENCELLI COURT FARM, LD3 7LX CPH not provided for Pentwyn Farm, Llanfrynach, Brecon. LD3 7AQ RFI MyMap shows as 52/051/0013 EJ GM AND KJ PROTHERO, PENTWYN, LLANFRYNACH, LD3 7BQ

Protection of habitats and ecosystems Evidence provided

Agricultural benefit calculations: See comments regarding pH for soils and wastes RFI Cefn Phosphate – nominal addition of 4.28 to all fields Potash – zero addition for all fields Crai Phosphate – nominal addition of 9.92 to all fields Potash – zero addition for all fields Hirwaun Phosphate – nominal addition of 2.36 to all fields Potash – zero for all fields Elan Phosphate – nominal addition of 2.4 to all fields Potash – zero for all fields Llyswen Phosphate – high addition of 329.98 to fields Tylibrithos 3, 4, Pentwyn 2, 3. 241.53 to Pentwyn 4, Pencelli 1-4. Moderate addition of 17.01 to Aber 2 which is below the offtake Potash – zero to all fields Pontsticill Phosphate – addition of almost twice the offtake for all fields with the exception of Aber 2 which shows a nominal addition. Potash – Zero for all fields Tal-y-Bont Phosphate – moderate addition of 35.37 which almost matches the offtake for all fields Potash – zero for all fields Whitbourne Phosphate – nominal addition of 9.71 for all fields which is below the offtake Potash – zero for all fields Andigestion Phosphate – nominal over addition with a difference of 9.96 to all fields with the exception of Aber 2 which has a moderate addition of 21.02 which is below the offtake. Potash – addition of 104.4 to all fields with the exception of Aber 2 which has a moderate addition of 44.59 which is below the offtake for all fields

Request for information: BB, 01/02/24 – RFI sent 01/02/2024. Due 15/02/24. Main points of the RFI relate to - Main farm NGR incorrect - CPH numbers not all provided - Landowner/occupier consent forms incomplete - Mapping errors / omissions - Storage lagoon and pit construction - Field pH inconsistent with analyses - Waste pH inconsistent with analyses - Flood risk not considered BB, 08/02/24 - Response received 07/02/24. Not all points were satisfied. Point 4a required the applicant to ‘Provide new landowner/occupier consents which include the waste storage locations and identification of the 19 06 06 Andigestion waste for each of the fields on which it is to be spread. The new consents must be signed and dated by the landowners/occupiers; this is to ensure that the landowners/occupiers are fully aware of the proposed activities.’ The consents provided in the RFI response are the originally provided documents with manual amendments and do not include the storage locations nor are they redated by the landowners/occupiers. Therefore, it cannot be verified if the landowners/occupiers are fully aware of, and fully agree to, the proposed activities. This is a requirement that applies to all mobile plant and site based waste activities to reduce waste crime. RFI 2 RFI 2 sent 08/02/24. Due 22/02/24 Response received 20/02/24

Other information to note: Waste pH comments in section 7.2 of the benefit statement are incorrect and do not align with the lab analysis RFI Waste interpretation for Andigestion not provided RFI

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date | 22/02/24 |
| ## Effective date | 22/02/24 |
| ## Permitting Officer Date | Bryan Burn 22/02/24 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 22/02/2024 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 1740 |
| Sign off | 10 |
| Issuing | 30 |
| Total | 1840 |