# Natural Resources Wales permitting decisions

# Bespoke permit

We have decided to grant the permit for Peterstone Lakes Golf Club operated by P W Millar Golf LImited.

The application number is PAN-021801

The permit number is EPR/DB3191FJ.

This permit regulates an unregulated discharge of domestic sewage effluent from a  golf club which has been in existence since 1991. The total daily volume is 10m3/day. The discharge is treated by passing through a cesspit and grey water feed pipe where it then passes through a digester system before discharging to the reen leading to Gout Fawr Pill. The treatment system has been upgraded by changing the pipework and air blower system.

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

Purpose of this document

This decision document:

- explains how the application has been determined

- provides a record of the decision-making process

- shows how all relevant factors have been taken into account

- justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

Structure of this document

- Annex 1 the decision checklist

# Annex 1: decision checklist

This checklist should be read in conjunction with the Duly Making checklist.

| ### Aspect | ### Justification / Detail | Criteria met |
| --- | --- | --- |
|  |  | Yes |
| Consultation |  |  |  |
| Scope of consultation | The consultation requirements were identified and implemented. The decision was taken in accordance with RGN 6 High Profile Sites, our Public Participation Statement and our Working Together Agreements. | ü |
| ### Responses to web publicising | No responses were received from web publicising. | ü |
| Operator |  |  |  |
| Control of the facility | We are satisfied that the applicant (now the operator) is the person who will have control over the operation of the facility after the grant of the permit. The decision was taken in accordance with EPR RGN 1 Understanding the meaning of operator. | ü |
| European Directives |  |  |  |
| Applicable directives | All applicable European directives have been considered in the determination of the application. | ü |
| WFD | WFD stage 2 scoping has been completed and the activity/project is considered as having no risk of causing deterioration or preventing any water body or WFD Protected Area from reaching its objectives and taking account of the advice from technical officers - is considered compliant with the WFD Regulations 2017. | ü |
| The site |  |  |  |
| Extent of the site of the facility | The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility including discharge points. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary. | ü |
| Biodiversity, Heritage, Landscape and Nature Conservation and a designated site under the Wildlife and Countryside Act. | The application is within the relevant distance criteria of a site of heritage, landscape or nature conservation, and/or protected species or habitat . A full assessment of the application and its potential to affect the site has been carried out as part of the permitting process. We consider that the application will affect the features of the site. Emission limits have been placed on the permit to mitigate against these effects. A limit of 10mg/l Ammonia has been put on the permit in order to protect the special features of the Gwent Levels SSSI. Formal consultation has been carried out with Natural Resources Wales’s local Ecology and Conservation Officer. | ü |
| Habitat Regulation Assessment for a designated site under the EU Habitats Directive | HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site This discharge by virtue of the volume and the watercourse it is flowing into will render the discharge insignificant by scale and virtue. Gout Fawr Pill, the watercourse that flow into the Severn is large and substantial meaning there will be significant dilution of the discharge prior to it reaching to Severn SAC, SPA and RAMSAR. I can conclude no likely significant effect by virtue of the scale of the project. | ü |
| Environmental Risk Assessment and operating techniques |  |  |  |
| Environmental risk | We have carried out a risk assessment on behalf of the operator. The operator considers this risk assessment is satisfactory. The assessment shows that, applying the conservative criteria in our guidance on Environmental Risk Assessment, all emissions may be categorised as environmentally insignificant with the expectation of Ammonia.We have therefore imposed a limit of 10mg/l Ammonia on the permit in order to protect the special features of the Gwent Levels SSSI. | ü |
| Operating techniques | We have reviewed the techniques used by the operator and compared these with the relevant guidance notes. The proposed techniques / emission levels for priorities for control are in line with the benchmark levels contained in the “How to comply with your permit” and we consider them to represent appropriate techniques for the facility. | ü |
| The permit conditions |  |  |  |
| Emission limits | We have decided that emission limits should be set for the parameters listed in the permit. It is considered that the numeric limits described below will prevent significant deterioration of receiving waters. We have imposed numeric limits because either a relevant environmental quality or operational standard requires this. - ATU-BOD as O2 – 20mg/l - Suspended solids (measured after drying at 105o C) – 30mg/l - Ammoniacal nitrogen (expressed as N) – 10mg/l The limits have been set to ensure continued compliance with current and future WFD requirements of the Rhosog Fach Reen and to protect the receiving water course from pollution. The limits have been set in compliance with current guidance: No Deterioration policy. In addition, the limit of 10mg/l Ammonia has been set to protect the special features of the Gwent Levels SSSI. | ü |
| Considerations of foul sewer | We agree with the operators justification for not connecting to foul sewer. | ü |
| Operator Competence |  |  |  |
| Environment Management System | There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. | ü |
| Sustainable Management of Natural Resources (SMNR) |  |  |
| Considerations of SMNR - Compliance with our General Purpose | We are satisfied that this decision is compatible with our general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources” | ü |
| Environment Wales Act 2016 |  |  |
| Biodiversity and resilience of ecosystems duty | Section 6 of the Environment Wales Act 2016 requires that we seek to maintain and enhance biodiversity in the exercise of our functions, and in so doing promote the resilience of ecosystems, in a manner that is consistent with the proper exercise of our functions. NRW is satisfied that in this case, we have taken into account and had due regard to this duty in so far as it is consistent with the function of determining an application for a stand alone water discharge activity permit under EPR2016. | ü |