| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-024462 |
| Permit Reference Number: | EPR-AB3891CX |
| Permit Holder: | Mr Daniel James & Mrs Carys James |
| Site Address: | Trefwtial, Blaenannerch, Cardigan, Ceredigion, SA43 2AG |
| Grid Reference: | Farm location: SN 23733 48030 Fields: Trefwtial 16 SN 24342 48926 Trefwtial 17 SN 24340 48710 Trefwtial 18 SN 24280 48470 Rhosygadair 4 SN 24247 50153 Rhosygadair 5 SN 24339 50241 Rhosygadair 6 SN 24418 50292 Rhosygadair 7 SN 24616 50113 Land at Bigni 8 SN 21122 51123 Land at Bigni 9 SN 20930 51312 Storage: SN 24543 48682 SN 24252 50344 SN 20654 51375 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: GwastraffaGorfodi.Canolbarth@cyfoethnaturiolcymru.gov.uk Environment team: CeredigionEnvironmentTeam@cyfoethnaturiolcymru.gov.uk NLO: Malcolm.Dines@cyfoethnaturiolcymru.gov.uk |
| Date due: | 05/02/2024 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating C- 110% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | Welsh- No variations |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and deployment fee- Charging Scheme |
| Fee on MyNRW | Y | Amount: £1717.10 Date cleared on MyNRW: 09/01/23 Transaction number: 2125407 |
| Location map | Y |  |
| Benefit statement | Y |  |
| Waste Analysis | Y |  |
| Soil Analysis | Y |  |
| EIA Agri Regulations evidence | Y |  |
| Site Specific Risk Assessment | N/A | High risk location only |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| - About the Permit |  |  |
| ## Has there been any pre-application discussions? | N |  |
| ## Has the permit number been provided? | Y |  |
| ## Has the permit holder name been provided? | Y |  |
| ## What permit is being deployed under? | SR2010 No4 |  |
| - Contact details |  |  |
| ## Has contact information been provided? | Y |  |
| ## Has the technically competent manager details been provided? | Y |  |
| ## Has the nominated competent person details been provided? | Y |  |
| - About the land you want to treat |  |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y |  |
| ## Has all of parcels of land table been completed? | Y |  |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y | 49ha |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | Y |  |
| ## Has written consent of all owners(s) or occupiers(s) been given? | Y | As above |
| - Previous land treatment |  |  |
| Has all of the previous land treatment table been completed? | n/a | Previous deployment over 12 months ago |
| - Which risk band does the activity fall within? - SR2010No4 List A wastes (Lower risk) - Location risk - Low risk deployment - Medium risk (1) deployment - Medium risk (2) deployment - High risk deployment |  |  |
| ## Which risk band does the activity fall within? | Medium (1) |  |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) or High risk deployment has a SSRA been provided? | n/a |  |
| - Additional information on sensitive receptors |  |  |
| ## What type of risk assessment has been submitted? | benefit statement |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | N |  |
| If no have details of the assessment evidence to support this conclusion been provided? | Y | Fields are of established agriculture |
| - About the waste |  |  |
| Have all parts of the waste-types table been completed? - List of waste code (6 digit) - Description of waste (this must be an accurate description specific to this waste that refers to the List of Waste written description) - Physical form of waste/Is it stackable or non-stackable - Is the waste high in readily available nitrogen? - Producer of waste & permit number (if applicable) - Total amount being spread/used (tonnes) | Y | Dairy Partners -WP3231NB) Volac (BP3135EB) First Milk (XP3830UR) Dwyr Cymru 6835 t |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y | Google, DMS |
| - About the storage |  |  |
| Has all of waste storage table been completed? | Y |  |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y |  |
| 18. Supporting documents |  |  |
| Has checklist been completed? | Y |  |
| 20. Confidentiality and national security |  |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| 21. Declaration |  |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | N/A |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has the operator confirmed they will comply with the NRW pre-notification requirements. | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y |  |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no storage or spreading within 10 meters) (buffer zones needed) | Y | Field 18-17, correct buffer zone applied |
| groundwater source protection zones (permit condition: no storage or spreading in SPZ1) (high risk deployment if in SPZ2) | N/A | Not within a protection zone |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no storage or spreading within 50 meters) | N/A |  |
| location of all storage sites for the waste to be spread | Y |  |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | N/A | 51m from spread zone.No action was needed. |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | N/A |  |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | Y | SSSI present, due to 30m buffer zone beyond 500m |
| location of any public rights of way (shown on map) | Y | See RFI |
| any buildings or houses within 250m (shown on map) | Y | Yes, site specific odour management plan submitted |
| Within Groundwater Safeguard Zone | N/A |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Habitats - Section 7 species - Scheduled Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Hedges - Ditches | Y | See site mapping plan |
| Flood risk | N |  |
| What is the slope of the fields? | 0-7 degrees (transparent)- fields T16, T18, R4, R5, R6, R7, B9, B8 7 -12 degrees (yellow)- fields T17 |  |

| Waste Storage | Y N n/a | Comments |
| --- | --- | --- |
| Is waste storage occurring at the place where waste is being deployed? | Y | One nurse tank in a no-spread zone. Additional nurse tank on consent document |
| Is only waste as specified in this deployment being stored in the proposed storage locations? i.e. slurry in lagoons Note: There may be multiple deployments using one storage location | Y | Resolved see RFI 1 |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in the same place one constructed within the last two years? or in different location? | n/a |  |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in a field: - liable to flooding or becoming waterlogged, or - within 50 metres of a borehole, spring or well or - within 10 metres of surface water or a land drain - within 30 meters of a watercourse on steeply sloping land | n/a |  |
| If the waste is non-stackable is the waste stored within either a vessel, a covered building, or on an impermeable surface | Y | Nurse tank |
| If the waste is high RAN and it is being stored within 200 metres of a European Site (SAC/SPA), Ramsar Site or a Site of Special Scientific Interest (SSSI) is the waste being covered to prevent loss of ammonia? (condition 3.2.4 of the permit) | n/a | Wastes are not high in RAN |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | Y |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | Y | 02 05 02 – Fats and oils 19 09 02 – Iron and Aluminium |
| - If yes; have they been tested for? | Y |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | N/A |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | N/A |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | N/A |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | Y |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y | Maintenance level for all fields |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y | FE/578 |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y |  |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | N/A |  |
| - Soil physical properties (e.g. organic matter and soil structure) | Y |  |
| - Soil water content | Y |  |
| - Levelling the land | N/A |  |
| Timing of application- if any of the wastes are high RAN, are they being spread OUTSIDE of the closed periods | N/A |  |
| Groundwater Safeguard Zone for nitrate- if any of the wastes are high RAN, are the application rates less than 50 m3/ha, with a minimum 3 week gap between applications | N/A |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | N/A |  |
| - Included the nutrient level in their environmental risk assessment | N/A |  |
| - Stated the source of information that they used to set the nutrient level. | N/A |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y |  |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need, as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If the cropping is any of the following, is the maximum nitrogen limit in line with that given in The Water Resources (Control of Agricultural Pollution) (Wales) Regulations 2021: - 0 kg/ha- Field beans, Peas - 120 kg/ha- Sugar beet - 150 kg/ha- Asparagus, Carrots, Forage maize. Radish, Spring barley, Swedes - 180 kg/ha- Spring-sown wheat, Winter barley - 220 kg/ha- Autumn or early winter sown wheat | Y |  |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | N/A |  |
| - Is it only for operational purposes? | N/A |  |
| - Have individual application rates for each waste been given? | N/A |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | N/A |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients given by the applicant match NRW calculations? | Y |  |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y |  |
| - How much material was spread? | Y |  |
| - At what application rate material was spread? | Y | PAN-019723 |
| - The nutrients supplied by the material? | Y |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | N/A |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | N | Protected habitats and species were found within 500m. Due to distance and age of the records no risk assessment is required |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | N/A |  |
| - Pathways | N/A |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | N/A |  |
| - Measures that will minimise or mitigate the impact will be put in place | N/A |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | N |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | N/A |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | N |

Comments

Consultation responses: - No comments were left on the form

MyMap/location and slope assessments: MyMap/location assessment For details of the Mymap assessment refer to the DMS. Slope assessment Field number: Trefwtial 17 Slope of the land: 7-12 degrees Soil type: Medium Wastes types applied (solid, liquid, sludge etc.): Sludge Cropping type: Grass Silage Method of application: Dribble bar Weather conditions before during and after spreading: Following CGAP Other risk factors not mentioned above: Conclusions (is run off risk high?): No, low slope with minimal risk to watercourses

CPH details: CPH Number given in LDP1 form Applicant provided: 55/226/0017 & 52/220/002 Mr Gwyndaf Davies Found On GIS search: 16: 55/220/0002 17: 55/220/0002 18: 55/220/0002 R4: 55/220/0002 R5: 55/220/0002 R6: 55/220/0002 R7: 55/220/0002 B8: Unavailable B9: Unavailable The applicant has provided the correct CPH numbers provided in the request for information 1 response. hand

Agricultural benefit calculations: None of the EWC 19 09 02 wastes are high in aluminium so can be spread on fields with a soil p.H of less than 6. EWC 19 09 02 wastes DCWW Cray, Bryn Gwyn Capel Dewi and Bontgoch contain high levels of flocculated iron from the water treatment process. The fields in this deployment are all above a p.H of 5.5 so these wastes streams high in Iron can be spread on the fields in compliance with TGN 8.01 (metal containing wastes).. Request for information: 26/02/23 - Update the ABS so that includes a statement that only the waste from this deployment will be stored in the nurse tanks. The applicant has updated the ABS submitted which confirms only waste in this deployment will be stored in the nurse tanks. - Provide a new, signed consent document that contains no alterations. The applicant has Updated the Consent submitted that has no alterations. - Update the site plans to include a 10m no spread buffer zone around the footpath. The applicant has updated the ABS but still specifies only a 2m buffer zone will be implemented. - Relocate the nurse tank outside of the no spread buffer zone and update the location maps accordingly. The applicant has submitted maps with alternative location for the nurse tank outside of no spread buffer zone submitted, an additional nurse tank has also been added. 5. Update the ABS so that it includes the following Information: - How the nurse tanks are constructed to ensure they are impermeable. The applicant has Updated the section in the ABS and clarified the construction of the tanks. Which comply with the storage requirements of TGN 8.01. - If the nurse tanks have secondary containment or not. If they do not, the ABS needs to be updated to include the appropriate measures that will be in place to prevent and minimise leaks and spills. The applicant has updated the ABS to include clarity around structural features which minimise the risk of spills as well as protocol they will follow to additionally minimise risks. - Provide justification for the need in this increased phosphate or appropriately adjust the application rate of waste streams Dairy partner & Volac so that the phosphate added meets crop offtake. Applicant still maintains that no phosphate increase will occur as a result of the waste spread. Request for information 2 05/03/24 - Reduce the application rate of waste streams Dairy partner & Volac in fields Trefwtial 16 and Bigni 8 so that the phosphate added meets crop offtake. Through discussion with applicant and Owen, deciphered the applicant meant to include grain and straw as the crop for these fields as opposed to grain only. With this phosphate value, calculations are correct. - Update both the map index and ABS to state a 10m buffer zone will be implemented around the public footpath in Field Bigni 8 and 9 as opposed to the 2m buffer zone currently suggested. Through discussion Owen determined he had interpreted TGN 8.01 incorrectly and no buffer min was given. 2m buffer suggested is sufficient..

Other information to note:

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date | 07/03/24 |
| ## Effective date | 07/03/24 |
| ## Permitting Officer Date | Megan Tucker 07/03/23 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Anna Gale 07/03/23 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 3016 |
| Total | 3076 |