| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-024209 |
| Permit Reference Number: | EPR-GP3792SK |
| Permit Holder: | ByProduct Recovery Limited |
| Site Address: | Robeston Hall, Robeston West, Milford Haven, Pembrokeshire, SA73 3TL |
| Grid Reference: | Farm location: SM 88552 09623 Fields: Robeston Hall 13 SM8836109732 14 SM8810109804 15 SM8765609942 Storage: SM8847909694 RH- hardstanding South Leys fields and storage removed in RFI response 08/03/2024 A SM9493220386 B SM9519020433 C SM9506820606 D SM9484720713 E SM9501020996 SM8846109639 RH- Slurry pit/lagoon SM9532420748 SL- Lagoon |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: wasteregulation.swwales@cyfoethnaturiolcymru.gov.uk Environment team: pembsdistrictteam@cyfoethnaturiolcymru.gov.uk NLO: Don.Rother@environment-agency.gov.uk |
| Date due: | 09/01/2024 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating B- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | EA public register |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and deployment fee- Charging Scheme |
| Fee on MyNRW | Y | Transaction ref: 2124675 Date: 01/12/2023 Ref: BS/011223/09 BY PRODUCT RECOVERY PSCAPPBYPRO1385 amount: 1,837.00 |
| Location map | Y | SL&R MAPS |
| Benefit statement | Y | SL & R ABS Robeston checklist LPD1 Supplement wastes Waste interpretations - SLR LPD1 Supplement (2023) BPR Approval to sign on behalf (2023) |
| Waste Analysis | N | Preseli cake (Mar 2023) Preseli Liquid (Mar 2023) Strata florida cake (Mar 2023) Bolton Hill liquid sludge (June 2023) Bontgoch liquid (Jan 2023) LLECHRYD CAKE (March 2023) LLechryd Liquid Jan23 Bolton Hill Cake analysis is missing- provided in RFI response 08/03/2024 |
| Soil Analysis | Y | South Ley soils 5.2_Robeston soils |
| EIA Agri Regulations evidence | Y | Combined photos Field photos |
| Site Specific Risk Assessment | Y | SSRA 2023 |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| - About the Permit |  |  |
| ## Has there been any pre-application discussions? | N |  |
| ## Has the permit number been provided? | Y | GP3792SK |
| ## Has the permit holder name been provided? | Y | BYPRODUCT RECOVERY LIMITED |
| ## What permit is being deployed under? | SR2010 No4 |  |
| - Contact details |  |  |
| ## Has contact information been provided? | Y | Elinor Garside |
| ## Has the technically competent manager details been provided? | Y | Nigel Box |
| ## Has the nominated competent person details been provided? | Y | Elinor Garside |
| - About the land you want to treat |  |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | Only one given on LPD1 form- 55/550/0016. See end of document for further comments RFI response 08/03/2024- applicant has re-confirmed the CPH number for RH fields. While this doesn’t match our records this has been accepted. |
| ## Has all of parcels of land table been completed? | Y |  |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y | 45.05 |
| Is the land to be treated within a 10 miles radius of the main address? | Y | 7 miles |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | N | Mr Geoffrey Harris- name and address only. No phone number or email address on the form. This doesn’t match with information from the CPH numbers. RFI response 08/03/2024- contact number given- 07816765886 |

| ## Has written consent of all owners(s) or occupiers(s) been given? We will require a document signed and dated by the landowner/occupier that includes: - the farm name(s) - field reference(s) - waste(s) to be spread on each field - waste storage locations | N | Consent document is “Robeston checklist”- only includes the 3 Robeston Hall fields. This doesn’t match with information from the CPH numbers. - Farm name given - Field references given - Wastes to be spread- just codes - Storage location given No consent given for South Leys fields. RFI response 08/03/2024- South Leys fields removed from application. |
| --- | --- | --- |
| - Previous land treatment |  |  |
| Has all of the previous land treatment table been completed? | N/A | States no previous spreading has occurred. Review of MyMap shows the Robeston Hall fields have been included in 3 previous deployment applications, but longer than 12 months ago- PAN-001336 (Mar 2017), PAN-011040 (Aug 2020), PAN-015760 (Nov 2021). |
| - Which risk band does the activity fall within? - SR2010No4 List A wastes (Lower risk) - Location risk - Low risk deployment - Medium risk (1) deployment - Medium risk (2) deployment - High risk deployment |  |  |
| ## Which risk band does the activity fall within? | High |  |
| Is the risk banding correct? | Y | High risk due to SAC and SSSI within 100 meters RFI response 08/03/2024- South Leys fields removed from application, therefore application is no longer high risk |
| ## If Medium risk (2) or High risk deployment has a SSRA been provided? | Y | SSRA 2023 |
| - Additional information on sensitive receptors |  |  |
| ## What type of risk assessment has been submitted? | site‐specific risk assessment |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | N |  |
| If no have details of the assessment evidence to support this conclusion been provided? | N | Documents provided “Field photos” with application and later “Combined photos”. These are the same document and only include photos from the South Leys fields, and none from Robeston Hall RFI response 08/03/2024- photo evidence provided show fields are in agricultural use, however RH 15 does not appear to be managed as grassland as per the ABS- further RFI is required Response 14/03/24- resolved |
| - About the waste |  |  |
| Have all parts of the waste-types table been completed? - List of waste code (6 digit) - Description of waste (this must be an accurate description specific to this waste that refers to the List of Waste written description) - Physical form of waste/Is it stackable or non-stackable - Is the waste high in readily available nitrogen? - Producer of waste & permit number (if applicable) - Total amount being spread/used (tonnes) | N | - Given and correct - Given and correct - Physical form is missing- given in then ABS - Given and correct - Waste source is just given as “DCWW” no actual sources of the waste have been given- not acceptable - Given and correct (e) some information is given on “LPD1 Supplement wastes” document, but still not all the information for example, Water Treatment works? RFI response 08/03/2024-clarified waste sources as Water treatment works |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | N | Sources of the waste are not given on the application form RFI response 08/03/2024-clarified waste sources as Water treatment works |
| - About the storage |  |  |
| Has all of waste storage table been completed? | Y |  |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y | 2 lagoons, 1 “hardstanding”. RFI response 08/03/2024- reduced to 1 hardstanding storage area |
| 18. Supporting documents |  |  |
| Has checklist been completed? | Y |  |
| 20. Confidentiality and national security |  |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| 21. Declaration |  |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | Y | BPR Approval to sign on behalf (2023) |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has the operator confirmed they will comply with the NRW pre-notification requirements. | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y |  |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no storage or spreading within 10 meters) (buffer zones needed) | Y | South Leys and Robeston Hall- Relevant buffers in place |
| groundwater source protection zones (permit condition: no storage or spreading in SPZ1) (high risk deployment if in SPZ2) | n/a | South Leys and Robeston Hall- none |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no storage or spreading within 50 meters) | N | South Leys- springs requiring buffers RFI response 08/03/2024- SL fields removed from application Robeston Hall- wells requiring buffers. Slurry Pit storage location will have to be removed from application RFI response 08/03/2024- slurry pit storage removed RFI response 08/03/2024- wells shown on map do not match with locations- further RFI is needed Response 14/03/24- resolved |
| location of all storage sites for the waste to be spread | Y |  |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | N | South Leys- springs to be shown on maps RFI response 08/03/2024- SL fields removed from application Robeston Hall- wells to be shown on map RFI response 08/03/2024- wells shown on map do not match with locations- further RFI is needed Response 14/03/24- resolved |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | N | South Leys- springs to be shown on maps RFI response 08/03/2024- SL fields removed from application Robeston Hall- wells to be shown on map RFI response 08/03/2024- wells shown on map do not match with locations- further RFI is needed Response 14/03/24- resolved |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | N | South Leys Only SAC given on Map- SSSI is in same location. Only includes SAC location to the east of the fields, not the SAC/SSSI to the south and west - Afonydd Cleddau / Cleddau Rivers SAC - Afon Cleddau Gorllewinol / Western Cleddau River SSSI RFI response 08/03/2024- SL fields removed from application Robeston Hall- none |
| location of any public rights of way (shown on map) | Y N n/a | South Leys- none Robeston Hall- buffered |
| any buildings or houses within 250m (shown on map) | Y |  |
| Within Groundwater Safeguard Zone | n/a |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Habitats - Section 7 species - Scheduled Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Hedges - Ditches | Y N n/a | South Leys - None - Within watercourse- not fields - None - None - None - Within 500 meters, but not bordering the fields - None - None - Y - Y RFI response 08/03/2024- SL fields removed from application Robeston Hall - None - Within watercourse- not fields - None - None - None - South of field 14 - None - Edge of Arfordir Penfro / Pembrokeshire Coast NP - Y - Y RFI response 08/03/2024-resolved |
| Flood risk | n/a | Storage/spreading areas not at risk of flooding |
| What is the slope of the fields? | 0-7 degrees (transparent)- all Robeston Hall and South Lays fields 7 -12 degrees (yellow)- none 12 degrees plus (red)- none |  |

| Waste Storage | Y N n/a | Comments |
| --- | --- | --- |
| Is waste storage occurring at the place where waste is being deployed? | Y |  |
| Is only waste as specified in this deployment being stored in the proposed storage locations? i.e. slurry in lagoons Note: There may be multiple deployments using one storage location | Y | “Storage pits for liquid sludges will be emptied and no residual waste prior to the start of a new deployment. During the active period of the deployment, the designated sludge storage areas will contain wastes specified in Table 2 only. “ RFI response 08/03/2024-slurry pit/lagoon removed from application |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in the same place one constructed within the last two years? or in different location? | n/a | Waste not being stored in field heaps- lagoons and on farm hardstanding |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in a field: - liable to flooding or becoming waterlogged, or - within 50 metres of a borehole, spring or well or - within 10 metres of surface water or a land drain - within 30 meters of a watercourse on steeply sloping land | n/a | See above |
| If the waste is non-stackable is the waste stored within either a vessel, a covered building, or on an impermeable surface | Y | RH lagoon will need to be removed as it is within 50 meters of a well RFI response 08/03/2024-slurry pit removed from application |
| If the waste is high RAN and it is being stored within 200 metres of a European Site (SAC/SPA), Ramsar Site or a Site of Special Scientific Interest (SSSI) is the waste being covered to prevent loss of ammonia? (condition 3.2.4 of the permit) | n/a | None of the wastes are high RAN |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | Y |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | Y |  |
| - If yes; have they been tested for? | Y |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a | Not in a GSZ |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | N | All indices are above target at 3 and 4, however offtake is more than additions for all wastes and fields |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y | Overall decrease in indices. |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y |  |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | n/a |  |
| - Soil physical properties (e.g. organic matter and soil structure) | Y |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| Timing of application- if any of the wastes are high RAN, are they being spread OUTSIDE of the closed periods | n/a | Not high RAN |
| Groundwater Safeguard Zone for nitrate- if any of the wastes are high RAN, are the application rates less than 50 m3/ha, with a minimum 3 week gap between applications | n/a | Not in GSZ |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | n/a | All below offtake |
| - Included the nutrient level in their environmental risk assessment | n/a |  |
| - Stated the source of information that they used to set the nutrient level. | n/a |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y |  |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need, as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If the cropping is any of the following, is the maximum nitrogen limit in line with that given in The Water Resources (Control of Agricultural Pollution) (Wales) Regulations 2021: - 0 kg/ha- Field beans, Peas - 120 kg/ha- Sugar beet - 150 kg/ha- Asparagus, Carrots, Forage maize, Radish, Spring barley, Swedes - 180 kg/ha- Spring-sown wheat, Winter barley - 220 kg/ha- Autumn or early winter sown wheat | Y | All N additions are below the 150kg/ha limit for forage maize RFI response 08/03/2024-maize (SL) fields removed from application |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | Y |  |
| - Is it only for operational purposes? | Y |  |
| - Have individual application rates for each waste been given? | Y |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | Y |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients given by the applicant match NRW calculations? | N | See comments RFI response 08/03/2024-partially resolved- see comments Response 14/03/24- resolved |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | n/a |  |
| - How much material was spread? | n/a |  |
| - At what application rate material was spread? | n/a |  |
| - The nutrients supplied by the material? | n/a |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | Y |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | Y |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | Y |  |
| - Pathways | Y |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | Y |  |
| - Measures that will minimise or mitigate the impact will be put in place | Y |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | Y | RFI response 08/03/2024-SL fields removed from application, removing the need for an Appendix 4 to be completed |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | N |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | Y RFI response 08/03/2024-SL fields removed from application, removing the need for an HRA to be completed |

Comments

Consultation responses: - no comments made

MyMap/location and slope assessments: MyMap/location assessment - springs and wells missing from maps- RFI response 08/03/2024-not resolved Response 14/03/24- resolved - SSSI and SAC only identified to the east of SL fields RFI response 08/03/2024-SL fields removed - RH slurry pit needs to be removed as its within 50 meters of a well RFI response 08/03/2024- resolved - RH- ancient woodland south of field 14 RFI response 08/03/2024- resolved - RH fields within Arfordir Penfro / Pembrokeshire Coast National Park RFI response 08/03/2024- resolved Slope assessment All fields have between 0 and 7 degrees slope; no assessment required.

CPH details: Only one given on LPD1 form- 55/550/0016 From internal GIS: South Leys fields (A-E)- 55/455/0018 -J W C GEORGE AND SONS of BRYNHYFRYD, WOLFSCASTLE, HAVERFORDWEST, PEMBROKESHIRE, SA62 5DT Robeston Hall Field 13 – no data Field 14 and 15- 55/461/0125- HOOK FARM VEGETABLE GROWERS, HOOK FARM, AMBLESTON, HAVERFORDWEST, PEMBROKESHIRE, SA62 5QX RFI response 08/03/2024- resolved

Protection of habitats and ecosystems Documents provided “Field photos” with application and later “Combined photos”. These are the same document and only include photos from the South Leys fields, and none from Robeston Hall RFI response 08/03/2024- resolved

Agricultural benefit calculations: All fields are a minimum of index 3, but all wastes have offtakes. Yield for maize not given- only grass. Have presumed 40t/ha as per RB209 Interpretation document Preseli liquid- interpretation doesn’t match by a significant amount. Preseli cake- ok Bolton Hill cake- unable to check as analysis is missing Bolton hill liquid- ABS says 60, interpretation says 90. Correct at 90 Llechryd liquid- ABS says 26, interpretation says 30. Correct at 30 Llechryd cake- ABS says 22, interpretation says 25. Correct at 25 Bontgoch liquid- ABS says 30, interpretation says 35. Correct at 25 Strata Florida cake- ABS says 21, interpretation says 25. Correct at 25 - interpretation document includes Bontgoch cake which is not in the application RFI response 08/03/2024- updated interpretation document provided, however Bolton Hill cake information is wrong and not based on the waste analysis provided- further RFI is needed Response 14/03/24- resolved

Request for information: - Sent 22/02/24- deadline of 08/03/24. - Response received 08/03/24

Other information to note: Storage/ABS issues: - Benefit statement Table 4 states stackable waste to be stored in field heaps- however these have not been included in the application - Benefit statement Table 4 only includes one hardstanding storage location and one lagoon. Application form and spreading maps shows one hardstanding storage and two lagoons. - Benefit statement table 4 states “The storage locations are not within 10m of any ditch, watercourse, or footpath, not within a SPZ1, and are at least 50m from any well spring or borehole. “ This is not correct as the slurry pit at RH is within 50 meters of a well. RFI response 08/03/2024- all resolved

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date | 15/03/24 |
| ## Effective date | 15/03/24 |
| ## Permitting Officer Date | Louise Bailey 15/03/24 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 15/03/2024 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 760 |
| Total | 820 |