| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-024677 |
| Permit Reference Number: | EPR-BB3990ZX |
| Permit Holder: | Beacon Foods Limited |
| Site Address: | Aberbran Fawr, Aberbran, Brecon, Powys, LD3 9NG |
| Grid Reference: | Farm location: SN 98874 28985 Fields: 6875 9 Acre SN 99659 30740 9776 Top Field (5) SO 01955 27775 1881 Opposite Rape Field (6) SO 02163 27836 2495 8 Acres (8) SO 02246 27938 3871 By Rhydywernon (7) SO 02358 27708 4530 Long Field SN 99403 31365 2316 SN 9925 31163 7992 L Shaped SN 99752 28931 8176 Cows Field SN 99807 28785 5372 Quarry SN 99520 28729 1865 Reservoir SN 99187 28649 6255 Bank Field / Right of Pond SN 98613 28557 1211 Brick Field SN 99104 29089 Storage: SN9959930835 SO0255127990 SN9921129054 SN9963128832 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: GwastraffaGorfodi.Canolbarth@cyfoethnaturiolcymru.gov.uk Environment team: environmentteam.southpowys@cyfoethnaturiolcymru.gov.uk NLO: Liz.Park@cyfoethnaturiolcymru.gov.uk |
| Date due: | 20/02/2024 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating A- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | Welsh, SRP |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and deployment fee- Charging Scheme |
| Fee on MyNRW | Y | Date cleared on MyNRW: 14/12/2023. Amount: £1,837.00 Transaction number: 2124846 Applicant is using an original fee from a deployment they requested to be withdrawn. A new deployment was submitted where the old fee will be used. The withdrawn deployment was not allocated so the fee can be retained in this case. |
| Location map | Y |  |
| Benefit statement | Y |  |
| Waste Analysis | Y |  |
| Soil Analysis | Y |  |
| EIA Agri Regulations evidence | Y |  |
| Site Specific Risk Assessment | Y | High risk location only |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| - About the Permit |  |  |
| ## Has there been any pre-application discussions? | N |  |
| ## Has the permit number been provided? | Y |  |
| ## Has the permit holder name been provided? | Y |  |
| ## What permit is being deployed under? | SR2010 No4 |  |
| - Contact details |  |  |
| ## Has contact information been provided? | Y |  |
| ## Has the technically competent manager details been provided? | Y |  |
| ## Has the nominated competent person details been provided? | Y |  |
| - About the land you want to treat |  |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | See comments at the bottom of the decision document |
| ## Has all of parcels of land table been completed? | N | Field 3572 doesn’t appear in the field list but does in the site plan as an X. Field 5494 doesn’t appear in the field list but does in the site plan as an X. |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y |  |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | Y | One has been provided but does not cover the full information required. |
| ## Has written consent of all owners(s) or occupiers(s) been given? | N | One has been provided but does not cover the full information required. Therefore, written consent cannot be verified. |
| - Previous land treatment |  |  |
| Has all of the previous land treatment table been completed? | Y | Previous waste spread did not require a deployment. |
| - Which risk band does the activity fall within? - SR2010No4 List A wastes (Lower risk) - Location risk - Low risk deployment - Medium risk (1) deployment - Medium risk (2) deployment - High risk deployment |  |  |
| ## Which risk band does the activity fall within? | High |  |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) or High risk deployment has a SSRA been provided? | Y |  |
| - Additional information on sensitive receptors |  |  |
| ## What type of risk assessment has been submitted? | Site‐specific risk assessment |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | n/a |  |
| If no have details of the assessment evidence to support this conclusion been provided? | Y |  |
| - About the waste |  |  |
| Have all parts of the waste-types table been completed? - List of waste code (6 digit) - Description of waste (this must be an accurate description specific to this waste that refers to the List of Waste written description) - Physical form of waste/Is it stackable or non-stackable - Is the waste high in readily available nitrogen? - Producer of waste & permit number (if applicable) - Total amount being spread/used (tonnes) | N | The physical form of the waste is not given (stackable or non-stackable). The wastes readily available nitrogen content has not been given. The producer of the waste and permit reference has not been given. |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | N | No producer of waste given or permit reference. |
| - About the storage |  |  |
| Has all of waste storage table been completed? | Y | The location of the storage locations have been given but 2 field heaps locations are not on fields under this deployment. |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y |  |
| 18. Supporting documents |  |  |
| Has checklist been completed? | Y |  |
| 20. Confidentiality and national security |  |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| 21. Declaration |  |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | N | The applicant has answered no to signing off as a relevant person. But has uploaded an incorrect declaration. |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has the operator confirmed they will comply with the NRW pre-notification requirements. | Y |  |
| Has a relevant person or approved person signed the declaration? | N | The person who has signed the declaration is not a relevant person for beacon foods. Incorrect declaration given. The person signing off declaration is not a relevant person for beacon foods. |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y |  |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no storage or spreading within 10 meters) (buffer zones needed) | Y |  |
| groundwater source protection zones (permit condition: no storage or spreading in SPZ1) (high risk deployment if in SPZ2) | Y |  |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no storage or spreading within 50 meters) | n/a |  |
| location of all storage sites for the waste to be spread | Y |  |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | n/a |  |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | n/a |  |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | N | Newton farm site plan has not labelled the River Usk SAC and SSSI on the siter plan. |
| location of any public rights of way (shown on map) Geospatial Map Viewer> | Y N n/a |  |
| any buildings or houses within 250m (shown on map) | Y |  |
| Within Groundwater Safeguard Zone | n/a |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Habitats - Section 7 species - Scheduled Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Hedges - Ditches | Y |  |
| Flood risk | n/a |  |
| What is the slope of the fields? | 0-7 degrees (transparent)- Fields: 1881, 2495, 3871 7 -12 degrees (yellow)- Fields- 6225, 1865, 5372, 8176, 7992, 1211, 9776, 6875, 4530 and 2316 12 degrees plus (red)- Fields 6255 1865, 5372, 7992 and 1211 |  |

| Waste Storage | Y N n/a | Comments |
| --- | --- | --- |
| Is waste storage occurring at the place where waste is being deployed? | N | Two storage locations (temporary field heaps) are located outside the fields of this deployment. |
| Is only waste as specified in this deployment being stored in the proposed storage locations? i.e. slurry in lagoons Note: There may be multiple deployments using one storage location | N |  |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in the same place one constructed within the last two years? or in different location? | n/a | n/a due to fields not being previously deployed on. |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in a field: - liable to flooding or becoming waterlogged, or - within 50 metres of a borehole, spring or well or - within 10 metres of surface water or a land drain - within 30 meters of a watercourse on steeply sloping land | n/a | The field heaps are located on moderately sloping land but not within 30 metres of a watercourse. |
| If the waste is non-stackable is the waste stored within either a vessel, a covered building, or on an impermeable surface | N |  |
| If the waste is high RAN and it is being stored within 200 metres of a European Site (SAC/SPA), Ramsar Site or a Site of Special Scientific Interest (SSSI) is the waste being covered to prevent loss of ammonia? (condition 3.2.4 of the permit) | n/a | Waste is not stored within 200m of a designated site. |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y | The p.H of the waste is 3.99 which is considered acidic. |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | n/a |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | Y | EWC 02 03 04 Odour management plan and pests due to the waste originating from the food industry. |
| - If yes; have they been tested for? | N |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y N n/a | The Soil analysis naming convention does not match field names in deployment application. Therefore, an assessment of nutrient additions was not possible. |
| Does the analysis include the following? |  |  |
| - pH | Y N | See above comment. |
| - Potassium (K) | Y N |  |
| - Phosphorus (P) | Y N |  |
| - Magnesium (Mg) | Y N |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | Y N n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y N |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y N n/a |  |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | N | p.H of waste is 3.99 which is acidic and has not been addressed in the ABS or SSRA. |
| - Is the material suitable for its intended use? | N | See above comment. |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | N |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | N | Nitrogen limits for other fertilizers added up are over 250 kg. |
| - Timing of application | Y |  |
| - Amount of waste to be applied and application rate | N | Nitrogen limits for other fertilizers added up are over 250 kg/ha. |
| - Soil chemical properties (e.g. liming and pH) | N | EWC 02 03 04 has a p.H of 3.99. This is an acidic waste stream and the ABS and wider deployment documents do not provide any mitigation measures to minimize the risk of stunted growth from the grass and crops. Therefore, the deployment doesn’t not meet agricultural benefit. |
| - Soil physical properties (e.g. organic matter and soil structure) | n/a |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| Timing of application- if any of the wastes are high RAN, are they being spread OUTSIDE of the closed periods | n/a |  |
| Groundwater Safeguard Zone for nitrate- if any of the wastes are high RAN, are the application rates less than 50 m3/ha, with a minimum 3 week gap between applications | n/a |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | Y N n/a | This assessment was not possible due to the soil analysis provided does not match the field named in the deployment application. |
| - Included the nutrient level in their environmental risk assessment | Y N n/a |  |
| - Stated the source of information that they used to set the nutrient level. | Y N n/a |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y | Very limited information has been given for potential negative impacts and does not mention the low p.H of the waste. The odour risk has been acknowledged but due to the solid nature of the waste storing in temporary field heaps an odour management plan needs to be submitted as part of the application. |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need, as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | N | The p.H of waste is 3.99 which is acidic and has not been addressed in the ABS or SSRA. There is a risk of stunted crop and grass growth. Therefore, agricultural benefit cannot be distinguished. |
| - Waste analysis | N | See above comment. |
| - Crop nutrient requirements | N | The waste additions from the deployment meet the requirements of the Nitrogen limit set out in SR2010No4 but when including artificial fertilizers that will also be spread on the fields the N limit of 250 kg is exceeded in some fields. |
| If the cropping is any of the following, is the maximum nitrogen limit in line with that given in The Water Resources (Control of Agricultural Pollution) (Wales) Regulations 2021: - 0 kg/ha- Field beans, Peas - 120 kg/ha- Sugar beet - 150 kg/ha- Asparagus, Carrots, Forage maize. Radish, Spring barley, Swedes - 180 kg/ha- Spring-sown wheat, Winter barley - 220 kg/ha- Autumn or early winter sown wheat | N | See above comment. |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | n/a |  |
| - Is it only for operational purposes? | n/a |  |
| - Have individual application rates for each waste been given? | n/a |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | n/a |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients given by the applicant match NRW calculations? | Y |  |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | n/a |  |
| - How much material was spread? | n/a |  |
| - At what application rate material was spread? | n/a |  |
| - The nutrients supplied by the material? | n/a |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | Y | The risk assessment is not site specific. i.e., odour risk, pest risk and acidity issues., |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | Y |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | N | p.H of the waste has not been considered. Odour has not been considered. Pests have not been considered. |
| - Pathways | Y |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | Y |  |
| - Measures that will minimise or mitigate the impact will be put in place | N | Measures in place for the risk of Odour , acidic nature of waste and pests have not been considered. |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | Y |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | n/a | n/a due to the applicant being returned. |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | Y |
| No FORM 1 completed: Due to this deployment being returned to the applicant. |  |

Comments

Consultation responses:

MyMap/location and slope assessments: MyMap/location assessment For details of the Mymap assessment see the DMS. Slope assessment Field number: 6255 1865, 5372, 7992 and 1211 Slope of the land: areas of land sloping at 12o+ Soil type: Medium Wastes types applied: Solid Cropping type: Grass, winter wheat and Spring barley Method of application: Rear discharge spreader Weather conditions before during and after spreading: In compliance with the restriction outlined in CoGAP. Conclusions (is run off risk high?): Low Field number: 6225, 1865, 5372, 8176, 7992, 1211, 9776, 6875, 4530 and 2316 Slope of the land: areas of land sloping at 7-12o+ Soil type: Medium Wastes types applied: Solid Cropping type: Grass, winter wheat and Spring barley Method of application: Rear discharge spreader Weather conditions before during and after spreading: In compliance with the restriction outlined in CoGAP. Conclusions (is run off risk high?): Low The waste to be spread (EWC 02 03 04) is a solid meaning risk of surface runoff is minimal due to its solid nature. The submitted SSRA has taken into account slopes of the fields where the operator will take the following measures to minimize runoff: - Abide by 10m no spread zones in place around field boundaries. - Weather conditions are assessed prior to spreading to ensure, risk of run-off is low. The applicant intents to spread the waste via rear discharge spreader which is an appropriate method for the waste type but as a consequence is less accurate at applying the waste to the soil. The application rate of the food waste is low at 15 t/ha to 16.5 t/ha. As the waste to be spread is at a lower application rate there is less chance of surface runoff when applying the wastes in suitable weather conditions. Therefore, for this waste the surface runoff risk has been determined as low as long as measures in the SSRA are adhered too.

CPH details: CPH given in LPD1: 52/034/0013 CPH number found on the Mymap record: 52/034/0013 A/G Matthews (all fields). The CPH number provided matches the Mymap record.

Protection of habitats and ecosystems For details of the Mymap assessment see the DMS.

Agricultural benefit calculations: The p.H of the waste is 3.99 which is acidic. A full Agricultural benefit calculation was not possible due to the fields names in the soil analysis being different from the fields names listed under the deployment.

Request for information: n/a – Due to this deployment application being returned to the applicant.

Other information to note: This deployment application is being retuned due to the following reasons: 1. The p.H of the waste is very acidic (3.99) and has not been considered in the deployment application therefore Agricultural benefit cannot be established due to the possible risk of stunted grass and crop growth. 2. The p.H of the waste will also lower the soil p.H of the fields further increasing the risk of detriment to the grass and crops. 3. The applicant proposed to use temporary field heaps and has not addressed how the underlying soil with be protected from the acidic waste stream. 4. The waste originates from the food industry and is essentially food waste. The applicant has not provided an odour management plan As the waste was proposed to be stored in temporary field heaps , pests should have also been accounted for which have not been. 5. The site specific risk assessment provided was not site specific which should have included odour risk , pests, and the acidic nature of the waste and the mitigation methods to reduce these risks. 6. The soil analysis provided cannot be crossed referenced with the fields in the deployment due to a different naming convention between the soil analysis and the fields in the submitted deployment therefore a full assessment of nutrient additions was not possible.

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | No |
| ## Issue date | n/a |
| ## Effective date | n/a |
| ## Permitting Officer Date | Owen Clee 04/04/2024 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 04/04/2024 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 1,180 |
| Total | 1,240 |