# Effluent calculation – Metal Box, Neath

Our site at the Metal Box, Neath will hold one module. One module consists of two independent RAS systems; an on grow tank for adult shrimp and a nursery tank for juvenile shrimp which has channel for post larvae shrimp. This is attached to the side of the nursery system. Below will be the calculation for one module running at Metal Box.

## Calculation for one module (On grow tank, nursery tank with nursery channel)

| On grow tank | m3 | Litres (L) |
| --- | --- | --- |
| Tank | 1,114 | 1,114,000 |
| Biofilter 1 | 30 | 30,000 |
| Biofilter 2 | 30 | 30,000 |
| Biofilter 3 | 30 | 30,000 |
| Protein skimmer 1 | 3 | 11,300 |
| Protein skimmer 2 | 11.5 | 11,500 |
| Radial Flow Separator 1 | 27 | 27,000 |
| Drum filter 1 | 15 | 15,000 |
| Biofilter 4 | 15 | 15,000 |
| Biofilter 5 | 15 | 15,000 |
| Final sump | 60 | 60,000 |

Total system volume = 1,350.5 m3

Rounded up to include pipework = 1,500 m3 (1,500,000 L)

| Nursery tank | m3 | Litres (L) |
| --- | --- | --- |
| Tank | 495 | 495,000 |
| Biofilter 1 | 30 | 30,000 |
| Biofilter 2 | 20 | 20,000 |
| Biofilter 3 | 20 | 20,000 |
| Protein Skimmer 1 | 8 | 8,000 |
| Protein Skimmer 2 | 2 | 2,000 |
| Radial Flow Separator 1 | 4.5 | 4,500 |
| Radial Flow Separator 2 | 16 | 16,000 |
| Drum filter 1 | 20 | 20,000 |
| Biofilter 4 | 12 | 12,000 |
| Biofilter 5 | 12 | 12,000 |
| Final sump | 50 | 50,000 |
| Nursery channel | 75 | 75,000 |

Total system volume = 764.5 m3

Rounded up to include pipework = 780 m3 (780,000L)

Total volume of one module = 2,280m3 (2,280,000L)

Maximum effluent discharged per day will be 1% for each system in the module

## 1% of each individual system:

On grow system = 1,500m3 x 0.01 = 15m3 (15,000L)

Nursery system = 780m3 x 0.01 = 7m3 (7,000L)

### 1% of module total:

2,275m3 x 0.01 = 22.75 m3 (22,750L)

Systems will be running 24hrs/ day

1,440 minutes

86,400 seconds

## Maximum rate of discharge (L/s)

## Per individual system:

On grow system = 15,000 / 86,400 = 0.1736 L/s

Nursery system = 7,000 / 86,400 = 0.0810 L/s

## For one module:

22,750 / 86,400 = 0.2633 L/s

## Conclusion:

Total volume of module is 2,280m3 (2,280,000L)

1% of total module exchanged per day is 22.75 m3 (22,750L)

Maximum rate of discharge (L/s) is 0.2633 L/s