| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-024482 |
| Permit Reference Number: | EPR/GP3792SK |
| Permit Holder: | BYPRODUCT RECOVERY LIMITED |
| Site Address: | Abergelli Farm, Felindre, Swansea, SA7 5NN |
| Grid Reference: | Farm location: SN 65120 01620 Fields: 1 SN 64750 13300 2 SN 64910 13200 3 SN 64650 13190 4 SN 64470 13060 5 SN 64420 13160 6 SN 64530 13240 6201 SN 65620 01010 3907 SN 65390 02070 1163 SN 66016 00728 0173 SN 66120 00645 0455 SN 66030 00562 16 SN 64640 01470 17 SN 64550 01640 18 SN 64470 01520 striked out fields have been removed from the deployment (see RFI section). Storage: N/A |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: wasteregulation.swwales@cyfoethnaturiolcymru.gov.uk Environment team: Carms.EnvironmentTeam@cyfoethnaturiolcymru.gov.uk swanseaenvironmentteam@cyfoethnaturiolcymru.gov.uk NLO: Don.Rother@environment-agency.gov.uk |
| Date due: | 04/03/2024 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating B- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | English – EA public register N/A |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and deployment fee- Charging Scheme |
| Fee on MyNRW | Y | Amount: £1561.00 Date cleared on MyNRW: 12/01/2024 Transaction number: 2125650 |
| Location map | Y |  |
| Benefit statement | Y |  |
| Waste Analysis | Y |  |
| Soil Analysis | Y |  |
| EIA Agri Regulations evidence | Y |  |
| Site Specific Risk Assessment | N/A |  |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| - About the Permit |  |  |
| ## Has there been any pre-application discussions? | N |  |
| ## Has the permit number been provided? | Y |  |
| ## Has the permit holder name been provided? | Y |  |
| ## What permit is being deployed under? | SR2010 No4 |  |
| - Contact details |  |  |
| ## Has contact information been provided? | Y |  |
| ## Has the technically competent manager details been provided? | Y |  |
| ## Has the nominated competent person details been provided? | Y |  |
| - About the land you want to treat |  |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | Correct CPH numbers have been provided in the consent document for all fields |
| ## Has all of parcels of land table been completed? | Y |  |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y |  |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | Y |  |
| ## Have details of the land owner or occupier been provided? | Y |  |
| ## Has written consent of all owners(s) or occupiers(s) been given? | Y |  |
| - Previous land treatment |  |  |
| Has all of the previous land treatment table been completed? | Y | PAN-018834 Expired: 04/10/2023 |
| - Which risk band does the activity fall within? - SR2010No4 List A wastes (Lower risk) - Location risk - Low risk deployment - Medium risk (1) deployment - Medium risk (2) deployment - High risk deployment |  |  |
| ## Which risk band does the activity fall within? | Low Medium (1) Medium (2) High Bespoke |  |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) or High risk deployment has a SSRA been provided? | N/A |  |
| - Additional information on sensitive receptors |  |  |
| ## What type of risk assessment has been submitted? | Benefit statement |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | N/A |  |
| If no have details of the assessment evidence to support this conclusion been provided? | N/A |  |
| - About the waste |  |  |
| Have all parts of the waste-types table been completed? - List of waste code (6 digit) - Description of waste (this must be an accurate description specific to this waste that refers to the List of Waste written description) - Physical form of waste/Is it stackable or non-stackable - Is the waste high in readily available nitrogen? - Producer of waste & permit number (if applicable) - Total amount being spread/used (tonnes) | Y |  |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y |  |
| - About the storage |  |  |
| Has all of waste storage table been completed? | N/A |  |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | N/A |  |
| 18. Supporting documents |  |  |
| Has checklist been completed? | Y |  |
| 20. Confidentiality and national security |  |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| 21. Declaration |  |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | Y |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has the operator confirmed they will comply with the NRW pre-notification requirements. | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | N/A |  |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no storage or spreading within 10 meters) (buffer zones needed) | Y |  |
| groundwater source protection zones (permit condition: no storage or spreading in SPZ1) (high risk deployment if in SPZ2) | N/A |  |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no storage or spreading within 50 meters) | N/A |  |
| location of all storage sites for the waste to be spread | N/A |  |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | Y |  |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | Y |  |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | N/A |  |
| location of any public rights of way (shown on map) | Y | Rectified via RFI 1 |
| any buildings or houses within 250m (shown on map) | Y |  |
| Within Groundwater Safeguard Zone | N/A |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Habitats - Section 7 species - Scheduled Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Hedges - Ditches | Y | All appropriately mapped but field falling within a SINC will be removed |
| Flood risk | N | High flood risk in the south of field 6201 |
| What is the slope of the fields? – | 7 -12 degrees (black) – 3, 1163 12 degrees plus (Blue) – 2,3 |  |

| Waste Storage | Y N n/a | Comments |
| --- | --- | --- |
| Is waste storage occurring at the place where waste is being deployed? | N |  |
| Is only waste as specified in this deployment being stored in the proposed storage locations? i.e. slurry in lagoons Note: There may be multiple deployments using one storage location | N/A |  |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in the same place one constructed within the last two years? or in different location? | N/A |  |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in a field: - liable to flooding or becoming waterlogged, or - within 50 metres of a borehole, spring or well or - within 10 metres of surface water or a land drain - within 30 meters of a watercourse on steeply sloping land | N/A |  |
| If the waste is non-stackable is the waste stored within either a vessel, a covered building, or on an impermeable surface | N/A |  |
| If the waste is high RAN and it is being stored within 200 metres of a European Site (SAC/SPA), Ramsar Site or a Site of Special Scientific Interest (SSSI) is the waste being covered to prevent loss of ammonia? (condition 3.2.4 of the permit) | N/A |  |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | N |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | Y | iron, aluminium, sulphur and manganese |
| - If yes; have they been tested for? | Y |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | N/A |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | N/A |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | N/A |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | Y | Not high in PTE’s but have been tested for |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y |  |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y |  |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | Y |  |
| - Soil physical properties (e.g. organic matter and soil structure) | Y |  |
| - Soil water content | N/A |  |
| - Levelling the land | N/A |  |
| Timing of application- if any of the wastes are high RAN, are they being spread OUTSIDE of the closed periods | N/A |  |
| Groundwater Safeguard Zone for nitrate- if any of the wastes are high RAN, are the application rates less than 50 m3/ha, with a minimum 3 week gap between applications | N/A |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | N/A |  |
| - Included the nutrient level in their environmental risk assessment | N/A |  |
| - Stated the source of information that they used to set the nutrient level. | N/A |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y |  |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need, as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If the cropping is any of the following, is the maximum nitrogen limit in line with that given in The Water Resources (Control of Agricultural Pollution) (Wales) Regulations 2021: - 0 kg/ha- Field beans, Peas - 120 kg/ha- Sugar beet - 150 kg/ha- Asparagus, Carrots, Forage maize. Radish, Spring barley, Swedes - 180 kg/ha- Spring-sown wheat, Winter barley - 220 kg/ha- Autumn or early winter sown wheat | N/A |  |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | N/A |  |
| - Is it only for operational purposes? | N/A |  |
| - Have individual application rates for each waste been given? | N/A |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | N/A |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | N | Total given for fields with an SNS index <2 |
| Does the total nutrients given by the applicant match NRW calculations? | Y | Missing analysis provided via RFI 1 |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y | PAN - 018834 |
| - How much material was spread? | Y |  |
| - At what application rate material was spread? | Y |  |
| - The nutrients supplied by the material? | Y |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | N/A |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | N/A |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | N/A |  |
| - Pathways | N/A |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | N/A |  |
| - Measures that will minimise or mitigate the impact will be put in place | N/A |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | N |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | N/A |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | N |

Comments

Consultation responses: Field 3907 - NE boundary is alongside the Rhyd y pandy valley grasslands SINC – we would advise a 10m buffer zone along this boundary. – A 10m no spread Buffer is being applied via RFI 1 10m no spread buffer zone has been applied to the field borders on the SINC. Fields numbers 16, 17 & 18 are within the Waun Garn Wen SINC – the fields are listed as B.4 Improved Grassland on the JNCC habitat survey, but we would recommend seeking further information from the Local Authority about the potential impact from the land spreading deployment application on the SINC features. – These field are being removed from the deployment via RFI 1 Fields 16, 17 and 18 have been removed from the deployment via RFI 1.

MyMap/location and slope assessments: MyMap/location assessment 7 -12 degrees (black) – 3, 1163 12 degrees plus (Blue) – 2,3 For details of the Mymap assessment refer to the DMS. Slope assessment Field number: 3 Slope of the land: 7-12 Degrees and 12degrees plus Soil type: Medium Wastes types applied (solid, liquid, sludge etc.): Sludge Cropping type: Grass Method of application: Dribble bar Weather conditions before during and after spreading: Following Cogap Other risk factors not mentioned above: Conclusions (is run off risk high?): No, the steepest slope is only very minimally within the spreading zone. A 10m buffer has been implemented on the border so runoff risk to the watercourse has been mitigated for. Field number: 1163 Slope of the land: 7-12 Degrees Soil type: Medium Wastes types applied (solid, liquid, sludge etc.): Sludge Cropping type: Grass Method of application: Dribble bar Weather conditions before during and after spreading: Following Cogap Other risk factors not mentioned above: Conclusions (is run off risk high?): No, the slope of the field was assessed and ran in a direction that it would be contained within the field with no risk to nearby receptors. Field number: 2 Slope of the land: 12 degrees plus Soil type: Medium Wastes types applied (solid, liquid, sludge etc.): Sludge Cropping type: Grass Method of application: Dribble bar Weather conditions before during and after spreading: Following Cogap Other risk factors not mentioned above: Ancient woodland b0orderin the field Conclusions (is run off risk high?): No, the steepest slope is only very minimally within the spreading zone. A 10m buffer has been implemented on the border so runoff risk to the sensitive receptor has been mitigated for.

CPH details: 1, 2, 3, 4, 5, 6, 0173, 0455, 1163 – 55/002/0122 3907, 6201 – 59/156/0050

Protection of habitats and ecosystems

Agricultural benefit calculations: The proposed waste to be spread is of a high aluminium content. As such, field 6 which has a PH of 5.8 is being removed from the deployment.

Request for information 1 - Update the location maps to clearly identify the Rhyd-Y-Pandy Valley as a sensitive environmental receptor. The location maps have been resubmitted correctly identifying the SINC bordering the field. - Update the ABS so that it considers the identified SINC under the potential negative impacts section of the ABS and the mitigation measures in place to prevent detriment to the SINC i.e., a no spread buffer zone. A new ABS document has been submitted which provides mitigation for the neighbouring SINC in the form of a 10m no spread buffer zone, which has also been reflected in the location maps. - Update the site plans to clearly identify the public footpath bordering field 3907. A new location map has been submitted which clearly identifies the footpath on the maps and in the key. - Remove fields 16, 17 and 18 from the deployment application and update the location maps. Field 16,17 and 18 have been appropriately removed from both the ABS and maps. - Update the ABS to provide appropriate mitigation for the flood risk associated with field 6201. A new ABS has been submitted with appropriate mitigation for the present flood risk. - Confirm if the wastes will be spread on delivery or not (A) IF YES, Remove the storage sentence for the ABS. (B) If NOT, Provide the relevant information for us to assess the storage requirements The applicant has confirmed waste will be spread on delivery and the storage sentence has been removed from the ABS. - Provide the available percentage value used in the nutrient requirement table for fields with a soil index of >2. The available percentage used has been provided and they match our own calculations. - pdate the ABS under table 6 so that the key matches the total and available concentration of the waste applied. A new ASB has been submitted with an updated key which clearly identifies where totals have been used. - Provide the percentage dry matter( %DM) of the grass crop. The new ABS states 25%DM grass crop is to be the planted crop. 10. Update the ABS to state closed periods do not apply for this waste stream and therefore no closed period dates are needed. A new ABS has been submitted which has correctly stated closed periods do not apply. 11. Provide the soil analysis reports for field 6201, 3907, 1163, 0173 and 0455 or remove these fields from the deployment application. The soil analysis for the missing fields has been provided. On using this information to conduct our assessment it is found the current application rate on these fields over applies arsenic over the allowed limit. 12. Remove field 6 form the deployment and update the location maps accordingly. Field 6 has been removed by showing a no spread buffer zone over this field. For clarity it will need to be appropriately removed. Request for information 2 1. Remove field 6 from the deployment without the use of a no spread buffer zone. A new location map was submitted with field 6 appropriately removed from the deployment. 2. Lower the application rate on fields 6201, 3907, 1163 and 0173 so the arsenic addition does not exceed the limit of 0.7kg/ha. The application rate has been lowered to 200 t/ha, giving an arsenic addition of 0.69kg/ha, therefore under the limit.

Other information to note:

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date | 17/04/23 |
| ## Effective date | 17/04/23 |
| ## Permitting Officer Date | Megan Tucker 11/04/2024 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 17/04/2024 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 2834 |
| Total | 2894 |