Cardiff Coastal Defences - Saltmarsh Management Plan

Ground Investigation Works

Marine Licence (Activity 3)

April 2024

www.jbaconsulting.com

Prepared for:

Cardiff Council

Document Status

Issue date	April 2024

Issued to	Simon Dooley

Revision	02

Prepared by 	Hannah Webster BSc MSc

Ecologist

Reviewed by 	Jonathan Harrison BSc MSc MCIEEM

Senior Ecologist

Authorised by 	Anne-Marie Moon BSc MSc CEng MICE

Project Director

___________________________________________________________________

Carbon Footprint

The format of this report is optimised for reading digitally in pdf format. Paper consumption produces substantial carbon emissions and other environmental impacts through the extraction, production and transportation of paper. Printing also generates emissions and impacts from the manufacture of printers and inks and from the energy used to power a printer. Please consider the environment before printing.

___________________________________________________________________

Contract

JBA Project Manager	Hannah Webster

Address	Kings Chambers, 8 High Street, Newport NP20 1FQ

JBA Project Code	2024s0084

This report describes work commissioned by Cardiff Council. Jonathan Harrison and Hannah Webster of JBA Consulting carried out this work.

Purpose and Disclaimer

Jeremy Benn Associates Limited (“JBA”) has prepared this Report for the sole use of Cardiff Council and its appointed agents in accordance with the Agreement under which our services were performed.

JBA has no liability for any use that is made of this Report except to Cardiff Council for the purposes for which it was originally commissioned and prepared.

No other warranty, expressed or implied, is made as to the professional advice included in this Report or any other services provided by JBA. This Report cannot be relied upon by any other party without the prior and express written agreement of JBA.

Where assessments of works or costs identified in this Report are made, such assessments are based upon the information available at the time and where appropriate are subject to further investigations or information which may become available.

JBA disclaims any undertaking or obligation to advise any person of any change in any matter affecting the Report, which may come or be brought to JBA’s attention after the date of the Report.

Unless otherwise stated in this Report, the assessments made assume that the sites and facilities will continue to be used for their current purpose without significant changes.

Where field investigations are carried out, these have been restricted to a level of detail required to meet the stated objectives of the services. The results of any measurements taken may vary spatially or with time and further confirmatory measurements should be made after any significant delay in issuing this Report.

___________________________________________________________________

Copyright

© Jeremy Benn Associates Limited 2024

___________________________________________________________________

Contents

1	Introduction	1

1.1	Purpose of the Saltmarsh Management Plan	1

1.2	Aim of the Saltmarsh Management Plan	1

2	Summary of the Scope of Works	2

2.1	Site Location and Description	2

2.2	Scope of investigation	3

3	Pre-Construction Actions	5

4	Saltmarsh Management Plan	7

A	Saltmarsh Maps	i

A.1	Extent of Saltmarsh	i

A.2	Saltmarsh extents April 2024	ii

List of Figures

Figure 2-1. Location of Rhymney River and its confluence into the Severn Estuary with key features.	2

List of Tables

Table 4-1. Saltmarsh Management Plan	6

Abréviations

CEMP		Construction Environmental Management Plan

EIA		Environmental Impact Assessment

SAC		Special Area of Conservation

SMP		Saltmarsh Management Plan

SSSI		Site of Special Scientific Interest

NRW		Natural Resources Wales

# Introduction

## Purpose of the Saltmarsh Management Plan

This Saltmarsh Management Plan (SMP) sets out the management approach, as agreed during consultation with Natural Resources Wales (NRW) for protection of the saltmarsh at the mouth of the Rhymney River and Severn Estuary to the east of Cardiff City Centre during the works to upgrade the flood defences at this location.

This version of the plan has been developed to protect the areas of saltmarsh during final ground investigation works prior to the main works taking place.

This management plan has been identified as essential mitigation within the Environmental Impact Assessment Report (EIA Report) and details other essential mitigation identified to avoid or minimise effects on the saltmarsh. Much of the proposed works and the surrounding saltmarsh habitat fall within the Severn Estuary Special Area of Conservation (SAC) and border the Gwent Levels Site of Special Scientific Interest (SSSI).

## Aim of the Saltmarsh Management Plan

The aim of the Saltmarsh Management Plan in this phase of the works is to ensure that no works temporary or permanent take place within the saltmarsh and to protect the habitat from indirect impacts.

This will include pre- and post-construction monitoring and identify points at which intervention may be required. This plan is a live document which will be updated throughout the construction period. The plan will ensure that essential mitigation strategies required for safeguarding protected species and habitats are implemented as part of the contract.

This Management Plan will be included as an annex to the Construction Environmental Management Plan (CEMP) which will be developed by the Contractor prior to construction.

# Summary of the Scope of Works

## Site Location and Description

The proposed development comprises  new fluvial and coastal flood defences along both banks of the River Rhymney on the outskirts of Cardiff from where the A4232 crosses the river down to the confluence between the river and the Severn Estuary. The defences will then extend along the Severn Estuary coastline, east and west from the River Rhymney. This area includes the DCWW Cardiff East WWTW and northwards along the eastern extent of the Lamby Way landfill site (see Figure 2-1).

Figure 2-1. Location of Rhymney River and its confluence into the Severn Estuary with key features.

## Scope of investigation

This plan addresses the ground investigation works along the coastal frontage within area 12 only, shown in figure 2-2..

The scope of the ground investigation comprises;

5 No. boreholes to be formed using a multipurpose drilling rig with rotary capability (e.g. Comacchio Rig or similar) and associated in-situ Standard Penetration Tests to a depth of 15m.

8 No. machine excavated trial pits to a depth of 3.5m.

Geotechnical and geochemical laboratory testing.

Figure 2-2. Location of proposed ground investigations

# Pre-Construction Actions

The following conditions regarding the saltmarsh have been put in place during the planning process and the actions undertaken to date are detailed.

P1.1 - Confirm extents of valuable and sensitive saltmarsh habitat for future monitoring/ comparison. P1.2 - Conduct walkover survey to identify areas of erosion/accretion.

A site survey was undertaken on the 6th of November 2023 by Jonathan Harrison and Hannah Webster of JBA consulting and the current extent of the saltmarsh was determined. To more accurately map the western boundary of the saltmarsh Knights Brown took GPS points with the supervision of an ecologist in April 2024.  This has ensured that these GI works will not require works within any saltmarsh habitat. The updated map is shown in Appendix A

P1.3 - Ensure mitigation of temporary works impacts is agreed. This should involve the production of a temporary works methods statement detailing mitigation measures for saltmarsh. This should also include details of the type of track matting to be used, the length of tracks required, and plant to be used on top of areas of saltmarsh.

These ground investigations will not require any works within the saltmarsh and as such tracking through the saltmarsh will not be required or permitted.   The saltmarsh has been demarcated and the ECoW and the GI supervisor will be responsible for ensuring no works (temporary or permanent) take place within the marsh.  It should be noted that it is likely that temporary tracks through the saltmarsh to facilitate the main works can be avoided and this is being developed.

P1.4 - Baseline invertebrate survey should be conducted to inform post-works monitoring – including Shrill Carder Bee.

A baseline invertebrate survey to include the Shrill Carder Bee will be completed in throughout the survey season (April - October).  No construction works which could damage the saltmarsh will be permitted prior to this initial survey taking place. Until the survey is available the ECoW will be responsible for providing guidance on operations likely to damage the saltmarsh on case by case basis.  Further surveys on the undisturbed saltmarsh will be undertaken between April and September.

P1.5 - A baseline rare plant species survey should be completed in the summer months prior to construction.

A baseline rare plant species survey will be completed in prior to any works being undertaken in the saltmarsh. No construction works likely to damage the saltmarsh will be permitted prior to this survey taking place.

P1.6 - Review the need for further mitigation measures in the Environmental Statement and habitat losses from temporary works should be revisited to identify further relevant actions.

A review of the Environmental Statement has been completed and no further mitigation measures have been identified.

It is intended that this document will be live throughout the construction period.  It will be updated following the relevant baseline surveys and with further mitigation identified. Further enhancement measures will be included if opportunities are identified through the construction period.

# Saltmarsh Management Plan

Table 4-1. Saltmarsh Management Plan

| Ref. | Action | Responsibility | Ref to further information | Progress and further action | Sign off and date |
| --- | --- | --- | --- | --- | --- |
| Pre-Construction |  |  |  |  |  |
| P1.1 | Confirm extents of valuable and sensitive saltmarsh habitat for future monitoring/comparison. | Construction Contractor ECoW | Environmental Statement | Completed Updated 12/04/2024 | 6/11/23 |
| P1.2 | Conduct walkover survey to identify areas of erosion/accretion. | Construction Contractor ECoW | Environmental Statement NVC Survey |  | 6/11/23 |
| P1.3 | Ensure mitigation of temporary works impacts is agreed. This should involve the production of a temporary works methods statement detailing mitigation measures for saltmarsh. | Construction Contractor | Environmental Statement | No GI works will take place within the saltmarsh. The saltmarsh has been demarcated and no temporary or permanent works will be permitted within the saltmarsh. | 27/02/24 |
| P1.4 | Baseline invertebrate survey should be conducted to inform post-works monitoring – including Shrill Carder Bee. | Construction Contractor | Environmental Statement | Will be completed in between April and September 2024. |  |
| P1.5 | A baseline rare plant species survey should be completed in the summer months prior to construction. | Construction Contractor | Environmental Statement | Will be completed before any works likely to damage the saltmarsh are completed. |  |

| Ref. | Action | Responsibility | Ref to further information | Progress and further action | Sign off and date |
| --- | --- | --- | --- | --- | --- |
| During Construction |  |  |  |  |  |
| C1.1 | Presence of a suitably qualified Ecological Clerk of Works (ECoW) is required to ensure that all mitigation measures are implemented | Construction Contractor ECoW | Environmental Statement | ECoW appointed by Cardiff Council |  |
| C1.2 | Best practice construction methods (CIRIA 2015) will be used including the use of appropriate pollution controls (i.e. Guidance for Pollution Prevention (GPPs)), such as construction drainage, a strict re-fuelling protocol and removal of all loose materials from the intertidal area | Construction Contractor | Environmental Statement |  |  |
| C1.3 | No works (temporary or permanent are permitted within the saltmarsh. | Construction Contractor | Environmental Statement |  |  |

| Ref. | Action | Responsibility | Ref to further information | Progress and further action | Sign off and date |
| --- | --- | --- | --- | --- | --- |
| Post Construction (Permanent Works) |  |  |  |  |  |
| PC1.1 | Surveys of the saltmarsh plant species composition will be important to inform the restoration of the saltmarsh and to demonstrate enhancement in the areas of saltmarsh that are retained. Monitoring of rare plant species should be undertaken to inform management practices. This should take the form of a fixed-point quadrat survey to record abundance, as well as an updated NVC survey to record habitat type. | Contractor – survey subsequent two years after completion of works. | Environmental Statement |  |  |
| PC1.2 | Monitoring of saltmarsh post-construction to inform future management practices and enhancement opportunities. Methods and frequency of monitoring to be agreed with NRW, and would likely include surveys of accretion/erosion, grazing pressure etc. | Construction Contractor | Environmental Statement |  |  |
| PC1.3 | Post-construction invertebrate survey to be completed in subsequent years post-construction to inform management practices. | Construction Contractor | Environmental Statement |  |  |

##### Saltmarsh Maps

###### Extent of Saltmarsh

Offices at

Bristol

Coleshill

Doncaster

Dublin

Edinburgh

Exeter

Glasgow

Haywards Heath

Isle of Man

Leeds

Limerick

Newcastle upon Tyne

Newport

Peterborough

Portsmouth

Saltaire

Skipton

Tadcaster

Thirsk

Wallingford

Warrington

Registered Office

1 Broughton Park

Old Lane North

Broughton

SKIPTON

North Yorkshire

BD23 3FD

United Kingdom

+44(0)1756 799919

info@jbaconsulting.com

www.jbaconsulting.com

Follow us:

Jeremy Benn Associates Limited

Registered in England 3246693

JBA Group Ltd is certified to:

ISO 9001:2015

ISO 14001:2015

ISO 27001:2013

ISO 45001:2018