| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-024951 |
| Permit Reference Number: | EPR-EB3205CY |
| Permit Holder: | 4Recycling Limited |
| Site Address: | Mount Farm, Ffrith, Wrexham, LL11 5HU |
| Grid Reference: | Farm location: SJ 26379 55521 Fields: 1 SJ2638055410 2 SJ2615055480 3 SJ2606055410 4 SJ2596055330 6 SJ2566055400 10 SJ2598055130 11 SJ2645055250 12 SJ2663055340 13 SJ2665055580 14 SJ2677055420 15 SJ2687055500 16 SJ2677055760 17 SJ2681057070 18 SJ2698057030 19 SJ2687056910 Storage: SJ2654555557- field heap- field 13 SJ2691956878- field heap- field 19 SJ2519456027- field heap- not within or next to spreading fields- use justified 07/05/24 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team:WasteReg.NE@cyfoethnaturiolcymru.gov.uk Environment team: WrexhamFlintshireEnvironmentTeam@cyfoethnaturiolcymru.gov.uk NLO: Don.Rother@environment-agency.gov.uk |
| Date due: | 12/03/2024 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating A- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | EA public register |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and deployment fee- Charging Scheme |
| Fee on MyNRW | Y | Transaction number: 2126981 Date: 16/02/2024 Ref: BS/160224/12 BYPRODUCT RECOVERY WAA0032 CH0048 APP BYPROD1419 Amount: 1,837.00 |
| Location map | Y | 2. Spreading Area (2024) |
| Benefit statement | Y | 3. Agricultural benefit statement(2024) 1. LPD1 Supplement_Parcels of Land (2024) 8. EA Technical Note on EBP3205CY haz coding 06 02 05 (Paul Murray June 2023) 9. Farmer consent (All fields) 4Recycling Ltd. Approval to sign on behalf (2024) |
| Waste Analysis | Y | 4. TATA Lime waste analysis (AUG 2023) 4.1 Waste Interpretation_Dry matter (Tata Lime 2024) |
| Soil Analysis | Y | 5. Soil analysis (2024) |
| EIA Agri Regulations evidence | Y | 7. Field photo evidence Mount Farm |
| Site Specific Risk Assessment | Y | 6. SSRA_Mount Farm (2024) |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| - About the Permit |  |  |
| ## Has there been any pre-application discussions? | Y | This application is a resubmission of deployment ref PAN-023482 which was returned to us 1/12/2023. |
| ## Has the permit number been provided? | Y |  |
| ## Has the permit holder name been provided? | Y |  |
| ## What permit is being deployed under? | Bespoke |  |
| - Contact details |  |  |
| ## Has contact information been provided? | Y | Esther Koroma |
| ## Has the technically competent manager details been provided? | Y | Nigel Box |
| ## Has the nominated competent person details been provided? | Y | Jolyon Pridding |
| - About the land you want to treat |  |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | LPD1 form: 56 261 0093 NRW mapping system: 56/261/0093- RG AND E EDWARDS- MOUNT FARM |
| ## Has all of parcels of land table been completed? | Y | On separate document: 1. LPD1 Supplement_Parcels of Land (2024).pdf |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y | 48 ha |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | N | LPD1 form: Mr R.G Edwards- Mount Farm NRW mapping system: RG AND E EDWARDS- MOUNT FARM |
| ## Has written consent of all owners(s) or occupiers(s) been given? | Y | Meets guidance |
| - Previous land treatment |  |  |
| Has all of the previous land treatment table been completed? | n/a | No previous land treatment (on form). ABS states “25 t/ha of FYM across fields 1-12.” Resolved 07/05/24 |
| - Which risk band does the activity fall within? - SR2010No4 List A wastes (Lower risk) - Location risk - Low risk deployment - Medium risk (1) deployment - Medium risk (2) deployment - High risk deployment |  |  |
| ## Which risk band does the activity fall within? | Bespoke |  |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) or High risk deployment has a SSRA been provided? | Y |  |
| - Additional information on sensitive receptors |  |  |
| ## What type of risk assessment has been submitted? | benefit statement and bespoke permit- site‐specific risk assessment |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | n/a |  |
| If no have details of the assessment evidence to support this conclusion been provided? | Y | Document: 7. Field photo evidence Mount Farm.pdf |
| - About the waste |  |  |
| Have all parts of the waste-types table been completed? - List of waste code (6 digit) - Description of waste (this must be an accurate description specific to this waste that refers to the List of Waste written description) - Physical form of waste/Is it stackable or non-stackable - Is the waste high in readily available nitrogen? - Producer of waste & permit number (if applicable) - Total amount being spread/used (tonnes) | Y | Form gives waste as “06 02 05*” The “*” usually denotes that the waste is hazardous, however in the permit, this is not given as a hazardous waste. This is explained in the document “8. EA Technical Note on EBP3205CY haz coding 06 02 05 (Paul Murray June 2023).pdf” supplied with the application All other required information has been provided. |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y | Waste from a sole individual producer |
| - About the storage |  |  |
| Has all of waste storage table been completed? | Y |  |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y | Each field heap given as the maximum waste tonnage |
| 18. Supporting documents |  |  |
| Has checklist been completed? | Y |  |
| 20. Confidentiality and national security |  |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| 21. Declaration |  |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | Y | Updated document provided: 4Recycling Ltd. Approval to sign on behalf (2024) |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has the operator confirmed they will comply with the NRW pre-notification requirements. | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y | SJ 25194 56027- field heap- not within or next to spreading fields use justified 07/05/24 |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no storage or spreading within 10 meters) (buffer zones needed) | Y | Ponds in fields 2/4/10 are buffered Water courses along boundaries of field 16 are buffered. |
| groundwater source protection zones (permit condition: no storage or spreading in SPZ1) (high risk deployment if in SPZ2) | n/a |  |
| location of all storage sites for the waste to be spread | Y | SJ 25194 56027- field heap- not within or next to spreading fields |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no storage or spreading within 50 meters) | Y | Issues around field 16 all buffered, issue alongside field 6 buffered |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | Y | As above, shown on plan |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | Y | As above, shown on plan |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | n/a |  |
| location of any public rights of way (shown on map) | Y | All shown (and buffered) |
| any buildings or houses within 250m (shown on map) | Y |  |
| Within Groundwater Safeguard Zone | n/a |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Habitats - Section 7 species - Scheduled Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Hedges - Ditches | Y | Reedbeds, Lowland dry acid grassland and Lowland calcareous grassland, Lowland dry acid grassland and Lowland calcareous grassland (alongside fields 14/15), Threatened Bryophyte (pink square to the south of the fields). None within fields- no action needed. Ancient woodland and LWS, shown and buffered. |
| Flood risk | Y | Low risk- no action needed |
| What is the slope of the fields? | 0-7 degrees (transparent)- parts of 1/6/12/13/14/15/16, all of 2/3/4/10/11/17/18/19, Storage field heap- field 19. Storage field heap at SJ 25194 56027 use justified 07/05/24 7 -12 degrees (yellow)- parts of 1/6/12/13/14/15/16, Storage field heap- field 13 12 degrees plus (red)- parts of 1 (no spread areas, however these do not match- RFI), 13/14/15 See below for full assessment |  |

| Waste Storage | Y N n/a | Comments |
| --- | --- | --- |
| Is waste storage occurring at the place where waste is being deployed? | N | field heap at SJ 25194 56027 is some distance away. use justified 07/05/24 |
| Is only waste as specified in this deployment being stored in the proposed storage locations? i.e. slurry in lagoons Note: There may be multiple deployments using one storage location | Y |  |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in the same place one constructed within the last two years? or in different location? | n/a | No previous deployment |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in a field: - liable to flooding or becoming waterlogged, or - within 50 metres of a borehole, spring or well or - within 10 metres of surface water or a land drain - within 30 meters of a watercourse on steeply sloping land | N | Storage field heap- field 13 is in an area of slope 7-12 degrees |
| If the waste is non-stackable is the waste stored within either a vessel, a covered building, or on an impermeable surface | n/a |  |
| If the waste is high RAN and it is being stored within 200 metres of a European Site (SAC/SPA), Ramsar Site or a Site of Special Scientific Interest (SSSI) is the waste being covered to prevent loss of ammonia? (condition 3.2.4 of the permit) | n/a | Not high RAN |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | Y |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | Y | Liming values, conductivity |
| - If yes; have they been tested for? | Y |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y | field 4 is counted twice with differing numbers: “MOUNT FARM 2/3/4” “MOUNT FARM 4/5”- RFI Confirmed “MOUNT FARM 2/3/4” is correct and “MOUNT FARM 4/5” not used in application 07/05/24 |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y |  |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | n/a | Not claiming for nutrients, although waste does provide some nutrients |
| - Timing of application | Y |  |
| - Amount of waste to be applied and application rate | n/a |  |
| - Soil chemical properties (e.g. liming and pH) | Y | Liming is slightly overapplied (max of 2.2kg/ha) |
| - Soil physical properties (e.g. organic matter and soil structure) | n/a |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| Timing of application- if any of the wastes are high RAN, are they being spread OUTSIDE of the closed periods | n/a | Not High RAN |
| Groundwater Safeguard Zone for nitrate- if any of the wastes are high RAN, are the application rates less than 50 m3/ha, with a minimum 3 week gap between applications | n/a | Not High RAN |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | n/a |  |
| - Included the nutrient level in their environmental risk assessment | n/a |  |
| - Stated the source of information that they used to set the nutrient level. | n/a |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y |  |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need, as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If the cropping is any of the following, is the maximum nitrogen limit in line with that given in The Water Resources (Control of Agricultural Pollution) (Wales) Regulations 2021: - 0 kg/ha- Field beans, Peas - 120 kg/ha- Sugar beet - 150 kg/ha- Asparagus, Carrots, Forage maize. Radish, Spring barley, Swedes - 180 kg/ha- Spring-sown wheat, Winter barley - 220 kg/ha- Autumn or early winter sown wheat | n/a |  |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | n/a |  |
| - Is it only for operational purposes? | n/a |  |
| - Have individual application rates for each waste been given? | n/a |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | n/a |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients given by the applicant match NRW calculations? | Y |  |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y | 25 t/ha of FYM across fields 1-12 |
| - How much material was spread? | Y |  |
| - At what application rate material was spread? | Y |  |
| - The nutrients supplied by the material? | Y |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | Y | SSRA provided as permit is bespoke, not because it is in a high risk location |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | Y |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | Y |  |
| - Pathways | Y |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | Y |  |
| - Measures that will minimise or mitigate the impact will be put in place | Y |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | N |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | n/a |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | N |

Comments

Consultation responses: - Liming amounts- see above/below - References in ABS to SR2010No4- on in terms of meeting recovery - Stockpile SJ 25194 56027- RFI. “use justified 07/05/24 - Some no spread areas for slope above 12 degrees have not been identified or are in incorrect place e.g. field 1- RFI. No spread areas increased 07/05/24

MyMap/location and slope assessments: MyMap/location assessment - Required buffers in place - See below for slope Slope assessment Field number: parts of 1/6/12/13/14/15/16, Storage field heap- field 13 Slope of the land: 7 -12 degrees (yellow)- Soil type: Sandy Loam Wastes types applied (solid, liquid, sludge etc.): Solid Cropping type: Permanent pasture (Grazing) Method of application: The lime will be spread using a standard rear discharge spreader (with spinning discs). The spreader is machine towable with the ability to adjust disc spreading rates, to reflect variable rates across the bespoke application. Weather conditions before during and after spreading: - When the ground and weather conditions are suitable (i.e. avoiding frosty, moisture laden or dry and cracked conditions) with little to no wind. Optimum conditions for spreading are a following light dew with zero wind. - Liming in September is standard practice (AHDB, Timing of liming on a grazing platform ¦ AHDB). It is also recommended that Liming is needed throughout the grazing rotation for grazing. - However, to mitigate negative effect of conductivity and limit leaf scorch, the application will be limited an application in autumn 2024 (September- October). - Wastes will be applied when ground and weather conditions are suitable, following CoGAP to avoid soil damage including wheel ruts, compaction, structural damage, erosion, and run-off. Other risk factors not mentioned above: Low application rates and total tonnages. Conclusions (is run off risk high?): No Field number: parts of 1 (no spread areas, however these do not match- RFI), 13/14/15 Slope of the land: 12 degrees plus (red) Soil type: Sandy Loam Wastes types applied (solid, liquid, sludge etc.): Solid Cropping type: Permanent pasture (Grazing) Method of application: The lime will be spread using a standard rear discharge spreader (with spinning discs). The spreader is machine towable with the ability to adjust disc spreading rates, to reflect variable rates across the bespoke application. Weather conditions before during and after spreading: - When the ground and weather conditions are suitable (i.e. avoiding frosty, moisture laden or dry and cracked conditions) with little to no wind. Optimum conditions for spreading are a following light dew with zero wind. - Liming in September is standard practice (AHDB, Timing of liming on a grazing platform ¦ AHDB). It is also recommended that Liming is needed throughout the grazing rotation for grazing. - However, to mitigate negative effect of conductivity and limit leaf scorch, the application will be limited an application in autumn 2024 (September- October). - Wastes will be applied when ground and weather conditions are suitable, following CoGAP to avoid soil damage including wheel ruts, compaction, structural damage, erosion, and run-off. Other risk factors not mentioned above: - Some steeply sloping areas have been highlighted as “no spreading” based on a previous returned application. These don’t match our mapping so will have to be corrected. - Low application rates and total tonnages. Conclusions (is run off risk high?): No, given the waste type and how it is spread, however no spread areas need to be corrected. No spread areas increased 07/05/24

CPH details: - LPD1 form: 56 261 0093 - NRW mapping system: 56/261/0093- RG AND E EDWARDS- MOUNT FARM

Protection of habitats and ecosystems - Evidence provided and meets guidance

Agricultural benefit calculations: - Over application of P on fields 1, 11 and 12 (by 2.42 kg/ha)- P index 2- minimal over application, therefore permitted. - Over application of lime on all fields ranging from 0.97 kg/ha to 2.22 kg/ha- minimal over application, therefore permitted. - High conductivity/pH- justification and management procedures included in the ABS.

Request for information: Sent 02/05/24 - Previous land treatment - Storage location - Soil analysis - Slope All points resolved 07/05/24.

Other information to note: n/a

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date | 08/05/24 |
| ## Effective date | 08/05/24 |
| ## Permitting Officer Date | Louise Bailey 08/05/24 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 08/05/2024 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 385 |
| Total | 445 |