| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-024750 |
| Permit Reference Number: | EPR-GP3792SK |
| Permit Holder: | ByProduct Recovery Limited |
| Site Address: | Bwlchmwr Farm, Brynteg, Llanybydder, SA40 9XA |
| Grid Reference: | Farm location: SN 57750 43030 Fields: 2 SN 57980 43040 4 SN 57590 42980 6 SN 58120 42480 7 SN 57990 42340 8 SN 58280 42280 9 SN 58500 42430 10 SN 58410 42530 12 SN 58080 42210 13 SN 57380 42970 0273 SN 52990 40820 1976 SN 53190 40770 0656 SN 53050 40620 2194 SN 53200 40460 8690 SN 52840 40890 Storage: SN 57960 42920 SN 58310 42450 SN 57990 42230 SN 53100 40450 SN 55230 43340 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: wasteregulation.swwales@cyfoethnaturiolcymru.gov.uk Environment team: Carms.EnvironmentTeam@cyfoethnaturiolcymru.gov.uk NLO: Don.Rother@environment-agency.gov.uk |
| Date due: | 23/02/2024 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating B- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | English- NLO |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and deployment fee- Charging Scheme |
| Fee on MyNRW | Y | Amount: £1561.00 Date cleared on MyNRW: 02/02/24 Transaction number: 2126690 |
| Location map | Y |  |
| Benefit statement | Y |  |
| Waste Analysis | Y |  |
| Soil Analysis | Y |  |
| EIA Agri Regulations evidence | Y |  |
| Site Specific Risk Assessment | Y | High risk location only |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| - About the Permit |  |  |
| ## Has there been any pre-application discussions? | N |  |
| ## Has the permit number been provided? | Y |  |
| ## Has the permit holder name been provided? | Y |  |
| ## What permit is being deployed under? | SR2010 No4 |  |
| - Contact details |  |  |
| ## Has contact information been provided? | Y |  |
| ## Has the technically competent manager details been provided? | Y |  |
| ## Has the nominated competent person details been provided? | Y |  |
| - About the land you want to treat |  |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | Provided by Applicant: 55/294/0014 Mr Dafydd Davies & 55/055/0031 GA & A Jones Signed consent has not been provided for GA and A jones. This was resolved in the response for information Confirmed on GIS |
| ## Has all of parcels of land table been completed? | Y |  |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y |  |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | Y |  |
| ## Has written consent of all owners(s) or occupiers(s) been given? | Y |  |
| - Previous land treatment |  |  |
| Has all of the previous land treatment table been completed? | n/a | No spreading took place in 2023 |
| - Which risk band does the activity fall within? - SR2010No4 List A wastes (Lower risk) - Location risk - Low risk deployment - Medium risk (1) deployment - Medium risk (2) deployment - High risk deployment |  |  |
| ## Which risk band does the activity fall within? | Medium (1) |  |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) or High risk deployment has a SSRA been provided? | n/a |  |
| - Additional information on sensitive receptors |  |  |
| ## What type of risk assessment has been submitted? | benefit statement |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | N |  |
| If no have details of the assessment evidence to support this conclusion been provided? | Y | Field photos for fields 8 and 12 are missing. This was resolved from the request for information response. fields 2, 4, 6, 7, 13, & 8690 appear to contain semi natural habitat/unimproved grass land. This was resolved from the request for information response. |
| - About the waste |  |  |
| Have all parts of the waste-types table been completed? - List of waste code (6 digit) - Description of waste (this must be an accurate description specific to this waste that refers to the List of Waste written description) - Physical form of waste/Is it stackable or non-stackable - Is the waste high in readily available nitrogen? - Producer of waste & permit number (if applicable) - Total amount being spread/used (tonnes) | Y |  |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y |  |
| - About the storage |  |  |
| Has all of waste storage table been completed? | Y |  |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y |  |
| 18. Supporting documents |  |  |
| Has checklist been completed? | Y |  |
| 20. Confidentiality and national security |  |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| 21. Declaration |  |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | Y |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has the operator confirmed they will comply with the NRW pre-notification requirements. | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y |  |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no storage or spreading within 10 meters) (buffer zones needed) | Y | Correctly buffered. |
| groundwater source protection zones (permit condition: no storage or spreading in SPZ1) (high risk deployment if in SPZ2) | n/a |  |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no storage or spreading within 50 meters) | n/a |  |
| location of all storage sites for the waste to be spread | Y |  |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | n/a |  |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | n/a |  |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | n/a |  |
| location of any public rights of way (shown on map) | Y |  |
| any buildings or houses within 250m (shown on map) | Y |  |
| Within Groundwater Safeguard Zone | n/a |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Habitats - Section 7 species - Scheduled Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Hedges - Ditches | Y |  |
| Flood risk | Y | No issue to note |
| What is the slope of the fields? | 0-7 degrees (transparent)- fields: all fields 7 -12 degrees (yellow)- portions of fields: 9,10,8,12,7,6,2, 13,4,2194,0656,1976,0273, and 8690 12 degrees plus (red)- Portions of fields: 0273,2194,1976,13,4,12,8 and 9, Following RFI these slope are covered by a no spread zone. |  |

| Waste Storage | Y N n/a | Comments |
| --- | --- | --- |
| Is waste storage occurring at the place where waste is being deployed? | Y |  |
| Is only waste as specified in this deployment being stored in the proposed storage locations? i.e. slurry in lagoons Note: There may be multiple deployments using one storage location | Y |  |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in the same place one constructed within the last two years? or in different location? | Different |  |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in a field: - liable to flooding or becoming waterlogged, or - within 50 metres of a borehole, spring or well or - within 10 metres of surface water or a land drain - within 30 meters of a watercourse on steeply sloping land | N | Field heap D is stored a steep slope, but no water course is within 30 metres. Following RFI 1 stockpile was relocated. |
| If the waste is non-stackable is the waste stored within either a vessel, a covered building, or on an impermeable surface | n/a | No details provided for farm store Following RFI 1 response farm store storage was removed. |
| If the waste is high RAN and it is being stored within 200 metres of a European Site (SAC/SPA), Ramsar Site or a Site of Special Scientific Interest (SSSI) is the waste being covered to prevent loss of ammonia? (condition 3.2.4 of the permit) | n/a | Not stored within 200m |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | Y |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | Y | 02 02 04 requires fats oils and grease analysis, odour should also be considered. |
| - If yes; have they been tested for? | Y |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y |  |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y |  |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | Y |  |
| - Soil physical properties (e.g. organic matter and soil structure) | Y |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| Timing of application- if any of the wastes are high RAN, are they being spread OUTSIDE of the closed periods | n/a |  |
| Groundwater Safeguard Zone for nitrate- if any of the wastes are high RAN, are the application rates less than 50 m3/ha, with a minimum 3 week gap between applications | n/a |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | n/a |  |
| - Included the nutrient level in their environmental risk assessment | n/a |  |
| - Stated the source of information that they used to set the nutrient level. | n/a |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y |  |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need, as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If the cropping is any of the following, is the maximum nitrogen limit in line with that given in The Water Resources (Control of Agricultural Pollution) (Wales) Regulations 2021: - 0 kg/ha- Field beans, Peas - 120 kg/ha- Sugar beet - 150 kg/ha- Asparagus, Carrots, Forage maize. Radish, Spring barley, Swedes - 180 kg/ha- Spring-sown wheat, Winter barley - 220 kg/ha- Autumn or early winter sown wheat | n/a | Grass |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | Y |  |
| - Is it only for operational purposes? | Y |  |
| - Have individual application rates for each waste been given? | Y |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | Y |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients given by the applicant match NRW calculations? | Y |  |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | n/a |  |
| - How much material was spread? | n/a |  |
| - At what application rate material was spread? | n/a |  |
| - The nutrients supplied by the material? | n/a |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | n/a |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | n/a |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | n/a |  |
| - Pathways | n/a |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | n/a |  |
| - Measures that will minimise or mitigate the impact will be put in place | n/a |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | N |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | n/a |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | N |

Comments

Consultation responses: No comments

MyMap/location and slope assessments: https://cyfoethnaturiolcymru.sharepoint.com/:w:/r/teams/Regulatory/Permitting/Mobile%20Plant/EPR-GP3792SK/PAN-024750/PAN-024750%20Mapping%20report.docx?d=w19a4c5e1acb14651a96ed3f552e12459&csf=1&web=1 MyMap/location assessment 0-7 degrees (transparent)- fields: all fields 7 -12 degrees (yellow)- portions of fields: 9,10,8,12,7,6,2,13,4,2194,0656,1976,0273, and 8690 12 degrees plus (red)- Portions of fields: 0273,2194,1976,13,4,12,8 and 9 Slope assessment Field number: portions of fields: 9,10,8,12,7,6,2,13,4,2194,0656,1976,0273, and 8690 Slope of the land: 7-12 degrees Soil type: medium Wastes types applied (solid, liquid, sludge etc.): sludge and sludge cake Cropping type: grass Method of application: Trailing shoe. Weather conditions before during and after spreading: CoGAP will be observed Other risk factors not mentioned above: n/a Conclusions (is run off risk high?): medium due to spread rates. RFI required. Low, applicant is now spreading waste at root level using a trailing shoe applicator. Slope assessment Field number: portions of fields: 0273,2194,1976,13,4,12,8 and 9 Slope of the land:12+ degrees Soil type: medium Wastes types applied (solid, liquid, sludge etc.): sludge and sludge cake Cropping type: grass Method of application: Trailing shoe Weather conditions before during and after spreading: CoGAP will be observed Other risk factors not mentioned above: n/a Conclusions (is run off risk high?): high due to spread rates. RFI required. Low following RFI all 12+ slopes are covered by a no spread zone.

CPH details: Provided by Applicant: 55/294/0014 Mr Dafydd Davies & 55/055/0031 GA & A Jones Signed consent has not been provided for GA and A jones. This was rectified in the request for information 2 response. Confirmed on GIS

Protection of habitats and ecosystems Fields 2,4,6,7,13 and 8690 appear to be unimproved grassland or semi natural habitat, fields 12 and 8 are missing from EIA evidence. This has been

Agricultural benefit calculations: Liarage waste stream has been marked as ‘sample as received’ in analysis however waste additions are very high, adding more phosphate and potash than is acceptable, waste interpretation provided appears to have been calculated on a dry matter basis. RFI response amended spread rates and they are now acceptable.

Request for information: SENT: 30/04/24 1. Consent for fields 0273, 1976, 0656, 2194, 8690 2. Manure and animal faeces from abattoir holding areas/lairage? 3. Liarage testing and waste interpretation 4. Spreading methods 5. Slopes 6. Stock pile on slope 7. Stock piles odour 8. Farm stores/storage tanks 9. EIA missing fields 12 and 8 10. EIA fields 2,4,6,7,13 and 8690 semi natural/unimproved. Response 1: 02/05/24 Concluded all issued aside from questions 1,9 and 10 Response 2: 09/05/24 1. The applicant has now provided written consent to spread wastes on these fields from A Jones. 9. The applicant has provided EIA evidence for fields 12 & 8 and show the fields are agriculturally improved. No further action needed. 10. The applicant has provided photos of fields 2,4,6,7,13 and 8690 and shows the fields are agriculturally improved. No further action needed.

Other information to note: n/a

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date | 09/05/24 |
| ## Effective date | 09/05/24 |
| ## Permitting Officer Date | Owen Clee (on behalf of Ethan Banfield) 09/05/24 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 09/05/2024 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 1040 |
| Total | 1,100 |