| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-025308 |
| Permit Reference Number: | EPR-AB3891CX |
| Permit Holder: | Mr Daniel James and Mrs Carys James |
| Site Address: | Ffynnoncyff Farm, Ferwig, Cardigan, Ceredigion, SA43 1QD |
| Grid Reference: | Farm location: SN 19382 50533 Fields: 8/6895 SN 19681 50953 9/8498 SN 19847 50972 11/7715 SN 19783 51150 12/5507 SN 19574 51068 13/7176 SN 19720 50767 20/6128 SN 19603 51277 21/4734 SN 19471 51345 Ffynnoncyff 22 SN 19460 50325 Ffynnoncyff 23 SN 19288 50416 F/CYFF 24/2821 SN 19277 50224 Ffynnoncyff 25 SN 19098 50195 Ffynnoncyff 26 SN 19395 50700 Ffynnoncyff 27 SN 19518 50846 Ffynnoncyff 29 SN 19585 50251 Ffynnoncyff 30 SN 19799 50234 Storage: SN 19962 51001 SN 19395 50573 SN 19770 50920 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: GwastraffaGorfodi.Canolbarth@cyfoethnaturiolcymru.gov.uk Environment team: CeredigionEnvironmentTeam@cyfoethnaturiolcymru.gov.uk NLO: Malcolm.Dines@cyfoethnaturiolcymru.gov.uk |
| Date due: | 22/04/2024 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating B- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | Welsh- DMS |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and deployment fee- Charging Scheme |
| Fee on MyNRW | Y | Amount: £2020.70 Date cleared on MyNRW: 28/03/24 Transaction number: 2128408 Additional fee only: Amount: £101.20 Date cleared on MyNRW: 04/04/24 Transaction number: 2128759 |
| Location map | Y |  |
| Benefit statement | Y |  |
| Waste Analysis | Y |  |
| Soil Analysis | Y |  |
| EIA Agri Regulations evidence | Y |  |
| Site Specific Risk Assessment | Y | High risk location only |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| - About the Permit |  |  |
| ## Has there been any pre-application discussions? | Y |  |
| ## Has the permit number been provided? | Y |  |
| ## Has the permit holder name been provided? | Y |  |
| ## What permit is being deployed under? | SR2010 No4 |  |
| - Contact details |  |  |
| ## Has contact information been provided? | Y |  |
| ## Has the technically competent manager details been provided? | Y |  |
| ## Has the nominated competent person details been provided? | Y |  |
| - About the land you want to treat |  |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | Provided by applicant: 55/226/0017 Mr Eilir Williams Confirmed by GIS check. |
| ## Has all of parcels of land table been completed? | Y |  |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y |  |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | Y |  |
| ## Has written consent of all owners(s) or occupiers(s) been given? | Y |  |
| - Previous land treatment |  |  |
| Has all of the previous land treatment table been completed? | Y | PAN-020931 |
| - Which risk band does the activity fall within? - SR2010No4 List A wastes (Lower risk) - Location risk - Low risk deployment - Medium risk (1) deployment - Medium risk (2) deployment - High risk deployment |  |  |
| ## Which risk band does the activity fall within? | High |  |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) or High risk deployment has a SSRA been provided? | Y |  |
| - Additional information on sensitive receptors |  |  |
| ## What type of risk assessment has been submitted? | site‐specific risk assessment benefit statement |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | N |  |
| If no have details of the assessment evidence to support this conclusion been provided? | Y | Photos of fields with quadrates showing the fields are of established agriculture and do not contain habitat |
| - About the waste |  |  |
| Have all parts of the waste-types table been completed? - List of waste code (6 digit) - Description of waste (this must be an accurate description specific to this waste that refers to the List of Waste written description) - Physical form of waste/Is it stackable or non-stackable - Is the waste high in readily available nitrogen? - Producer of waste & permit number (if applicable) - Total amount being spread/used (tonnes) | Y |  |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y | Google, Welsh water, dms. |
| - About the storage |  |  |
| Has all of waste storage table been completed? | Y |  |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y |  |
| 18. Supporting documents |  |  |
| Has checklist been completed? | Y |  |
| 20. Confidentiality and national security |  |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| 21. Declaration |  |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | Y |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has the operator confirmed they will comply with the NRW pre-notification requirements. | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y |  |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no storage or spreading within 10 meters) (buffer zones needed) | Y |  |
| groundwater source protection zones (permit condition: no storage or spreading in SPZ1) (high risk deployment if in SPZ2) | n/a |  |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no storage or spreading within 50 meters) | Y |  |
| location of all storage sites for the waste to be spread | Y |  |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | n/a |  |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | n/a |  |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | Y |  |
| location of any public rights of way (shown on map) | Y |  |
| any buildings or houses within 250m (shown on map) | Y |  |
| Within Groundwater Safeguard Zone | n/a |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Habitats - Section 7 species - Scheduled Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Hedges - Ditches | Y |  |
| Flood risk | Y | Small areas contain a low risk of flooding from small water courses, CoGAP and permit conditions should mitigate this risk. |
| What is the slope of the fields? | 0-7 degrees (transparent)- fields: all fields 7-12 degrees (green/yellow) -Portions of fields: 21/4734, 26, 27 13/7176, 11/7715, 20/6128 12 degrees plus (purple/red)- fields: all covered by no spread areas. |  |

| Waste Storage | Y N n/a | Comments |
| --- | --- | --- |
| Is waste storage occurring at the place where waste is being deployed? | Y |  |
| Is only waste as specified in this deployment being stored in the proposed storage locations? i.e. slurry in lagoons Note: There may be multiple deployments using one storage location | Y |  |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in the same place one constructed within the last two years? or in different location? | Different |  |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in a field: - liable to flooding or becoming waterlogged, or - within 50 metres of a borehole, spring or well or - within 10 metres of surface water or a land drain - within 30 meters of a watercourse on steeply sloping land | N | No problems to note |
| If the waste is non-stackable is the waste stored within either a vessel, a covered building, or on an impermeable surface | Y |  |
| If the waste is high RAN and it is being stored within 200 metres of a European Site (SAC/SPA), Ramsar Site or a Site of Special Scientific Interest (SSSI) is the waste being covered to prevent loss of ammonia? (condition 3.2.4 of the permit) | n/a | Not high RAN |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | Y | Only for 02 05 02 not required for 19 09 02 waste streams. |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | Y | 190902 – Iron and Aluminium 020502- FOGs pH and conductivity, odorous waste. |
| - If yes; have they been tested for? | Y |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y |  |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y |  |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | Y |  |
| - Soil physical properties (e.g. organic matter and soil structure) | Y |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| Timing of application- if any of the wastes are high RAN, are they being spread OUTSIDE of the closed periods | n/a |  |
| Groundwater Safeguard Zone for nitrate- if any of the wastes are high RAN, are the application rates less than 50 m3/ha, with a minimum 3 week gap between applications | n/a |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | Y |  |
| - Included the nutrient level in their environmental risk assessment | Y |  |
| - Stated the source of information that they used to set the nutrient level. | Y |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y |  |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need, as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If the cropping is any of the following, is the maximum nitrogen limit in line with that given in The Water Resources (Control of Agricultural Pollution) (Wales) Regulations 2021: - 0 kg/ha- Field beans, Peas - 120 kg/ha- Sugar beet - 150 kg/ha- Asparagus, Carrots, Forage maize. Radish, Spring barley, Swedes - 180 kg/ha- Spring-sown wheat, Winter barley - 220 kg/ha- Autumn or early winter sown wheat | n/a | Grass silage |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | Y |  |
| - Is it only for operational purposes? | Y | Liquid may be mixed |
| - Have individual application rates for each waste been given? | Y |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | Y |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients given by the applicant match NRW calculations? | Y |  |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y |  |
| - How much material was spread? | Y |  |
| - At what application rate material was spread? | Y |  |
| - The nutrients supplied by the material? | Y |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | Y |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | Y |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | Y |  |
| - Pathways | Y |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | Y |  |
| - Measures that will minimise or mitigate the impact will be put in place | Y |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | Y | PAN-025308 App 4 |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | N |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | N |
| No FORM 1 completed: HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site. The wastes will be applied following the Codes of Good Agricultural Practice. The maximum application rate for each field will be split into multiple applications throughout the growing season and will not exceed 50t/ha in any one application to a field. No waste will be spread within 10m of any ditch, pond or surface water, within 50m of any spring, well, borehole, or reservoir that supplies water for human consumption or farm dairies. Spreading to take place only when conditions are suitable with no or little wind and when the potential of any. Storage tanks and field stockpiles are well away from the SAC’s. The closest field to SAC Aberwrarth-Carreg Wylan is 200m away with fields roads and a cliff between the spread area and the SAC. The closest field to Caeau crug bychan is 474m away with fields and roads between there and the spread area. Waste is being applied using low trajectory accurate methods, dribble bar or shallow slot injection. These measures effectively remove any direct pathway and reduce the risk to the SACs to negligible. |  |

Comments

Consultation responses: No comments

MyMap/location and slope assessments: MyMap/location assessment PAN-025308 Mapping report Slope assessment 0-7 degrees (transparent)- fields: all fields 12 degrees plus (purple/red)- fields: all covered by no spread areas. Field number: 21/4734, 26, 27 13/7176, 11/7715, 20/6128 Slope of the land:7-12 degrees Soil type: medium Wastes types applied (solid, liquid, sludge etc.): sludge and sludge cake Cropping type: grass Method of application: dribble bar for sludge, rear discharge spreader for cake. Weather conditions before during and after spreading: CoGAP will be observed. Other risk factors not mentioned above: n/a Conclusions (is run off risk high?): low, sloped areas are relatively small, appropriate buffer zones are in place, spread methods are low trajectory.

CPH details: Provided by applicant: 55/226/0017, Mr Eilir Williams, Confirmed by GIS check.

Protection of habitats and ecosystems: Fields are of established agricultural use and contain no areas of unimproved ground or other habitat.

Agricultural benefit calculations: Calculations closely match those of the applicant, save for rounding errors. A maintenance amount of nutrients is being applied to soils above target index and all fields additions are acceptable and in line with guidance.

Request for information: n/a

Other information to note: n/a

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date |  |
| ## Effective date |  |
| ## Permitting Officer Date | Ethan Banfield 05/06/2024 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Anna Gale (ToP) 06/06/2024 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 910 |
| Total |  |