Permit Number: 	AB3697CN			Operator: 		Radnor Hills Mineral Water Company Ltd

Facility:		Radnor Hills			Form Number: 	Water1 / 12/08/2024

# Reporting of emissions to water (other than to sewer) and land for the period from DD/MM/YYYY to DD/MM/YYYY

| Emission Point | Substance / Parameter | Emission # Limit Value | Reference Period | Result [1] | Test Method [2] | Sample Date and Times [3] | Uncertainty [4] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| W1 | BOD | 5 mg/l | Flow weighted sample or composite sample |  | BS EN 1899-1 |  |  |
| W1 | COD | 100 mg/l | Flow weighted sample or composite sample |  | ISO 6060, or BS ISO 15705 |  |  |
| W1 | Ammonical Nitrogen as N | 0.5 mg/l | Flow weighted sample or composite sample |  | BS EN ISO 11732 |  |  |
| W1 | Total Nitrogen | 20 mg/l | Flow weighted sample or composite sample |  | EN 12260 or EN ISO 11905-1 |  |  |
| W1 | Soluble Reactive Phosphorus as P | 0.2 mg/l | Instananeous |  | BS EN ISO 15681-1 or BS EN ISO 15681-2 |  |  |
| W1 | Total Phosphorus as P | 2 mg/l | Instananeous |  | EN ISO 6878, EN ISO 15681-1 and -2, EN ISO 11885 |  |  |
| W1 | Total suspended solids | 5.00 mg/l | Flow weighted sample or composite sample |  | BS EN 872 |  |  |
| W1 | Flow | 8.1 m3/hour | Integrated daily flow rate |  | MCERTS certified equipment, method as per M18 |  |  |
|  |  | 194.4 m3/day | Integrated daily flow rate |  |  |  |  |
| W1 | Temperature | Max 25 °C | Instantaneous |  | MCERTS certified equipment, method as per M18 |  |  |
| W1 | pH | 6.0-9.0 | Instantaneous |  | EN ISO 10523 |  |  |
| W1 | Chloride | No limit set | Instantaneous |  | EN ISO 10304-1 or EN ISO 15682 |  |  |

- The result given is the maximum value (or the minimum value in the case of a limit that is expressed as a minimum) obtained during the reporting period, expressed in the same terms as the emission limit value. Where the emission limit value is expressed as a range, the result is given as the ‘minimum – maximum’ measured values.

- Where an internationally recognised standard test method is used the reference number is given. Where another method that has been formally agreed with Natural Resources Wales is used, then the appropriate identifier is given. In other cases the principal technique is stated, for example gas chromatography.

- For non-continuous measurements the date and time of the sample that produced the result is given. For continuous measurements the percentage of the process operating time covered by the result is given.

- The uncertainty associated with the quoted result at the 95% confidence interval, unless otherwise stated.

Signed …………………………………………………………….  		Date………………………..

(Authorised to sign as representative of Operator)