# Natural Resources Wales permitting decisions

# Bespoke permit

We have decided to grant the permit for Woodlands Green operated by Lewis Homes (Woodlands Green) Limited.

The application number is PAN-015905

The permit number is EPR-CB3595FZ

The permit regulates the discharge of rainfall dependent site drainage during construction of a new housing estate. Treated runoff and excavation waters will be discharged at a maximum rate of 16.6 l/s to the Nant Melyn, a tributary of the Afon Ely. Treatment consists of a catchment sump which consists of two pits. The water can be attenuated, and the silt settled out before discharging to the Nant Melyn. Should excessive rainfall occur, a Siltbuster pump would pump water from the pits into a Selwood Silt Trap (5000 litres capacity) then into a Siltbuster followed by a 16m3 settlement tank for optional dosing with an anionic polymer flocculant.

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

Purpose of this document

This decision document:

- explains how the application has been determined

- provides a record of the decision-making process

- shows how all relevant factors have been taken into account

- justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

Structure of this document

- Annex 1 the decision checklist

- Annex 2 the consultation responses

# Annex 1: decision checklist

This checklist should be read in conjunction with the Duly Making checklist.

| ### Aspect | ### Justification / Detail | Criteria met |
| --- | --- | --- |
|  |  | Yes |
| Consultation |  |  |  |
| Scope of consultation | The consultation requirements were identified and implemented. The decision was taken in accordance with RGN 6 High Profile Sites, our Public Participation Statement and our Working Together Agreements. | ü |
| ### Responses to consultation | The consultation responses (Annex 2) were taken into account in the decision. The decision was taken in accordance with our guidance. | ü |
| Operator |  |  |  |
| Control of the facility | We are satisfied that the applicant (now the operator) is the person who will have control over the operation of the facility after the grant of the permit. The decision was taken in accordance with EPR RGN 1 Understanding the meaning of operator. | ü |
| European Directives |  |  |  |
| Applicable directives | All applicable European directives have been considered in the determination of the application. | ü |
| The site |  |  |  |
| Extent of the site of the facility | The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility including discharge points. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary. | ü |
| Biodiversity, Heritage, Landscape and Nature Conservation and a designated site under the Wildlife and Countryside Act. | The application is within the relevant distance criteria of a site of heritage, landscape or nature conservation, and/or protected species or habitat . We have not formally consulted on the application. Rhos Tonyrefail SSSI falls within the 1km screening distance but is located approximately 512m north of the discharge point and is not hydrologically connected. The decision was taken in accordance with our guidance. The River Ely is designated as an SINC. The Rhondda Cynon Taff County Ecologist was consulted and the response saved to DMS. No issues were raised. | ü |
| Habitat Regulation Assessment for a designated site under the EU Habitats Directive | No FORM 1 completed: HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site. The risk screening report did not screen in any Natura 2000 or Ramsar site. Note: From OGN 200 - no conceivable impact on any Natura 2000 site, by virtue of the scale or location or nature of the project. |  |
| Environmental risk | We have carried out a risk assessment on behalf of the operator. The operator considers this risk assessment is satisfactory. | ü |
| Operating techniques | We have reviewed the techniques used by the operator and compared these with the relevant guidance notes. The proposed techniques / emission levels for priorities for control are in line with the benchmark levels contained in the TGN and we consider them to represent appropriate techniques for the facility. | ü |
| The permit conditions |  |  |  |
| Emission limits | We have decided that emission limits should be set for the parameters listed in the permit. It is considered that the numeric limits described below will prevent significant deterioration of receiving waters. We have imposed numeric limits because either a relevant environmental quality or operational standard requires this. Numeric limits have been put in place for suspended solids 60 mg/l based on the capability of the treatment system. A pH range of 6-9 has been imposed to ensure that the discharged effluent remains neutral. | ü |
| Considerations of foul sewer | We agree with the operators justification for not connecting to foul sewer. | ü |
| Operator Competence |  |  |  |
| Environment Management System | There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. | ü |
| Sustainable Management of Natural Resources (SMNR) |  |  |
| Considerations of SMNR - Compliance with our General Purpose | We are satisfied that this decision is compatible with our general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources” | ü |
| Environment Wales Act 2016 |  |  |
| Biodiversity and resilience of ecosystems duty | Section 6 of the Environment Wales Act 2016 requires that we seek to maintain and enhance biodiversity in the exercise of our functions, and in so doing promote the resilience of ecosystems, in a manner that is consistent with the proper exercise of our functions. NRW is satisfied that in this case we have taken into account and had due regard to this duty in so far as it is consistent with the function of determining an application for a stand alone water discharge activity permit under EPR2016. | ü |

# Annex 2: Consultation

Summary of responses to consultation and the way in which we have taken these into account in the determination process.  (Newspaper advertising is only carried out for certain application types, in line with our guidance.)

Response received from

Rhondda Cynon Taff Local County Ecologist

Brief summary of issues raised

‘Clearly any site drainage discharge into the River Ely SINC or tributary SINC has the potential for water quality impacts and riparian habitat, corridor and associated species impacts. Controlling site treatment to ensure water quality of the receiving water is not compromised will be an important mitigation requirement and something that I am sure NRW will address in its consideration of permitting of any temporary discharge.’

Summary of actions taken or show how this has been covered

No actions to take