# Natural Resources Wales permitting decisions

# Variation

We have decided to issue the variation for Treforest Transfer Station operated by Veolia ES Cleanaway (UK) Limited.

The variation number is EPR/HP3795FS/V003

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

Purpose of this document

This decision document:

- explains how the application has been determined

- provides a record of the decision-making process

- shows how all relevant factors have been taken into account

- justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

Structure of this document

- Annex 1 the decision checklist

- Annex 2 the consultation and web publicising responses

# Annex 1: decision checklist

This document should be read in conjunction with the Duly Making checklist, the application and supporting information and permit/ notice.

| ### Aspect considered |  | ### Justification / Detail | Criteria met |  |
| --- | --- | --- | --- | --- |
|  |  |  | Yes |  |
| Consultation |  |  |  |  |
| Scope of consultation |  | The consultation requirements were identified and implemented. The decision was taken in accordance with our Public Participation Statement and our Working Together Agreements. |  |  |
| ### Responses to consultation and web publicising |  | The web publicising and consultation responses (Annex 2) were taken into account in the decision. No responses were received from: - Environmental Health- Rhondda Cynon Taff County Borough Council - Cwm Taf Health Board - Members of the public The decision was taken in accordance with our guidance. |  |  |
| Operator |  |  |  |  |
| Control of the facility |  | We are satisfied that the applicant (now the operator) is the person who will have control over the operation of the facility after the grant of the permit. The decision was taken in accordance with EPR RGN 1 Understanding the meaning of operator. |  |  |
| European Directives |  |  |  |  |
| Applicable directives |  | All applicable European directives have been considered in the determination of the application. Hazardous waste storage under the Industrial Emissions Directive is included in the permit. Therefore, storage capacity limits have been set in Table S1.1 of the permit. |  |  |
| Sustainable Management of Natural Resources (SMNR) |  |  |  |
| Considerations of SMNR - Compliance with our General Purpose | We are satisfied that this decision is compatible with our general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources. |  |  |
| Environment Wales Act 2016 – Biodiversity and resilience of ecosystems duty |  |  |  |
| Consideration of Section 6 | Section 6 of the Environment Wales Act 2016 requires that we seek to maintain and enhance biodiversity in the exercise of our functions, and in so doing promote the resilience of ecosystems, in a manner that is consistent with the proper exercise of our functions. NRW is satisfied that in this case, we have taken into account and had due regard to this duty in so far as it is consistent with the function of determining an application for an EPR permit. |  |  |
| The site |  |  |  |  |
| Extent of the site of the facility |  | The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary. |  |  |
| Site condition report |  | The operator has provided a description of the condition of the site. We consider this description is not satisfactory. The decision was taken in accordance with our guidance on site condition reports – guidance and templates (H5). A Geo-technical and Geo-environmental Site Investigation report (ref:46167 prepared Nov 2021 by Eastwood & Partners) has been supplied in support of the application but no updated site condition report has been submitted in line with the H5 guidance. The Site Investigation report does not meet the requirement of a site condition report and focusses on the assessment of risk as opposed to establishing current site condition and setting a baseline. i.e. there is no reference to the permit within the report. |  |  |
| Environmental Risk Assessment and operating techniques |  |  |  |  |
| Environmental risk |  | We have reviewed the operator's assessment of the environmental risk from the facility. The operator’s risk assessment is satisfactory. |  |  |
| Operating techniques |  | We have reviewed the techniques used by the operator and compared these with the relevant guidance notes. The operator has identified appropriate measures, as set out in the relevant technical guidance note for waste activities; ‘How to comply with your environmental permit’, Sector Guidance Note IPPC S5.06: Guidance for the Recovery and Disposal of Hazardous and Non-Hazardous Waste and the Fire prevention and mitigation plan guidance. We have also incorporated the relevant management systems into the permit to manage site discharges, dust, odour, pests, battery storage, noise and fire. |  |  |
| The permit conditions |  |  |  |  |
| Updating permit conditions during consolidation |  | We have updated previous permit conditions to those in the new generic permit template as part of permit consolidation. The new conditions have the same meaning as those in the previous permit(s). The operator has agreed that the new conditions are acceptable. |  |  |
| Waste types |  | We have specified the permitted waste types, descriptions and quantities, which can be accepted at the regulated facility. We are satisfied that the operator can accept these wastes for the following reasons: - the proposed wastes are suitable for the nature of the permitted activity - appropriate measures for acceptance, storage and processing have been included in the permit and identified by the operator - the appropriate measures are in place to prevent emissions. We have excluded the following wastes: - consisting solely or mainly of dusts, powders or loose fibres - sludges - odorous or odour producing - hazardous wastes. as the exclusions will ensure risk from the activity is contained and that measures used on site remain appropriate for the actual permitted activity. We made these decisions with respect to waste types in accordance with waste facility technical guidance note ‘How to comply with your environmental permit’ and after full assessment of the risk controls used on site. |  |  |
| Incorporating the application |  | We have specified that the applicant must operate the permit in accordance with descriptions in the application, including all additional information received as part of the determination process. These descriptions are specified in the Operating Techniques table in the permit. We have incorporated relevant technical guidance documents such as Technical Guidance Document: ‘How to comply with your environmental permit’, Sector Guidance Note IPPC S5.06: Guidance for the Recovery and Disposal of Hazardous and Non-Hazardous Waste and Fire prevention and mitigation plan guidance - Waste Management. We have also incorporated relevant process documents, provided by the operator to ensure that the site is operated in a way that does not cause pollution. |  |  |
| Operator Competence |  |  |  |  |
| Environment management system |  | There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. |  |  |

# Annex 2: Consultation and web publicising responses

Summary of responses to consultation and web publication and the way in which we have taken these into account in the determination process. Newspaper advertising is only carried out for certain application types, in line with our guidance.

Response received from

Public Health Wales

Brief summary of issues raised

- Waste acceptance and rejection procedures should be followed to control odour and vermin hazards. - The fire service should be consulted on the fire plan, and the management of batteries should be sufficiently protective against fires. - Noise controls should be adequate, and any complaints are investigated, with remedial actions taken where necessary. - No formal site condition report has been submitted. The Regulator should ensure ground contamination is considered in future surrender of the permit.

Summary of actions taken or show how this has been covered

- The operator has provided waste acceptance and rejection procedures, an Odour Management Plan and a Pests Management Plan which have been reviewed against relevant guidance and incorporated in the permit via Table S1.2 where relevant to ensure compliance. The permit also includes specific conditions in the permit regarding Odour and Pests. - The Fire and Rescue Service have been consulted on the operators Fire Prevention and Mitigation Plan (FPMP)- see below. The FPMP has been assessed against our guidance. The final FPMP was received 11/07/24 which meets our guidance. The final FPMP (as well as our guidance) has been incorporated in the permit via Table S1.2 to ensure compliance. The permit also includes a specific condition in the permit regarding Fire. - Based on the permitted activity, and the location of the activity a dedicated Noise Management Plan was not required. Noise management was included with the Risk assessment which has been incorporated in the permit via Table S1.2 to ensure compliance. The operator has a complaints procedure which would be implemented when required. The permit also includes a specific condition in the permit regarding Noise. - The documents submitted as the sites “Site Condition Report” has been revied, and the operator has been informed of their responsibilities at permit surrender.

Response received from

Fire and Rescue Service

Brief summary of issues raised

Confirmation is required that the separation distances of baled materials meet GN16.

Summary of actions taken or show how this has been covered

The operator has been served 3 Schedule 5 Notices to ensure that their FPMP (and the layout of the site and separation distances) complies with our guidance. The final FPMP was received 11/07/24 which meets our guidance. The final FPMP (as well as our guidance) has been incorporated in the permit via Table S1.2 to ensure compliance.

Response received from

Planning- Rhondda Cynon Taff County Borough Council

Brief summary of issues raised

No issues raised.

Summary of actions taken or show how this has been covered

No actions required.