# Living Seawalls

#### North Wales Rivers Trust

# Preliminary Water Framework Directive Assessment

# 1) Living Seawalls WFD assessment

## 1.1 Introduction

The Menai Strait is a unique strip of tidal water, separating the mainland in Gwynedd with the island of Ynys Môn. Stretching for over 30km from the North in Trwyn Penmon to Abermenai Point. The Menai is also a heavily urbanised environment, where much of the shoreline has been modified for human activity. Thirteen rivers also enter the Menai, nine from the mainland and four from Ynys Môn. Although the Menai is not a river, the surrounding land and close proximity of agriculture and urbanisation, means it faces many challenges similar to the adjoining rivers within the catchment, especially in habitat loss and species decline. Living Sea Walls (LSWs) are an innovative way to utilise cutting-edge technology and apply it to degraded man-made marine habitats. LSWs are 3-D printed panels which help diversify the structural habitat of degraded man-made coastal structures. These urbanised shorelines are detrimental to marine life, providing a non-porous mono-textured environment and by diversifying the structural environment, it will boost marine biodiversity and benefit the Menai. This document provides the water directive compliance assessment for the proposed living seawalls along the Menai Strait

## 1.2 Approach to Assessment

This assessment analyses the potential of the proposed installation of Living Seawalls on the Menai Strait.

# 2) Screening

See the screening method below outlining the project and location.

## 2.1 Project Details

| Project Details |  |
| --- | --- |
| Applicant name | North Wales Rivers Trust (NWRT) |
| Application reference number (where applicable) | CML2423 |
| Name of activity | Living Seawalls |
| Brief description of activity | The proposed installation of Living Seawalls along the Menai Strait to boost habitat. The Living Seawalls panels will act as a pilot measuring the effectiveness of improving habitat under a variety of conditions. |
| Location of activity (central point XY coordinates or national grid reference) | Gwynedd - Site 1: Bangor Sea Defence Wall Grid Reference: SH 58680 72857 Latitude , Longitude (decimal): 53.233891 , -4.1187712 - Site 2: Shell Car park Grid Reference: SH 48033 63317 Latitude , Longitude (decimal): 53.145258 , -4.2736140 - Site 3: Garth Pier Car Park Grid Reference:SH 58412 73200 Latitude , Longitude (decimal):53.236893 , -4.1229458 - Site 4: Modern Seawall by garth pier car park Grid Reference: SH 58453 73234 Latitude , Longitude (decimal): 53.237214 , -4.1223450 Ynys Mon - Site 1: Beaumaris outdoor swimming pool Grid Reference: SH 60979 76299 Latitude , Longitude (decimal): 53.265416 , -4.0858626 - Site 2: Seawall Penmon Grid Reference:SH 62665 79483 Latitude , Longitude (decimal): 53.294452 , -4.0619802 - Site 3: Moel Y Don Grid Reference: SH 51913 67770 Latitude , Longitude (decimal): 53.186349 , -4.2177254 - Site 4: Near Ramp in Y Borth Grid Reference: SH 55848 71827 Latitude , Longitude (decimal): 53.223871 , -4.1607124 - Site 5: Ramp in Y Borth Grid Reference: SH 55840 71801 Latitude , Longitude (decimal): 53.223639 , -4.1608089 |
| Footprint of activity (ha) of the living seawalls on the Menai Strait | 0.012 |
| Timings of activity (including start and finish dates) | Installation to begin in Q2 2024 and end by Q3 2024. |
| Extent of activity (for example size, scale frequency, expected volumes of output or discharge) | Installation of living seawall panels at 9 locations. A total of around 154 panels are to be installed. |
| Use or release of chemicals (state which ones) | For the installation of the concrete panels, an epoxy may be used: Hiltri Hiltre 5000 or Chemset 101 |

## 2.2 Baseline Information

| Water body1 | Description, notes or more information |
| --- | --- |
| WFD water body name | Menai Strait |
| Water body ID | GB681010120000 |
| River basin district name | Coastal - C16 |
| Water body type (estuarine or coastal) | Coastal |
| Water body total area (ha) | NA |
| Overall water body status (2015) | Moderate |
| Ecological status | Moderate |
| Chemical status | Does not require an assessment |
| Target water body status and deadline | Good by 2027 |
| Hydromorphology status of water body | Heavily Modified |
| Heavily modified water body and for what use | Heavily Modified, ShellFisheries |
| Higher sensitivity habitats present | Yes |
| Lower sensitivity habitats present | Yes |
| Phytoplankton status | High |
| History of harmful algae | No |
| WFD protected areas within 2km | Bathing Water Directive, Natura 2000 (Habitats and/or Birds Directive), Shellfish Water Directive |

## 2.3 Protected Sites

The following WFD-protected areas have been identified within 2km of the scheme:

- Menai Strait and Conwy Bay SAC for mudflats and sandflats not covered by seawater at low tide,  reefs and sandbanks which are slightly covered by seawater all the time.

- Shellfish Water Directive

- Ynys Seiriol SPA

## 2.4 Risk Screening

Activities that may result in an impact on WFD quality elements have the potential to take place during the installation of the seawalls. The table below details the expected activities and screens them for inclusion in the next stage of the assessment.

| Proposed Activity | Screen In/Out | Justification |
| --- | --- | --- |
| Construction (Temporary) activities |  |  |
| Works to install the living seawalls | In | Potential to temporarily disturb the ground whilst installing the living seawalls and the impact of drilling into modern seawalls |

Section 1: Hydromorphology

| Consider if your activity: | Yes | No | Hydromorphology risk issue(s) |
| --- | --- | --- | --- |
| Could impact on the hydromorphology (for example morphology or tidal patterns) of a water body at high status | Requires impact assessment | Impact assessment not required | No pathway for the activity to affect |
| Could significantly impact the hydromorphology of any water body | Requires impact assessment | Impact assessment not required | No pathway for the activity to affect |
| Is in a water body that is heavily modified for the same use as your activity | Requires impact assessment | Impact assessment not required | No pathway for the activity to affect |

Section 2: Biology

Habitats

| Higher sensitivity habitats 2 | Lower sensitivity habitats 3 |
| --- | --- |
| chalk reef | cobbles, gravel and shingle |
| clam, cockle and oyster beds | intertidal soft sediments like sand and mud |
| intertidal seagrass | rocky shore |
| maerl | subtidal boulder fields |
| mussel beds, including blue and horse mussel | subtidal rocky reef |
| polychaete reef | subtidal soft sediments like sand and mud |
| saltmarsh |  |
| subtidal kelp beds |  |
| subtidal seagrass |  |

2 Higher sensitivity habitats have a low resistance to, and recovery rate, from human pressures.

3 Lower sensitivity habitats have a medium to high resistance to, and recovery rate from, human pressures.

| Consider if the footprint4 of your activity is: | Yes | No | Biology habitats risk issue(s) |
| --- | --- | --- | --- |
| 0.5km2 or larger | Yes to one or more – requires impact assessment | No to all – impact assessment not required | No pathway for the activity to affect |
| 1% or more of the water body’s area |  |  | No pathway for the activity to affect |
| Within 500m of any higher sensitivity habitat |  |  | Yes, the living seawalls are in close proximity to a higher-sensitivity habitat |
| 1% or more of any lower sensitivity habitat |  |  | No pathway for the activity to affect |

4 Note that a footprint may also be a temperature or sediment plume. For dredging activity, a footprint is 1.5 times the dredge area.

Fish

| Consider if your activity: | Yes | No | Biology fish risk issue(s) |
| --- | --- | --- | --- |
| Is in an estuary and could affect fish in the estuary, outside the estuary but could delay or prevent fish entering it or could affect fish migrating through the estuary | Continue with questions | Go to next section | No pathway for the activity to affect |
| Could impact on normal fish behaviour like movement, migration or spawning (for example creating a physical barrier, noise, chemical change or a change in depth or flow) | Requires impact assessment | Impact assessment not required | No pathway for the activity to affect |
| Could cause entrainment or impingement of fish | Requires impact assessment | Impact assessment not required | No pathway for the activity to affect |

Section 3: Water quality

| Consider if your activity: | Yes | No | Water quality risk issue(s) |
| --- | --- | --- | --- |
| Could affect water clarity, temperature, salinity, oxygen levels, nutrients or microbial patterns continuously for longer than a spring neap tidal cycle (about 14 days) | Requires impact assessment | Impact assessment not required | No pathway for the activity to affect |
| Is in a water body with a phytoplankton status of moderate, poor or bad | Requires impact assessment | Impact assessment not required | No pathway for the activity to affect |
| Is in a water body with a history of harmful algae | Requires impact assessment | Impact assessment not required | No pathway for the activity to affect |

Consider if water quality is at risk from your activity through the use, release or disturbance of chemicals.

| If your activity uses or releases chemicals (for example through sediment disturbance or building works) consider if: | Yes | No | Water quality risk issue(s) |
| --- | --- | --- | --- |
| The chemicals are on the Environmental Quality Standards Directive (EQSD) list | Requires impact assessment | Impact assessment not required | Yes, one of the chemicals that may be used to install the living seawalls is on the EQSD list |
| It disturbs sediment with contaminants above Cefas Action Level 1 | Requires impact assessment | Impact assessment not required | No pathway for the activity to affect |

| If your activity has a mixing zone (like a discharge pipeline or outfall) consider if: | Yes | No | Water quality risk issue(s) |
| --- | --- | --- | --- |
| The chemicals released are on the Environmental Quality Standards Directive (EQSD) list | Requires impact assessment | Impact assessment not required | No pathway for the activity to affect |

Section 4: WFD protected areas

Consider if WFD protected areas are at risk from your activity. These include:

| - special areas of conservation (SAC) | - bathing waters |
| --- | --- |
| - Special Protection areas (SPA) | - nutrient sensitive areas |
| - shellfish waters |  |

| Consider if your activity is: | Yes | No | Protected areas risk issue(s) |
| --- | --- | --- | --- |
| Within 2km of any WFD protected area | Requires impact assessment | Impact assessment not required | Yes- multiple sites are within protected areas including SAC, SPA and shellfish waters |

Section 5: Invasive non-native species (INNS)

| Consider if your activity could: | Yes | No | INNS risk issue(s) |
| --- | --- | --- | --- |
| Introduce or spread INNS | Requires impact assessment | Impact assessment not required | The living seawalls provide habitats that may increase the spread of INNS |

Summary

| Receptor | Potential risk to receptor? | Note the risk issue(s) for impact assessment |
| --- | --- | --- |
| Hydromorphology | No | No pathway for the activity to affect |
| Biology: habitats | No | No pathway for the activity to affect |
| Biology: fish | No | No pathway for the activity to affect |
| Water quality | Yes | A chemical used to install the living seawalls is on the EQSD list |
| Protected areas | Yes | In close proximity to protected areas |
| Invasive non-native species | Yes | The Living Seawalls have the potential to encourage INNS |

# 3. Detailed Compliance Assessment

## 3.1 Embedded Mitigation

| Proposed Activity | Quality element and description of potential impact(s) | Mitigation Measure(s) | Assessment Outcome |
| --- | --- | --- | --- |
| Construction (Temporary) activities |  |  |  |
| Works to install the living seawalls | Element: proximity to protected Areas Potential Impact: Potential indirect impact on the protected areas during construction | The selected sites of the living seawalls are to take place on pre-existing concrete structures to reduce the impact on the protected sites including the mudflats and sandflats. No pathway for the activity to affect the SAC, SPA and shellfish waters | No pathway for the activity to affect the SAC, SPA and shellfish waters from the installation of the living seawalls as they pose minimal risk to the protected areas. |
|  | Element: Invasive Non-Native Species Potential Impact: Potential for construction works to introduce or spread INNS. | All equipment and footwear will be cleaned thoroughly before entering the site with a suitable disinfectant. In addition, all equipment and footwear will be thoroughly cleaned and disinfected when leaving the site. No works will be undertaken within 7m of any INNS, where this is unavoidable INNS management as per the EAP will be required | Provided the mitigation measures listed are implemented, these activities pose a minimal risk to the quality elements. This activity is determined to not result in a decline in any WFD quality elements or prevent them from attaining Good status or potential in the future. |
|  | Element: Specific Pollutants and/or Priority Substances. Potential impact(s): Potential for construction works to disturb contaminated material or introduce pollutants as a result of spillage. | Contractor to implement best practice for working near to water (e.g. GPP5) to reduce the risk or impact of accidental spillage. For example, drip trays/bunds and providing spill kits. | Provided the mitigation measures listed are implemented, these activities pose a minimal risk to the quality elements. This activity is determined to not result in a decline in any WFD quality elements or prevent them from attaining Good status or potential in the future. |
| Permanent Activities |  |  |  |
| Living Seawalls | Element: Invasive Non-Native Species Potential Impact: Potential for the living seawalls to promote INNS | Continuous monitoring should be adhered to and no works will be undertaken within 7m of any INNS, where this is unavoidable INNS management as per the EAP will be required. | Provided the mitigation measures listed are implemented, these activities pose a minimal risk to the quality elements. This activity is determined to not result in a decline in any WFD quality elements or prevent them from attaining Good status or potential in the future. |

## 3.2 In combination impacts

The accumulated impact of the various activities that form the will be minimal, provided the proposed mitigation measures to avoid or reduce impacts are included in the design and construction of the scheme.