Dolobran Hall Poultry Unit

Decision Document

# Bespoke permit

## The Application number is: PAN-002776

## The Applicant / Operator is: Mr Robert G Evans, Mrs Edith M Evans, Mr Nicholas R W Evans and Mr Charlie J Evans

## The Installation is located at: Dolobran Hall Poultry Unit, Dolobran Hall, Pontrobert, Meifod, Powys, SY22 6HX

We have decided to grant the permit for Dolobran Hall Poultry Unit operated by Mr Robert G Evans, Mrs Edith M Evans, Mr Nicholas R W Evans and Mr Charlie J Evans.

The permit number is EPR/BB3390CR/A001

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

## Purpose of this document

This decision document:

explains how the application has been determined

provides a record of the decision-making process

shows how all relevant factors have been taken into account

justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

## Structure of this document

- Table of contents

- Key issues

- Annex 1 the consultation and web publicising responses

## Table of Contents

## Contents

Bespoke permit	2

The Application number is: PAN-002776	2

The Applicant / Operator is: Mr Robert G Evans, Mrs Edith M Evans, Mr Nicholas R W Evans and Mr Charlie J Evans	2

The Installation is located at: Dolobran Hall Poultry Unit, Dolobran Hall, Pontrobert, Meifod, Powys, SY22 6HX	2

Purpose of this document	2

Structure of this document	2

Table of Contents	3

Contents	3

Key issues of the decision	5

1	Our decision	5

2	How we reached our decision	5

2.1 Receipt of Application	5

2.2	 Consultation on the Application	6

2.2.1	Draft Permit Consultation	7

2.3	 Requests for Further Information	7

3	The Legal Framework	7

4	The Installation	8

4.1 Description of the Installation and related issues	8

4.2 The site and its protection	9

4.3 Operation of the Installation – general issues	10

5	Minimising the Installation’s environmental impact	13

5.1	 Assessment of Impact on Air Quality	14

5.2 Assessment of odour impact	14

5.3 Assessment of impact to surface and ground water	15

5.4	 Emissions to sewer	16

5.5 Fugitive emissions	16

5.6	 Noise Assessment	16

5.7	 Impact on Habitats sites, SSSIs, non-statutory conservation sites etc	17

HRA consultation:	19

6  Setting ELVs and other Permit conditions	19

6.1	Translating BAT into Permit conditions	19

6.2	Monitoring	19

6.3	 Reporting	20

ANNEX 1:	Consultation Reponses	21

A)	Advertising and Consultation on the Application	21

1)	Consultation Responses from Statutory and Non-Statutory Bodies	21

2)	Consultation Responses from Members of the Public and Community Organisations	22

a)	Representations from Local MP, Assembly Member (AM), Councillors and Parish / Town / Community Councils	22

b)	Representations from Community and Other Organisations	22

c)	Representations from Individual Members of the Public	22

# Key issues of the decision

## Our decision

Based on the information currently available to us we are currently minded to grant a permit to the Applicant. This would, if issued, allow it to operate the Installation, subject to the conditions in the Permit.

We consider that, in reaching that decision, we have taken into account all relevant considerations and legal requirements and that the permit will ensure that a high level of protection is provided for the environment and human health.

This Application is to operate an installation which is subject principally to the Environmental Permitting Regulation 2016 (‘EPR’) and is subject to the requirements of the Industrial Emissions Directive (IED).

The Permit contains many conditions taken from our Environmental Permit template including, the relevant Annexes. We developed these conditions in consultation with industry, having regard to the legal requirements of the EPR and other relevant legislation. This document does not therefore include an explanation for these template conditions. Where they are included in the Permit, we have considered the Application and accepted the details are sufficient and satisfactory to make the template conditions appropriate.

## How we reached our decision

### 2.1 Receipt of Application

The Application was accepted as duly made on 20/06/2018. This means we considered it was in the correct form and contained sufficient information for us to begin our determination, but not that it necessarily contained all the information we would need to complete that determination.

The Applicant made no claim for commercial confidentiality. We have not received any information in relation to the Application that appears to be confidential in relation to any party.

### 2.2	 Consultation on the Application

We carried out consultation on the Application in accordance with the Environment Permitting Regulations 2016 (‘EPR’), the Industrial Emissions Directive (‘IED’), our statutory Public Participation Statement (‘PPS’) and our Regulatory Guidance.

We advertised the application by a notice placed on our website, which contained all the information required by the EPR/IED, including advising people where and when they could see a copy of the Application. The consultation started 21/06/18 and ended 20/07/18.

A copy of the Application and all other documents relevant to our determination (see below) are available for the public to view. Anyone wishing to see these documents could arrange for copies to be made.

We sent copies of the Application to the following bodies, which includes those with whom we have “Working Together Agreements”:

Powys County Council Planning Authority

Powys County Council Environmental Protection Department

Food Standards Agency

Health Protection Agency

Public Health Wales (PHW)

These are bodies whose expertise, democratic accountability and/or local knowledge make it appropriate for us to seek their views directly.

Further details along with a summary of consultation comments and our response to the representations we received can be found in Annex 1.  We have taken all relevant representations into consideration in reaching our determination.

## 2.2.1	Draft Permit Consultation

We carried out a consultation on our draft decision. This consultation will begin on 29/05/19 and end on 26/06/19.

### 2.3	 Requests for Further Information

Further information was requested by way of a Schedule 5 Notice requiring the applicant to submit a revised site layout plan and technical standards for the air cleaning systems. The Schedule 5 Notice was sent on 17/04/19 with a response date of 03/05/19. The Applicants response to the Schedule 5 Notice was provided on 23/04/19, 24/04/19 and 08/05/19. The additional information supplied satisfied the requirements of the Schedule 5 notice.

A copy of the information notice and e-mails requesting further information were placed on our public register as were the responses when received.

## The Legal Framework

The Permit will be granted, under Regulation 13 of the EPR.  The Environmental Permitting regime is a legal vehicle which delivers most of the relevant legal requirements for activities falling within its scope.  In particular, the regulated facility is:

an installation as described by the IED;

subject to aspects of the Well-Being of Future Generations (Wales) Act 2015 and the Environment (Wales) Act 2016 which also have to be addressed.

We address the legal requirements directly where relevant in the body of this document. NRW is satisfied that this decision is consistent with its general purpose of pursuing the sustainable management of natural resources (SMNR) in relation to Wales, and applying the principles of SMNR. In particular, NRW acknowledges that it is a principle of sustainable management to take action to prevent significant damage to ecosystems. We consider that, in granting the Permit a high level of protection will be delivered for the environment and human health through the operation of the Installation in accordance with the permit conditions.

## 4	The Installation

### 4.1 Description of the Installation and related issues

#### 4.1.1	The permitted activities

The Installation is subject to the EPR because it carries out an activity listed in Schedule 1 Part 2 of the EPR:

- Section 6.9 Part A(1)(a)(i) Rearing poultry in an installation with more than 40,000 places.

An installation may also comprise “directly associated activities”, which at this Installation includes.

- Dirty water tanks

- Generator and Fuel storage

- Exhaust air scrubbing units

Together, these listed and directly associated activities comprise the Installation.

#### 4.1.2	The Site

The free range poultry unit lies on a small parcel of level ground to the east of Pontrobert. Predominant land use is arable and grazing, the site is bounded by hedgerows. There are nearby residences within 400 metres of the poultry houses. The National Grid Reference is 312009, 312654.

The operator has provided a drainage plan which includes discharge points. A site boundary plan is included in the permit and the operator is required to carry out the permitted activities within the site boundary.

#### 4.1.3	What the Installation does

There is an existing poultry house at the site which provides accommodation for up to 32,000 free range egg laying chickens. Under the proposal, a new poultry house would be constructed adjacent to the existing house. This new poultry house would provide accommodation for a further 32,000 free range layer chickens (64,000 birds total). The existing and proposed house would both have ammonia air cleaning systems fitted.

### 4.2 The site and its protection

#### 4.2.1	Proposed site design: potentially polluting substances and prevention measures

There will be two fan ventilated poultry houses, this will provide a bird stocking density of 64,000. Feed is delivered in covered lorries and stored on site in vermin proof steel galvanised bins which are protected by guard rails. Litter is removed from the houses twice weekly via manure belts and at the end of each production cycle, then removed off site. Any litter that is exported from the installation has records kept of the quantities, destination and the date of transfer to separate farming businesses.

The houses are then washed and disinfected prior to the cycle beginning again. Underground storage tanks will be installed to catch all wash waters. Fallen stock are removed from the houses and stored in sealed containers awaiting collection from a licensed agent. Diesel fuel storage is in a bunded tank, the tank will be regularly inspected. The bund meets the requirements of the Water Resources (Control of Pollution) SSAFO (Silage, Slurry and Agricultural Fuel Oil) Regulations 2010. The dirty water tanks will be built to conform to specifications in sector guidance note (SGN) EPR 6.09 ‘How to comply with your environmental permit for intensive farming’.

Due to exceedances at nearby ecological receptors (Cors Ty Gwyn SSSI), the applicant has agreed to install ammonia scrubbers to reduce ammonia emissions by 70% on both the existing and proposed poultry sheds. Table S1.3 Pre-operational measures has been inserted into the operator’s permit. PO1 requires the operator to install ammonia scrubbing units as stated in email dated 08/05/19, ensuring there will be at least a 70% ammonia reduction rate. PO2 requires that at least 10 days before the operation of poultry house 2 the operator shall submit written confirmation to Natural Resources Wales that the necessary procedures are in place for the operation of the ammonia air scrubbing units and that staff have received the necessary training.

#### 4.2.2	Closure and decommissioning

Permit condition 1.1.1 requires the Operator to have a written management system in place which identifies and minimises risks of pollution including those arising from closure.

At the definitive cessation of activities, the Operator has to satisfy us that the necessary measures have been taken so that the site ceases to pose a risk to soil or groundwater, taking into account both the baseline conditions and the site’s current or approved future use. To do this, the Operator has to apply to us for surrender, which we will not grant unless and until we are satisfied that these requirements have been met.

The Applicant has completed a Site Condition Report when applying for the permit. The land had previously been used predominantly for general agricultural use, grass and grazing. We consider that the description provided is satisfactory. The decision was taken in accordance with our guidance on site condition reports – guidance and templates (H5).

### 4.3 Operation of the Installation – general issues

#### 4.3.1	Administrative issues

An organisation of individuals, Mr Robert G Evans, Mrs Edith M Evans, Mr Nicholas R W Evans and Mr Charlie J Evans will be the Operators of the Installation. We are satisfied that the Applicants will have control over the operation of the Installation; and that the Applicants will be able to operate the Installation so as to comply with the conditions included in the Permit.

#### 4.3.2	Relevant convictions

NRW’s COLINS Database has been checked to ensure that all relevant convictions have been declared.

No relevant convictions were found.

#### 4.3.3	Management

The Applicant has stated in the Application that they will implement an Environmental Management System (EMS) that will meet the requirements for an EMS in our “How to comply with your environmental permit guidance”. The Applicant submitted a summary of the EMS with their application. All written management systems will be subject to regular review by the Operator.

We are satisfied that appropriate management systems and management structures will be in place for this Installation, and that sufficient resources are available to the Operator to ensure compliance with all the Permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence.

#### 4.3.4	Accident management

In order to ensure that the management system proposed by the applicant sufficiently manages the residual risk of accidents, permit condition 1.1.1(a) requires the implementation of a written management system which addresses the pollution risks associated with, amongst other things, accidents.

The Operator has an Emergency Plan which will be subject to regular review. The plan includes contingencies for events such as fire, power failure, flood, disease and containment failure. We are satisfied with the Applicant’s Emergency Plan.

#### 4.3.5	Site security

The site does not have a secure boundary fence, all fuel stores, poultry houses and all store rooms are kept locked and secure, preventing any unauthorised access. Having considered the information submitted in the application, we are satisfied that appropriate infrastructure and procedures will be in place prior to start up to ensure that the site remains secure.

#### 4.3.6	Off-site conditions

Based on the information submitted in the application, we do not consider that it is necessary to impose offsite conditions.

#### 4.3.7	Operating techniques

Birds will be housed at point of lay and depopulated at the end of the birds egg laying cycle, this will be done on an ‘all-in all-out’ basis. Birds are fed a minimum of three diets with reducing levels of protein and phosphorus, as the bird weight increases with age. Feed is delivered from a UKASTA accredited feed mill and blown into bulk feed bins situated adjacent to the houses. From the feed bins, the feed is piped into the houses and distributed to the birds via a pan feeding system.

There are pop holes on the side of the houses, which provide daytime access to an outside ranging area. At the end of the cycle the site will be pressure washed, disinfected and dried out prior to the cycle beginning again. All wash waters will be contained in sealed underground tanks.

Nipple drinkers with drip-cups are used to minimise water wasted and to improve litter quality and moisture content. Staff will check equipment daily to fix to any faulty/leaking equipment. Fallen stock will be recorded daily and securely stored in vermin proof containers awaiting regular collection by a licenced agent.

Pre-operational requirement PO2 requires the operator to submit written confirmation to NRW that the necessary operating techniques are in place for the operation of the air scrubbing units and that staff have received the necessary training before the operation of the second poultry house.

#### 4.3.8	Energy efficiency

We are satisfied that the applicant will ensure that energy is used in the most efficient way possible. The poultry houses are insulated and have an adequate ventilation system to help regulate temperature and maintain a healthy environment inside the house.

#### 4.3.9	Avoidance, recovery or disposal of wastes produced by the activities

At depletion any remaining litter will be removed from the poultry sheds and removed off site via a sheeted trailer. The site will then be pressure washed and disinfected, all wash waters will be contained in sealed underground tanks.

Inorganic waste generated by the farm will mainly consist of paper, plastic and glass. Plastic waste will normally be in two forms, wrapping from bales of wood shavings and bottles from used disinfectants and detergents. The amount of plastic waste will be minimised through good managerial techniques. By good management of the litter quality, fewer bales of wood shaving will be needed, thus lowering the amount of plastic wrapping discarded. Large empty plastic bottles from detergents will be ‘recycled’ and used for foot dip containers or smaller rubbish bins for the storerooms.

Poultry carcasses are, under normal circumstances, collected and stored in sealed containers awaiting regular collection under the fallen stock scheme by a licensed collection agent. As a contingency plan or if an outbreak of high mortality should arise, carcasses will be placed in sealed containers and removed, as detailed in the emergency plan.

In the event of high mortality caused by disease, the operator will follow the guidance of the allocated veterinarian dealing with the outbreak. The mortality would be disposed of at an approved landfill site under the advice of that veterinarian, after consideration of weather conditions and geographical haulage parameters.

We are satisfied that waste from the Installation that cannot be recovered will be disposed of offsite using a method that minimises any impact on the environment.  Permit condition 1.4.1  will ensure that this position is maintained.

## 5	Minimising the Installation’s environmental impact

Regulated activities such as Intensive Farming can present different types of risk to the environment, these include odour, noise and vibration; accidents, fugitive emissions to air and water; as well as point source releases to air, discharges to ground or groundwater and generation of waste. All these factors are discussed in this and other sections of this document.

The following sections of this document explain how we have considered the critical issue of assessing the likely impact of emissions from the Installation on human health and the environment and what measures we are requiring to ensure a high level of protection.

### 5.1	 Assessment of Impact on Air Quality

The applicant has carried out a risk assessment identifying potential risks to human health including dust and ammonia. Operating procedures have been put in place to minimise the risks, in line with BAT procedures. It is considered that if the site is operated in line with these procedures, there is no significant risk to human health as a result of activities at the site.

### 5.2 Assessment of odour impact

The applicant has submitted detailed dispersion modelling of the impact of odour from the proposed facility.

H4 Odour Management guidance explains that the odour benchmarks are based on the 98th percentile of hourly average concentrations of odour modelled over a year at the site/installation boundary. The benchmarks are:

- 1.5 odour units for most offensive odours

- 3 odour units for moderately offensive odours

- 6 odour units for less offensive odours

Odours from poultry rearing are usually placed in the moderately offensive category. Therefore for their modelling the applicant has used the benchmark of 3.0 OUE/m3 to assess the potential impact of odour on nearby sensitive receptors. Modelling has been carried out based on emissions from two poultry houses and their ranging areas.

The applicant has described the following measures which will be in place to minimise odour emissions during house depopulation/de-littering:

- Litter will be placed carefully into trailers positioned close to house doors;

- Trailers containing spent litter will be sheeted before transporting off site;

- De littering will be avoided at weekends during the summer months;

- Clean out will be carried out as soon as possible following destocking;

- Poultry house disinfection and fumigation will be carried out by specialist contractors using DEFRA approved chemicals, observing correct dilution rates.

We have compared the measures proposed for the site to the BAT standards in EPR 6.09 and are satisfied that the techniques represent appropriate measures for the installation.

The modelling indicates that the emissions from the poultry houses, are unlikely to result in an exceedance of the odour benchmark level. Management of the onsite manure store will comply with the BAT standards in 6.09 to reduce odour emissions. Effluent channels and collection tanks will be maintained to avoid blockages and contaminated run-off.

Permit condition 3.3.1 requires that emissions from the activities are free from odour at levels likely to cause pollution outside the site.  We are satisfied that this condition will be sufficiently protective in conjunction with the measures described by the applicant for minimising odour production at the installation.

It is recognised that this modelling does only represent the expected odour concentrations for 98% of the time and that odours may be higher for the remaining 2% of the time. NRW is not able to ensure that odour impacts on nearby receptors are reduced to zero, but is determined to ensure that they are minimised.

### 5.3 Assessment of impact to surface and ground water

Based upon the information in the application we are satisfied that the appropriate measures will be in place to prevent pollution of ground and surface water. Clean, uncontaminated rainwater from roofs and yard areas will drain to a balancing pond and soakaways. Discharge from the balancing pond will be controlled by a hydro brake (2.4 l/s). The soakaways will be stone filled infiltration trenches and a piped system each side of the poultry sheds. All dirty water from wash down will be collected in underground sealed tanks, each with a capacity of 10,000l.

### 5.4	 Emissions to sewer

There are no emissions to sewer. When birds are removed from the poultry houses, the site will be pressure washed, disinfected and dried out prior to the next cycle. All wash waters will be contained in sealed underground tanks.

### 5.5 Fugitive emissions

The applicant has assessed the risk of fugitive emissions from the site. Potential sources of fugitive emissions include dust emissions to air and pests. To minimise dust emissions, feed is stored in purpose built covered feed silos located next to the layer sheds. No milling or mixing of feed takes place at the farm, all feed is delivered to the farm by lorry from a UKASTA accredited feed mill. Feed is piped from the silos to the sheds minimising dust emissions. Dust is controlled through the management of litter and air quality. Feed spillages will be cleared up promptly and a specialist contractor will be used to control pests.

Based upon the information in the application we are satisfied that the appropriate measures will be in place to prevent or where that is not practicable to minimise fugitive emissions and to prevent pollution from fugitive emissions.

### 5.6	 Noise Assessment

A risk assessment of the potential impact of noise from the site on nearby sensitive receptors has been carried out by the applicant. Potential sources of noise include vehicles travelling to and from site, ventilation fans, feed transferring from lorries to bins, bird catching and clean out operations.

There are sensitive receptors within 400 metres of the installation. The applicant has submitted a noise assessment for the installation, the assessment describes the measures and controls in place to minimise noise and includes twice daily inspections of the site.

The noise assessment states that roof mounted ventilation fans will be subject to regular, end of cycle maintenance by qualified electricians and that noisy roof mounted ventilation fans will be isolated and an electrician notified. Good maintenance and cleaning procedures will ensure additional noise from out of balance or worn roof mounted ventilation fans is unlikely to occur. Effective inspection and maintenance forms a key part of compliance with permit condition 1.1.1 on environmental management systems and condition 1.1.2 on associated record keeping.

We are satisfied that vibration is unlikely to be an issue at the installation.  The nature of the intensive farming operation means that there are no significant sources of vibration on site. Therefore vibration does not need to be included in the noise management plan. We have compared the measures proposed for the site to the BAT standards in EPR 6.09 and are satisfied that the techniques represent appropriate measures for the installation.

Permit condition 3.4.1 requires that emissions from the activities are free from noise at levels likely to cause pollution outside the site, as perceived by an officer of NRW.  We are satisfied that this condition will be sufficiently protective in conjunction with the measures described by the applicant for minimising odour production at the installation.

### 5.7	 Impact on Habitats sites, SSSIs, non-statutory conservation sites etc

The application is within the relevant distance criteria of a site of heritage, landscape or nature conservation, and/or protected species or habitat.

A full assessment of the application and its potential to affect the sites has been carried out as part of the permitting process.  Providing the operator installs the air scrubbing systems (with an ammonia reduction rate of at least 70%), we consider that the application will not affect the features of the sites.

The following sites have been considered:

SAC, SPA and Ramsar sites (within 5km):

None

SSSI (within 5km):

Cors Ty-Gwyn SSSI

Gwaun Efail Wig SSSI

Gweunydd Ceunant SSSI

Pen-Dugwm Woods SSSI

Cors Farchwel SSSI

Glascoed, Meifod SSSI

Afon Banwy Ger Mathrafal SSSI

Coed Ty-Mawr SSSI

Cors Cefn Llwyd SSSI

Ffridd Mathrafal Track Section SSSI

Gweunydd Ger Fronhaul SSSI

Mawnog Gwaunynog SSSI

Gwaun Wern-y-Wig SSSI

We also considered the potential impacts on nearby ancient woodlands, local wildlife sites, local nature reserves and national nature reserves (within 2km).

The applicant has carried out detailed modelling of the potential impact of dispersion and deposition of ammonia from the site. The submitted report concludes should the permit be granted at Dolobran Hall Poultry Unit, the ammonia emissions from the unit would increase, the modelling predicts that the maximum annual ammonia concentrations at the nearest receptors considered would be above the threshold for Cors Ty-Gwyn SSSI (critical level of 1 μg/m3).

Given these predicted exceedances, especially at Cors Ty-Gwyn SSSI, NRW has requested the operator to install an ammonia air cleaning system. The applicant submitted proposals (23/04/2019 and 08/05/2019) to install ammonia air scrubbing units on both poultry houses. The air scrubbing units will reduce ammonia emissions from the installation by approximately 70%. This 70% reduction for 64,000 birds will bring ammonia emissions from the installation below the current unfiltered ammonia emissions for 32,000 birds, providing improved environmental benefits.

NRW have placed a maximum annual limit on ammonia emissions (1536kg as NH3 per year). This limit was calculated using the standard ammonia emission factor for free range egg laying chickens (0.08 kg-NH3/bird place/year) and is less than that modelled for the existing shed with no mitigation in place.

## HRA consultation:

CRoW Appendix 4 has been completed and forwarded onto our internal Natural Resource Management (NRM) team for consultation and notification. Full details of the assessment carried out of the potential effect of ammonia emissions from the proposed site on any SSSI site detailed above are detailed in the forms. Following advice from conservation officers, the impact of ammonia emissions at Cors Ty-Gwyn SSSI was predicted to have an adverse impact on the site. Cors Ty-Gwyn SSSI contains a nitrogen sensitive feature for which the site is designated (lichens and bryophytes). NRW required the applicant to apply mitigation to reduce ammonia emissions from the poultry houses. It was also noted that there were no known European Protected Species that would be adversely impacted by this installation.

## 6  Setting ELVs and other Permit conditions

### 6.1	Translating BAT into Permit conditions

Article 14(3) of the Industrial Emissions Directive (IED) states that BAT conclusions shall be the reference for setting the permit conditions to installations covered by the Directive. As a result of the Commission Implementing Decision (EU) 2017/302 of 15 February 2017 establishing best available techniques (BAT) conclusions, under Directive 2010/75/EU of the European Parliament and of the Council, for the intensive rearing of poultry or pigs, the format of our Permit for the intensive farming sector has been updated. Appendix 1 of the Permit sets out generic conditions which apply to all sites. Appendix 2 sets out site specific conditions based on the activities being carried out.

### 6.2	Monitoring

Monitoring should be carried out for the parameters listed in Appendix 1, Schedule 3 of the permit using the methods and to the frequencies specified in Table S3.1 for laying hens. These monitoring requirements have been introduced in order to demonstrate compliance with the best available techniques (BAT) conclusions for the intensive rearing of poultry or pigs, as set out in the Commission Implementing Decision (EU) 2017/302 of 15 February 2017.

Table S3.2 in the permit, requires modelling from point source emissions to air. The applicant is required to monitoring ammonia emissions from the air scrubbing units on both poultry sheds four times a year (quarterly). This will ensure the air scrubbing units are working effectively and reducing ammonia emissions by 70% as stated in the report ‘IPPC Update statement ammonia’, submitted to NRW (23/04/2019). Table S3.5 sets the annual ammonia emission limit of 1536kg-NH3 (there are no quarterly ammonia emission limits).

If the applicant can supply sufficient evidence that the air scrubbing unit are working effectively, the modelling frequency could potentially be decreased to annual monitoring. However, this would first have to be agreed in writing by NRW.

### 6.3	 Reporting

We have specified reporting requirements in Appendix 2, Schedule 4 of the Permit to ensure compliance with permit conditions and to monitor the efficiency of farming activities at the site in line with BAT. We made this decision in accordance with EPR 6.09 “How to Comply with your Permit for Intensive Farming”.

# ANNEX 1:	Consultation Reponses

### A)	Advertising and Consultation on the Application

The Application has been advertised and consulted upon in accordance with Natural Resources Wales Public Participation Statement.  The way in which this has been carried out along with the results of our consultation and how we have taken consultation responses into account in reaching our draft decision is summarised in this Annex.  Copies of all consultation responses have been placed on Natural Resources Wales public register.

### 1)	Consultation Responses from Statutory and Non-Statutory Bodies

| Response Received from Public Health Wales |  |
| --- | --- |
| Brief summary of issues raised: | Summary of action taken / how this has been covered |
| - A site ammonia and odour management scheme should be implemented and maintained. - Best Available Techniques (BAT) and management controls should be applied. - Appropriate assessment and operational management of any dusts or bio-aerosols is recommended. - All on-site storage of liquids is accompanied by bunding in compliance with industry practice and guidance. - The applicant should seek external accreditation for the Environmental Management System (EMS) e.g. ISO14001 standard. | - See section 5.2 for odour assessment. Site operating techniques will be used to manage ammonia at the site. - Operating techniques and permit conditions cover concerns. - See section 5.5 - See section 4.2.1 - The Environmental Management System is addressed within this decision document, in particular at section 4.3.3. |

### 2)	Consultation Responses from Members of the Public and Community Organisations

Guidance on the interaction between planning and pollution control is given in PPS23 / Planning Policy Wales.  It says that the planning and pollution control systems are separate but complementary.  We are only able to take into account those issues which fall within regulatory scope of the Environmental Permitting Regulations 2016.

### a)	Representations from Local MP, Assembly Member (AM), Councillors and Parish / Town / Community Councils

| Response Received from |  |
| --- | --- |
| Brief summary of issues raised: | Summary of action taken / how this has been covered |
| None | N/A |

### b)	Representations from Community and Other Organisations

| Response Received from |  |
| --- | --- |
| Brief summary of issues raised: | Summary of action taken / how this has been covered |
| None | N/A |

### c)	Representations from Individual Members of the Public

| Response Received from |  |
| --- | --- |
| Brief summary of issues raised: | Summary of action taken / how this has been covered |
| None | N/A |