# Natural Resources Wales permitting decisions

# Bespoke permit

We have decided to grant the permit for Plot B, Former Airfields operated by Marshall Construction (West Yorkshire) Limited.

The application number is PAN-023381.

The permit number is EPR/DB3296CG.

This permit regulates the temporary discharge of abstracted groundwater as a result of de-watering in order to commence a commercial development consisting of proposed storage and distribution unit with ancillary offices, associated accesses, car parking, service yards, security gatehouse, electricity substation, pump house and landscaping.

The abstracted groundwater will be pumped into a series of 4 temporary lined holding lagoons allowing for settlement and subsequently pumped through a silt buster before being pumped into a Swale and then finally discharged into adjacent surface water drainage ditches leading to the River Dee.

The discharge will have a maximum flow rate of 5 litres a second from 4 discharge points, with a maximum rate of 20m3/day. Only one discharge will take place at any one time.

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

Purpose of this document

This decision document:

- explains how the application has been determined

- provides a record of the decision-making process

- shows how all relevant factors have been taken into account

- justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

Structure of this document

- Annex 1 the decision checklist

# Annex 1: decision checklist

This checklist should be read in conjunction with the Duly Making checklist.

| ### Aspect | ### Justification / Detail | Criteria met |
| --- | --- | --- |
|  |  | Yes |
| Consultation |  |  |  |
| Scope of consultation | The consultation requirements were identified and implemented. The decision was taken in accordance with RGN 6 High Profile Sites, our Public Participation Statement and our Working Together Agreements. | ü |
| ### Responses to web publicising | No response were received from web publicising. | ü |
| Operator |  |  |  |
| Control of the facility | We are satisfied that the applicant (now the operator) is the person who will have control over the operation of the facility after the grant of the permit. The decision was taken in accordance with EPR RGN 1 Understanding the meaning of operator. | ü |
| European Directives |  |  |  |
| Applicable directives | All applicable European directives have been considered in the determination of the application. | ü |
| WFD | WFD stage 2 scoping has been completed and the activity/project is considered as having no risk of causing deterioration or preventing any water body or WFD Protected Area from reaching its objectives and taking account of the advice from technical officers - is considered compliant with the WFD Regulations 2017. | ü |
| The site |  |  |  |
| Extent of the site of the facility | The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility including discharge points. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary. | ü |
| Habitat Regulation Assessment for a designated site under the EU Habitats Directive | HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site by virtue of the scale or location or nature of the project. | ü |
| Environmental Risk Assessment and operating techniques |  |  |  |
| Environmental risk | We have carried out a risk assessment on behalf of the operator. The operator considers this risk assessment is satisfactory. The assessment shows that, applying the conservative criteria in our guidance on Environmental Risk Assessment, all emissions may be categorised as environmentally insignificant. | ü |
| Operating techniques | We have reviewed the techniques used by the operator and compared these with the relevant guidance notes. The proposed techniques / emission levels for priorities for control are in line with the benchmark levels contained in the TGN “How to Comply with Your Environmental Permit” and we consider them to represent appropriate techniques for the facility. | ü |
| The permit conditions |  |  |  |
| Emission limits | We have decided that emission limits should be set for the parameters listed in the permit. It is considered that the numeric limits described below will prevent significant deterioration of receiving waters. We have imposed numeric limits because either a relevant environmental quality or operational standard requires this. • Suspended Solids (measured after drying at 105oC) – 50 mg/l • pH between 6 -9 The suspended solids limits has been set at the Water Quality Standard of 50mg/l in order to protect the receiving watercourse. | ü |
| Considerations of foul sewer | We agree with the operators justification for not connecting to foul sewer. | ü |
| Operator Competence |  |  |  |
| Environment Management System | There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. | ü |
| Sustainable Management of Natural Resources (SMNR) |  |  |
| Considerations of SMNR - Compliance with our General Purpose | We are satisfied that this decision is compatible with our general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources” | ü |
| Environment Wales Act 2016 |  |  |
| Biodiversity and resilience of ecosystems duty | Section 6 of the Environment Wales Act 2016 requires that we seek to maintain and enhance biodiversity in the exercise of our functions, and in so doing promote the resilience of ecosystems, in a manner that is consistent with the proper exercise of our functions. NRW is satisfied that in this case, we have taken into account and had due regard to this duty in so far as it is consistent with the function of determining an application for a stand alone water discharge activity permit under EPR2016. | ü |