Notice of variation and consolidation

with introductory note

The Environmental Permitting (England & Wales) Regulations 2016

Dŵr Cymru Cyfyngedig

Llyn Cefni Water Treatment Works

Off B5109

Bodffordd

Llangefni

Ynys Môn

LL77 7PQ

Variation number

PAN-026240

Permit number

CG0176901

Llyn Cefni Water Treatment Works

Permit Number CG0176901

Introductory note

### This introductory note does not form a part of the notice

The following notice gives notice of the variation and consolidation of an environmental permit.

This is an operator initiated variation to change the location of the effluent quality monitoring point from the current end of pipe monitoring point to a newly constructed monitoring point, enabling more representative sampling.

It also changes the names of the discharges back to as it was before the 2016 permit variation.

The status log of the permit sets out the permitting history, including any changes to the permit reference number.

| Status log of the permit |  |  |
| --- | --- | --- |
| Description | Date | Comments |
| Application received (variation and consolidation) | Duly made 04/04/2016 | Application to vary and consolidate an environmental permit that regulates the discharge of trade effluent. Consolidation with permits CG0177001 and CG0176801 |
| Permit determined CG0176901 | 10/08/2016 | Variation issued to Dŵr Cymru Cyfyngedig. |
| Operator-initiated variation PAN-026240 | 01/07/2024 | Variation to change the effluent monitoring location and rename discharge locations |
| Variation determined CG0176901 | 06/09/2024 | Varied and consolidated permit issued to Dŵr Cymru Cyfyngedig. |

End of introductory note

Notice of variation and consolidation

The Environmental Permitting (England and Wales) Regulations 2016

### The Natural Resources Body for Wales (“Natural Resources Wales”) in exercise of its powers under regulation 20 of the Environmental Permitting (England and Wales) Regulations 2016 varies and consolidates

### permit number

### CG0176901

### issued to

### Dŵr Cymru Cyfyngedig (“the operator”)

### whose registered office is

### Linea

### Fortran Road

### St Mellons

### Cardiff

### CF3 0LT

### company registration number 02366777

### to operate a water discharge activity at

### Llyn Cefni Water Treatment Works

### Off B5109

### Bodffordd

### Llangefni

### Ynys Môn

### LL77 7PQ

### to the extent set out in the schedules.

### The notice shall take effect from 01/10/2024

| ### Name | ### Date |
| --- | --- |
| ### Emma Jones | ### 02/09/2024 |

### Authorised on behalf of Natural Resources Wales.

Schedule 1

The following conditions were varied as a result of the application made by the operator:

Schedule 3, Tables S3.2 and S3.3

Schedule 2 – consolidated permit

Consolidated permit issued as a separate document.

Permit

The Environmental Permitting (England and Wales) Regulations 2016

Permit number

### CG0176901

### This is the consolidated permit referred to in the variation and consolidation notice for variation PAN-026240 authorising

### Dŵr Cymru Cyfyngedig (“the operator”),

### whose registered office is

### Linea

### Fortran Road

### St Mellons

### Cardiff

### CF3 0LT

### company registration number 02366777

### to operate a water discharge activity at

### Llyn Cefni Water Treatment Works

### Off B5109

### Bodffordd

### Llangefni

### Ynys Môn

### LL77 7PQ

### to the extent authorised by and subject to the conditions of this permit.

### The permit variation and consolidation shall take effect from 01/10/2024.

| ### Name | ### Date |
| --- | --- |
| ### Emma Jones | ### 02/09/2024 |

### Authorised on behalf of Natural Resources Wales.

# Conditions

Management

  - General management

The operator shall manage and operate the activities:

in accordance with a written management system that identifies and minimises risks of pollution so far as is reasonably practicable, including those risks arising from operations, maintenance, accidents, incidents, non-conformances and those drawn to the attention of the operator as a result of complaints; and

using sufficient competent persons and resources.

Records demonstrating compliance with condition 1.1.1 shall be maintained.

Any person having duties that are or may be affected by the matters set out in this permit shall have convenient access to a copy of the permit.

Operations

Permitted activities

The only activities authorised by the permit are the activities specified in schedule 1 table S1.1.

The site

The discharge activities shall take place at the discharge points marked on the site plan at schedule 7 to this permit, and as listed in table S3.2.

Emissions and monitoring

Emissions to water

The limits given in schedule 3 table S3.1 shall not be exceeded.

Emissions of substances not controlled by emission limits

The operator shall take appropriate measures to minimise so far as reasonably practicable the polluting effects of the emissions of substances in the discharge not controlled by emission limits (excluding odour).

Monitoring

The operator shall, unless otherwise agreed in writing by Natural Resources Wales, undertake the monitoring specified in the following tables in schedule 3 to this permit:

point source emissions specified in tables S3.1 and S3.3.

The operator shall maintain records of all monitoring required by this permit.

Monitoring equipment, techniques, personnel and organisations employed for the emissions monitoring programme and the environmental or other monitoring specified in condition 3.3.1 shall have either MCERTS certification or MCERTS accreditation (as appropriate), where available, unless otherwise agreed in writing by Natural Resources Wales.

Accessible monitoring points shall be provided and maintained to enable the emissions monitoring programme [and the environmental or other monitoring]   to be carried out at the monitoring point(s) specified in table [S3.3] of schedule 3 and shown marked on the site plan(s) in schedule 7.

The monitoring programme for the parameters subject to schedule 3B shall be:

pre-scheduled to cover a calendar year and the programme recorded before the start of a calendar year sample period; and

spot samples collected at approximately equal intervals during the year, including samples from different days of the week and different times.  Approximately 10% of samples should be outside the normal sampling window which is 9am-3pm, Monday to Friday.

After becoming aware, or following a notification that a sample has not been taken on the schedule 3B Monitoring Programme pre-scheduled date, or is lost, or a result for that sample cannot be reported, the operator shall record the details and reschedule the sample.

Information

Records

All records required to be made by schedule 3, 4 and 5 to this permit shall:

be legible;

be made as soon as reasonably practicable;

if amended, be amended in such a way that the original and any subsequent amendments remain legible, or are capable of retrieval; and

be retained, unless otherwise agreed in writing by Natural Resources Wales, for at least 6 years from the date when the records were made.

The operator shall maintain convenient access, in either electronic or hard copy, to the records, plan and management system required to be maintained by this permit.

Reporting

The operator shall send all reports and notifications required by the permit to Natural Resources Wales using the contact details supplied in writing by Natural Resources Wales.

Within the time period after the end of the reporting period specified in schedule 4 table S4.1 the operator shall, unless otherwise agreed in writing by Natural Resources Wales, submit reports of the monitoring and assessment carried out in accordance with the conditions of this permit, as follows:

in respect of the parameters and monitoring points specified in schedule 4 table S4.1;

for the reporting periods specified in schedule 4 table S4.1 and using the forms specified in schedule 4 table S4.2; and

giving the information from such results and assessments as may be required by the forms specified in those tables.

Notifications

Natural Resources Wales shall be notified as soon as reasonably practicable following detection, within the site of the regulated facility of:

any malfunction, breakdown or failure of equipment or techniques, accident, or emission of a substance not controlled by an emission limit which has caused, is causing or may cause significant pollution; and

any breach of a limit specified in schedule 3 table S3.1.

any other significant adverse environmental effects, which may have been caused by the activity, shall also be notified to Natural Resources Wales as soon as reasonably practicable following detection.

The information provided under condition 4.3.1 shall be supported by sending the information listed in schedule 5 to this permit within the time period specified in that schedule.

Where Natural Resources Wales has requested in writing that it shall be notified when the operator is to undertake monitoring and/or spot sampling specified in schedule [3B/3C], the operator shall inform Natural Resources Wales when the relevant monitoring and/or spot sampling is to take place. The operator shall provide this information to Natural Resources Wales at least 14 days before the date the monitoring is to be undertaken.

Natural Resources Wales shall be notified within 14 days of the occurrence of the following matters, except where such disclosure is prohibited by Stock Exchange rules:

Where the operator is a registered company:

any change in the operator’s trading name, registered name or registered office address; and

any steps taken with a view to the operator going into administration, entering into a company voluntary arrangement or being wound up.

Where the operator is a corporate body other than a registered company:

      - any change in the operator’s name or address; and

any steps taken with a view to the dissolution of the operator.

Where the operator proposes to make a change in the nature of the activity by increasing the concentration of, or the addition of, or allowing the introduction of, a substance to the activity to an extent that the operator considers could have a significant adverse environmental effect on the receiving waters, and the change is not permitted by emission limits specified within schedule 3 table S3.1 or the subject of an application for approval under the EP Regulations or under the terms of this permit:

Natural Resources Wales shall be notified in writing at least 14 days before the increase or addition or allowing the introduction; and

the notification shall contain a description of the proposed change.

Interpretation

In this permit the expressions listed in schedule 6 shall have the meaning given in that schedule.

In this permit references to reports and notifications mean written reports and notifications, except where reference is made to notification being made “as soon as reasonably practicable”, in which case it may be provided by telephone.

# Schedule 1 – Operations

| Table S1.1 Activities |  |  |
| --- | --- | --- |
| Activity reference | Description of activity | Limits of specified activity |
| A1 | Trade effluent from a water treatment works comprising dechlorinated analyser waste; supernatant from dewatered sludge and filter backwashing and run-to-waste activities | N/A |
| A2 | Trade effluent from a settlement lagoon serving a water treatment works comprising dechlorinated analyser waste; supernatant from dewatered sludge and filter backwashing; supernatant from settled WTW sludge, solutions and clarified water and borehole flushings | N/A |
| A3 |  |  |

# Schedule 2 – Waste types, raw materials and fuels

Schedule 2 not in use.

# Schedule 3 – Emissions and monitoring

| Table S3.1 Point Source emissions to water (other than sewer) – emission limits and monitoring requirements |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Effluent(s) and discharge point(s) | Parameter | Limit (including unit) | Reference Period | Limit of effective range | Monitoring frequency | Compliance Statistic |
| A1 Trade effluent via WTW Discharge X | Maximum daily discharge volume | 600 m3/day | Total daily volume | N/A | Continuous | Maximum |
|  | Maximum rate of discharge | 7 litres per second | Instantaneous (spot sample) | N/A | Continuous | Maximum |
|  | 15-minute instantaneous or averaged flow | No limit set. Record as l/s | 15 minutes | N/A | Continuous | N/A |
|  | Suspended solids (measured after drying at 105oC) | 100 mg/l | Instantaneous (spot sample) | N/A | As specified in schedule 3B | Maximum |
|  | Total available chlorine as Cl | 40 µg/l | Instantaneous (spot sample) | N/A | As specified in schedule 3B | Maximum |
|  | Iron | 5000 ug/l | Instantaneous (spot sample) | N/A | As specified in schedule 3B | Maximum |
|  | Aluminium | 10000 µg/l | Instantaneous (spot sample) | N/A | As specified in schedule 3B | Maximum |
| A2 Trade effluent via Lagoon Discharge Y and A3 Trade effluent via Lagoon Discharge Z | Maximum daily discharge volume | 22 m3/day | Total daily volume | N/A | N/A | Maximum |
|  | Maximum rate of discharge | 1 litres per second | Instantaneous (spot sample) | N/A | N/A | Maximum |
|  | Suspended solids (measured after drying at 105oC) | 100 mg/l | Instantaneous (spot sample) | N/A | As specified in schedule 3B | Maximum |
|  | Total available chlorine as Cl | 40 µg/l | Instantaneous (spot sample) | N/A | As specified in schedule 3B | Maximum |
|  | Iron | 5000 ug/l | Instantaneous (spot sample) | N/A | As specified in schedule 3B | Maximum |
|  | Aluminium | 10000 µg/l | Instantaneous (spot sample) | N/A | As specified in schedule 3B | Maximum |

| Table S3.2 Discharge points |  |  |  |
| --- | --- | --- | --- |
| Effluent Name | Discharge Point | Discharge point NGR | Receiving water/Environment |
| A1 Trade effluent | WTW Discharge X | SH 44600 77020 | Afon Cefni |
| A2 Trade effluent | Lagoon Discharge Y | SH 44600 77070 | Afon Cefni |
| A3 Trade effluent | Lagoon Discharge Z | SH 44610 76970 | Afon Cefni |

| Table S3.3 Monitoring points |  |  |  |
| --- | --- | --- | --- |
| Effluent(s) and discharge point(s) | Monitoring type | Monitoring point NGR | Monitoring point reference |
| A1 Trade effluent via WTW Discharge X | Final effluent sampling | SH 44533 77023 | WTW Discharge X Sample Point |
|  | MCERTS continuous flow monitoring | SH 44533 77023 | MCERTS Flow Monitoring Point |
| A2 Trade effluent via Lagoon Discharge Y | Final effluent sampling | SH 44600 77070 | Lagoon Discharge Y Sample Point |
| A3 Trade effluent via Lagoon Discharge Z | Final effluent sampling | SH 44610 76970 | Lagoon Discharge Z Sample Point |

Schedule 3B - OSM tier 2 sampling frequency

| Tier 2 Category | Parameter | `Normal frequency` of samples per year | Reduced Sampling frequency after 12 consecutive months of numeric permit compliance, samples per year or pro rata over the remainder of a year | On numeric limit failure return to normal frequency as soon as reasonably practicable, samples per 12 months | Out of hours samples |
| --- | --- | --- | --- | --- | --- |
| > 5m3/d (but not in tier 3) with numeric limits for ammoniacal nitrogen (expressed as N), nutrients, hazardous pollutants, UWWTR self-monitoring | Sanitary Non sanitary | 12 6 | 6 6 | 12 6 | For 12 samples 1 out of hours sample per annum, for 6, 1 every 2 years on average |
| >20 m3/d (but not in tier3) with numeric limits for one or more of biochemical oxygen demand or suspended solids or pH or temperature only. | Sanitary | 12 | 4 | 12 | For 12 samples 1 out of hours sample per annum, for 4, 1 every 3 years on average |

# Schedule 4 – Reporting

Parameters, for which reports shall be made, in accordance with conditions of this permit, are listed below.

| Table S4.1 Reporting of monitoring data |  |  |  |
| --- | --- | --- | --- |
| Parameter | Monitoring point reference | Reporting period | Period begins |
| Total daily volume | MCERTS Flow Monitoring Point | Annually | 1 January |
| 15-minute flow | MCERTS Flow Monitoring Point | Reports to be provided to Natural Resources Wales upon request | Upon request by Natural Resources Wales |
| Operator Self Monitoring – Suspended solids (measured after drying at 105°C), total available chlorine as Cl, total iron as Fe, total aluminium as Al | WTW Discharge X Sample Point, and Lagoon Discharge Y Sample Point, and Lagoon Discharge Z Sample Point | Quarterly | 1st of month |
| Operator Self Monitoring summary report | WTW Discharge X Sample Point, and Lagoon Discharge Y Sample Point, and Lagoon Discharge Z Sample Point | Annually | 1st January |

| Table S4.2 Reporting forms |  |
| --- | --- |
| Parameter | Reporting format |
| Total daily volume | WISKI electronic format specified by Natural Resources Wales |
| 15-minute flow | WISKI electronic format specified by Natural Resources Wales |
| Operator Self Monitoring – Suspended solids (measured after drying at 105°C), total available chlorine as Cl, total iron as Fe, total aluminium as Al | Quarterly - Electronic format specified by Natural Resources Wales |
| Operator Self Monitoring summary report | Annually - Summary report of compliance with the monitoring programme specified in Table 3.1 and Schedule 3B in a format specified by Natural Resources Wales |

# Schedule 5 – Notification

These pages outline the information that the operator must provide.

Units of measurement used in information supplied under Part A and B requirements shall be appropriate to the circumstances of the emission. Where appropriate, a comparison should be made of actual emissions and authorised emission limits.

If any information is considered commercially confidential, it should be separated from non-confidential information, supplied on a separate sheet and accompanied by an application for commercial confidentiality under the provisions of the EP Regulations.

## Part A

| Permit Number | CG0176901 |
| --- | --- |
| Name of operator | Dŵr Cymru Cyfyngedig |
| Location of Facility | Llyn Cefni Water Treatment Works, Off B5109, Bodffordd, Llangefni, Ynys Môn, LL77 7PQ |
| Time and date of the detection |  |

| (a) Notification requirements for any malfunction, breakdown or failure of equipment or techniques, accident, or emission of a substance not controlled by an emission limit which has caused, is causing or may cause significant pollution |  |
| --- | --- |
| To be notified within 7 days of detection unless otherwise agreed in writing by Natural Resources Wales |  |
| Date and time of the event |  |
| Reference or description of the location of the event |  |
| Description of where any release into the environment took place |  |
| Substances(s) potentially released/type or nature of sewage released |  |
| Best estimate of the quantity or rate of release of substances and/or duration of discharge |  |
| Best estimate of the environmental impact of the discharge |  |
| Measures taken, or intended to be taken, to stop any emission |  |
| Description of the failure or accident. |  |

| (b) Notification requirements for the breach of a limit specified in schedule 3 table S3.1 |  |
| --- | --- |
| The information specified below is to be notified to Natural Resources Wales as soon as reasonably practicable following detection. |  |
| Monitoring point reference/ source |  |
| Self monitoring regime (where relevant) |  |
| Type of failure |  |
| Date of sample/event |  |
| Parameter |  |
| Result and units |  |
| Limit and units |  |

| (c) Notification requirements for the detection of any significant adverse environmental effect |  |
| --- | --- |
| To be notified within 24 hours of detection |  |
| Description of where the effect on the environment was detected |  |
| Substances(s) detected |  |
| Concentrations of substances detected |  |
| Date of monitoring/sampling |  |

## Part B – to be submitted as soon as reasonably practicable unless otherwise agreed in writing by Natural Resources Wales

| Any more accurate information on the matters for notification under Part A. |  |
| --- | --- |
| Measures taken, or intended to be taken, to prevent a recurrence of the incident/breach/exceedance |  |
| Measures taken, or intended to be taken, to rectify, limit or prevent any pollution of the environment which has been or may be caused by the emission |  |

| Name* |  |
| --- | --- |
| Post |  |
| Signature |  |
| Date |  |

* authorised to sign on behalf of the operator

## Schedule 6 – Interpretation

“accident” means an accident that may result in pollution.

"annually" means once every year.

“application” means the application for this permit, together with any additional information supplied by the operator as part of the application and any response to a notice served under Schedule 5 to the EP Regulations.

“appropriate measures” for the purposes of the emission of substances not controlled by emission limits condition (condition 3.2.1) do not require the operator to undertake treatment to a level beyond that specified in schedule 1 table S1.1, or to carry out routine monitoring for substances not controlled by emission limits.

“emissions of substances not controlled by emission limits” means emissions of substances to air, water or land from the permitted activities, which are not controlled by an emission limit.

“EP Regulations” means The Environmental Permitting (England and Wales) Regulations SI 2016 No.1154 and words and expressions used in this permit which are also used in the Regulations have the same meanings as in those Regulations.

“MCERTS” means ‘Environment Agency’s’ Monitoring Certification Scheme.

“Natural Resources Wales” means the Natural Resources Body for Wales established by article 3 of the Natural Resources Body for Wales (Establishment) Order 2012. The Natural Resources Body for Wales (Functions) Order 2013 transferred the relevant functions of the Countryside Council for Wales, and functions of the Environment Agency and the Forestry Commission in Wales to the Natural Resources Body for Wales.

"quarter" means a calendar year quarter commencing on 1 January, 1 April, 1 July or 1 October.

“significant pollution” means a high level impact incident as indicated by NRW incident classification system.

“year” means calendar year ending 31 December.

# Schedule 7 – Site plan

© Crown Copyright and database right 2024. Ordnance Survey licence number 100019741.

END OF PERMIT