Modelling for Blackhills Nursery

Hydrology Data 	
Watercourse name:   Unnamed ditch located within the Bishopston Stream Catchment

Waterbody Data

Mean Flow = 149 m3/day,

Q95 = 27m3/day

Mid class data has been used as there is no ‘Physchem’ data available.
Mid Class data (Type 1, Good Class): 
mg/l
Ammonia: 	Mean = 0.14		Standard Deviation =  0.09
BOD: 		Mean =2.01		Standard Deviation = 1.2

Discharge Quality
Volume:  6.6m3/day	Mean = 3.29	Standard Deviation = 1.09
Effluent quality: 
@ 15.4 mg/l Ammonia:   Mean = 10	Standard Deviation = 4.1	          
@ 16.7 mg/l BOD:           Mean = 8.4	Standard Deviation = 3.5

Modelling Results

Ammonia

| MASS BALANCE CALCULATION: MONTE CARLO METHOD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version 2.5 |  |  |  |  |
| Calculations done on 25/09/2024 at 12.56 |  |  |  |  |
| Name of discharge | Blackhills Nursery |  |  |  |
| Name of river | Trib of Bishopston Brook |  |  |  |
| Name of determinand | Ammonia - mg/l Discharge Point |  |  |  |
| INPUT DATA |  |  | RESULTS |  |
| UPSTREAM RIVER DATA |  |  | RIVER DOWNSTREAM OF DISCHARGE |  |
| Mean flow | 149.00 |  | Mean quality | 0.50 |
| 95% exceedence flow | 27.00 |  | Standard deviation of quality | 0.31 |
| Mean quality | 0.14 |  | 90-percentile quality | 0.90 |
| Standard deviation of quality | 0.09 |  | 95-percentile quality | 1.12 |
| 90-percentile | 0.25 |  | 99-percentile quality | 1.58 |
| DISCHARGE DATA |  |  | DISCHARGE QUALITY |  |
| Mean flow | 3.29 |  | Mean quality | 10.18 |
| Standard deviation of flow | 1.09 |  | Standard deviation of quality | 4.07 |
| Mean quality | 10.00 |  | 95-percentile quality | 17.84 |
| Standard deviation of quality | 4.10 |  | 99-percentile quality | 22.82 |
| ... or 95-percentile | 17.70 |  | 99.5-percentile quality | 24.39 |
| % det Mean | 257 |  |  |  |
| 90%ile deterioration | 260 |  |  |  |

The data from this modelling shows that the proposed system has an impact of  260%  deterioration in the 90%ile ammonia levels against existing concentrations. The data modelled projected concentrations of 0.90 mg/l of Ammonia which is beyond the WFD ‘Good’ compliance class of 0.3 mg/l.

BOD

| MASS BALANCE CALCULATION: MONTE CARLO METHOD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version 2.5 |  |  |  |  |
| Calculations done on 25/09/2024 at 13.50 |  |  |  |  |
| Name of discharge | Blackhills Nursery |  |  |  |
| Name of river | Trib of Bishopston Brook |  |  |  |
| Name of determinand | BOD - Mg/l |  |  |  |
| INPUT DATA |  |  | RESULTS |  |
| UPSTREAM RIVER DATA |  |  | RIVER DOWNSTREAM OF DISCHARGE |  |
| Mean flow | 149.00 |  | Mean quality | 2.26 |
| 95% exceedence flow | 27.00 |  | Standard deviation of quality | 1.19 |
| Mean quality | 2.01 |  | 90-percentile quality | 3.67 |
| Standard deviation of quality | 1.20 |  | 95-percentile quality | 4.41 |
| 90-percentile | 3.50 |  | 99-percentile quality | 6.07 |
| DISCHARGE DATA |  |  | DISCHARGE QUALITY |  |
| Mean flow | 3.29 |  | Mean quality | 8.55 |
| Standard deviation of flow | 1.09 |  | Standard deviation of quality | 3.47 |
| Mean quality | 8.40 |  | 95-percentile quality | 15.10 |
| Standard deviation of quality | 3.50 |  | 99-percentile quality | 19.39 |
| ... or 95-percentile | 14.97 |  | 99.5-percentile quality | 20.74 |
| 90%ile deterioration | 4.85714 |  |  |  |
| % Deterioration mean | 12.4378 |  |  |  |

The data from this modelling shows that the existing system has a 4.86% deterioration in the 90%ile to the BOD levels within the watercourse against existing concentrations during the growing season. The deterioration of mean is 12.4%.

Backwards Calculations

Ammonia

| MASS BALANCE CALCULATION: MONTE CARLO METHOD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version 2.5 |  |  |  |  |
| Calculations done on 25/09/2024 at 14.17 |  |  |  |  |
| Name of discharge | Blackhills Nursery |  |  |  |
| Name of river | Trib of Bishopston Brook |  |  |  |
| Name of determinand | Ammonia - 10% Deterioration |  |  |  |
| INPUT DATA |  |  | RESULTS |  |
| UPSTREAM RIVER DATA |  |  | RIVER DOWNSTREAM OF DISCHARGE |  |
| Mean flow | 149.00 |  | Mean quality | 0.17 |
| 95% exceedence flow | 27.00 |  | Standard deviation of quality | 0.09 |
| Mean quality | 0.14 |  | 90-percentile quality | 0.28 |
| Standard deviation of quality | 0.09 |  | 95-percentile quality | 0.33 |
| 90-percentile | 0.25 |  | 99-percentile quality | 0.46 |
|  |  |  | Quality target (90-percentile) | 0.28 |
| DISCHARGE DATA |  |  |  |  |
| Mean flow | 3.29 |  | DISCHARGE QUALITY NEEDED |  |
| Standard deviation of flow | 1.09 |  | Mean quality | 0.82 |
| Mean quality | 10.00 |  | Standard deviation of quality | 0.33 |
| Standard deviation of quality | 4.10 |  | 95-percentile quality | 1.44 |
| ... or 95-percentile | 17.70 |  | 99-percentile quality | 1.84 |
|  |  |  | 99.5-percentile quality | 1.96 |
| DOWNSTREAM RIVER QUALITY TARGET |  |  |  |  |
| Quality target | 0.28 |  |  |  |
| Percentile | 90.00 |  |  |  |
| For a 10% deterioration a discharge quality of 1.44 mg/l is required |  |  |  |  |

A backwards calculation is required to tell us what limit is required on the permit for ammonia to ensure there is no greater than 10% deterioration in the watercourse. Based on a 10% deterioration a discharge quality target for Ammonia would be 1.44 mg/l.

Conclusion

To alter the WFD status of the watercourse from higher status to a lower status is not permissible. In this instance the proposed discharge would alter the Ammonia WFD status from ‘Good to ‘Poor’. Limits of 1.44 mg/l  would achieve a WFD class status of ‘Good’ but would severely impact the stability of the watercourse and remove any scope for future use. Consequently this application should be refused with the applicant exploring a groundwater option.

Bishopton Stream (upper)

Waterbody Data

Mean Flow = 3787 m3/day,

Q95 = 678 m3/day

Mid class data has been used as there is no ‘Physchem’ data available.
Mid Class data (Type 1, Good Class): 
mg/l
Ammonia: 	Mean = 0.14		Standard Deviation =  0.09

Modelling was undertaken for the unclassified stretch of Bishopton Stream. The data from this modelling shows that the proposed system has an impact of  4%  deterioration in the 90%ile ammonia levels against existing concentrations. The data modelled projected concentrations of 0.26 mg/l of Ammonia which is within the WFD ‘Good’ compliance class of 0.3 mg/l

Conclusion

Owing to the increased level of dilution the discharge would not impact the unclassified stretch of Bishopton Stream. This is based on the discharge directly entering the stream. The modelling does not account for concentrated volumes of discharge entering the stream after a dry spell.