# Natural Resources Wales permitting decisions

# Bespoke permit

We have decided to grant the permit for Land adjacent to Town House operated by Kidson Homes LTD.

The application number is PAN-025642

The permit number is EPR/DB3390CY

The discharge will be derived from a new development consisting of 1 X 2 bedroom and 8 x 4 bedroom properties. Each property is serviced with its own treatment plant that all join a communal sewage discharge pipe.

Each of the treatment systems perform to better than British Standards discharging a combined maximum daily volume of 6.3 cubic metre. The effluent will discharge 15m upstream of the River Teme into a unnamed tributary.

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

Purpose of this document

This decision document:

- explains how the application has been determined

- provides a record of the decision-making process

- shows how all relevant factors have been taken into account

- justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

Structure of this document

- Annex 1 the decision checklist

- Annex 2 the consultation, web publicising and newspaper advertising responses

# Annex 1: decision checklist

This checklist should be read in conjunction with the Duly Making checklist.

| ### Aspect | ### Justification / Detail | Criteria met |
| --- | --- | --- |
|  |  | Yes |
| Consultation |  |  |  |
| Scope of consultation | The consultation requirements were identified and implemented. The decision was taken in accordance with RGN 6 High Profile Sites, our Public Participation Statement and our Working Together Agreements. | ü |
| ### Responses to consultation, web publicising and newspaper advertising | The web publicising, consultation and newspaper advertising responses (Annex 2) were taken into account in the decision. The decision was taken in accordance with our guidance. | ü |
| Operator |  |  |  |
| Control of the facility | We are satisfied that the applicant (now the operator) is the person who will have control over the operation of the facility after the grant of the permit. The decision was taken in accordance with EPR RGN 1 Understanding the meaning of operator. | ü |
| The facility |  |  |  |
| The regulated facility | ###### The extent/nature of the facilities taking place at the site required clarification. The regulated facility is a standalone water discharge activity in accordance with EPR RGN 2 Understanding the meaning of regulated facility. See easement details here | ü |
| European Directives |  |  |  |
| Applicable directives | All applicable European directives have been considered in the determination of the application. | ü |
| WFD | WFD stage 2 scoping has been completed and the activity/project is considered as having no risk of causing deterioration or preventing any water body or WFD Protected Area from reaching its objectives and taking account of the advice from technical officers - is considered compliant with the WFD Regulations 2017. The river quality modelling undertaken, demonstrated that there is no discernible impact on the River Teme or its WFD classification status of ‘Good’. See WFD form here |  |
| The site |  |  |  |
| Extent of the site of the facility | The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility including discharge points. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary. | ü |
| Biodiversity, Heritage, Landscape and Nature Conservation and a designated site under the Wildlife and Countryside Act. | The application is within the relevant distance criteria of a site of heritage, landscape or nature conservation, and/or protected species or habitat . A full assessment of the application and its potential to affect the site and species has been carried out as part of the permitting process. We consider that the application will not affect the features of the site or the species See the assessment here | ü |
| Habitat Regulation Assessment for a designated site under the EU Habitats Directive | No FORM 1 completed: HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site - no conceivable impact on any Natura 2000 site, by virtue of the scale or location or nature of the project. Our screening report did not identify any SAC’s within screening distance. A concern was raised by NE that the River Clun and Severn Estuary SAC’s were hydrologically linked. Consequently it was felt by NE that the discharge point was functionally linked to these sites and could affect migratory fish. The nearest designated site, River Clun, was approximately 25km downstream. The quality of the discharge would not affect any features of the River Clun or the Severn Estuary SAC. Whilst it is acknowledge that the migratory species would travel to the headwaters of the River Teme these have been assessed as part of the SSSI Assessment and WFD process. |  |
| Environmental Risk Assessment and operating techniques |  |  |  |
| Environmental risk | We have carried out a risk assessment on behalf of the operator. The operator considers this risk assessment is satisfactory. |  |
| Operating techniques | We have reviewed the techniques used by the operator and compared these with the relevant guidance notes. The proposed techniques / emission levels for priorities for control are in line with the benchmark levels contained in the TGN 7.01and we consider them to represent appropriate techniques for the facility. | ü |
| The permit conditions |  |  |  |
| Emission limits | We have decided that emission limits should be set for the parameters listed in the permit. Ammoniacal nitrogen 9mg/l ATU-BOD as O2 12 mg/l Total phosphorus as P 5.7 Suspended solids (measured after drying at 105o C) 20 mg/l Visible oil or grease No significant trace present These limits have been based on the modelling of the effluent against the WFD classification of ‘high’. These are tight limits to ensure that the treatment systems are well maintained and discharges the same quality of effluent throughout their lifecycle. This is to ensure that the downstream water quality is maintained and there is no deterioration to the current WDF status of the watercourse. | ü |
| Considerations of foul sewer | We agree with the operators justification for not connecting to foul sewer. The nearest public sewer line is approximately 8km away. | ü |
| Operator Competence |  |  |  |
| Environment Management System | There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. | ü |
| Sustainable Management of Natural Resources (SMNR) |  |  |
| Considerations of SMNR - Compliance with our General Purpose | We are satisfied that this decision is compatible with our general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources” | ü |
| Environment Wales Act 2016 |  |  |
| Biodiversity and resilience of ecosystems duty | Section 6 of the Environment Wales Act 2016 requires that we seek to maintain and enhance biodiversity in the exercise of our functions, and in so doing promote the resilience of ecosystems, in a manner that is consistent with the proper exercise of our functions. NRW is satisfied that in this case, we have taken into account and had due regard to this duty in so far as it is consistent with the function of determining an application for a stand-alone water discharge activity permit under EPR2016. | ü |

# Annex 2: Consultation, web publicising and newspaper advertising responses

Response received from

Environment Agency

Brief summary of issues raised

Our Specialist staff have stated no objection to the issuing of this permit. The developer should agree to construct a headwall in the bank at the end of the pipe (where the pipe discharges treated effluent into the small watercourse) to reduce the risk of bank erosion.

Summary of actions taken or show how this has been covered

I think this has already been addressed but will confirm with the developer that a headwall be installed at the discharge point.

Response received from

Natural England

Brief summary of issues raised

I would have expected to see an assessment of the impacts on the river and its features. I would also expect to see a Habitats Regulations Assessment (HRA) as the River Teme is functionally linked to the River Clun SAC and Severn Estuary SAC/RAMSAR and so consideration is needed to look at the impacts to migratory species including but not limited to salmon, shad, and eel. I would also expect to see consideration for WQ impact as this is a year-round discharge also accounting for changes in flow. Is there any work being done to put this discharge on the sewerage network? Are there any restrictions on the discharge in periods of low flow?

Summary of actions taken or show how this has been covered

An assessment for the River Teme SSSI was completed. There was no need to consult formally on that document as it concluded that the proposed permission is not likely to damage any of the flora, fauna or geological or physiological features which are of special interest. No HRA was completed as the sites affected were beyond the risk screening distances. Migratory species have been assessed as part of the SSSI and WFD process, The nearest public sewer line is approximately 8km.