| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-026978 |
| Permit Reference Number: | EPR-AB3891CX |
| Permit Holder: | Mr Daniel James and Mrs Carys James |
| Site Address: | Middle Woodstock, Clarbeston Road, Woodstock, Pembrokeshire, SA63 4TG |
| Grid Reference: | Farm location: SN 02894 25883 Fields: Court Farm 26 SN0172825074 Court Farm 27 SN0186025150 Court Farm 28 SN0196025180 Court Farm 29 SN0205925298 Court Farm 30 SN0158025420 Court Farm 31 SN0171025450 Court Farm 32 SN0182025490 Court Farm 35 SN0149025410 Court Farm 36 SN0259625603 Court Farm 37 SN0259425350 Court Farm 41 SN0239125604 Court Farm 42 SN0223925433 Storage: SN0304426372 SN 03089 26382 (updated 29/11/24) Lagoon in field on other deployment- PAN-025644 SN0283225916 In field on other deployment- PAN-025644 SN0160025254 Court Farm 30 SN0177725351 Court Farm 26 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: wasteregulation.swwales@cyfoethnaturiolcymru.gov.uk Environment team: pembsdistrictteam@cyfoethnaturiolcymru.gov.uk NLO: Malcolm.Dines@cyfoethnaturiolcymru.gov.uk |
| Date due: | 07/10/2024 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating B- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | Welsh permit |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and deployment fee- Charging Scheme |
| Fee on MyNRW | Y | Transaction ref: 2136323 Date: 30/08/2024 ref: BS/300824/39 STEPSIDE AGRICULTUEPDEPSTEPS0121FP 30/08/24 1451500000001413659000 Amount: 1,929.00 |
| Location map | Y | Middle Woodstock (2) Maps |
| Benefit statement | Y | Middle Woodstock (2) Ag Benefit Previous land treatment C. Vaughan, Middle Woodstock (2) Consent Letter Details of land to be treated |
| Waste Analysis | Y | Waste Analysis |
| Soil Analysis | Y | Middle Woodstock (2) Soil Analysis |
| EIA Agri Regulations evidence | Y | Middle Woodstock (2) Supporting Evidence |
| Site Specific Risk Assessment | Y | Middle Woodstock (2) SSRA |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| - About the Permit |  |  |
| ## Has there been any pre-application discussions? | N |  |
| ## Has the permit number been provided? | Y | AB3891CX |
| ## Has the permit holder name been provided? | Y | Mr Daniel James and Mrs Carys James |
| ## What permit is being deployed under? | SR2010 No4 |  |
| - Contact details |  |  |
| ## Has contact information been provided? | Y | Robert Tucker |
| ## Has the technically competent manager details been provided? | Y | Gwennan Rees |
| ## Has the nominated competent person details been provided? | Y | Daniel James |
| - About the land you want to treat |  |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land | 55/461/0058 |
| If it is agricultural land; has the holding number been given? | Y | LPD1 application form: 55/461/0058 Doesn’t match fields 41 and 42, however clarification and additional numbers given on document “C. Vaughan, Middle Woodstock (2) Consent Letter”. See comments for NRW Mapping system results |
| ## Has all of parcels of land table been completed? | Y |  |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y | 49.90 ha |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | Y |  |
| ## Have details of the land owner or occupier been provided? | Y | MRH MGP and CB Vaughan of Middle Woodstock Doesn’t match fields 41 and 42, however clarification and additional numbers given on document “C. Vaughan, Middle Woodstock (2) Consent Letter”. See comments for NRW Mapping system results |
| ## Has written consent of all owners(s) or occupiers(s) been given? | Y | Contains information on fields, wastes, storage, tenancy, and CPH |
| - Previous land treatment |  |  |
| Has all of the previous land treatment table been completed? | Y | Cattle slurry only, no previous approved deployment on these fields |
| - Which risk band does the activity fall within? - SR2010No4 List A wastes (Lower risk) - Location risk - Low risk deployment - Medium risk (1) deployment - Medium risk (2) deployment - High risk deployment |  |  |
| ## Which risk band does the activity fall within? | High |  |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) or High risk deployment has a SSRA been provided? | Y | Middle Woodstock (2) SSRA |
| - Additional information on sensitive receptors |  |  |
| ## What type of risk assessment has been submitted? | site‐specific risk assessment |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | N |  |
| If no have details of the assessment evidence to support this conclusion been provided? | Y | Middle Woodstock (2) Supporting Evidence |
| - About the waste |  |  |
| Have all parts of the waste-types table been completed? - List of waste code (6 digit) - Description of waste (this must be an accurate description specific to this waste that refers to the List of Waste written description) - Physical form of waste/Is it stackable or non-stackable - Is the waste high in readily available nitrogen? - Producer of waste & permit number (if applicable) - Total amount being spread/used (tonnes) | Y | Physical form of waste given in ABS |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y | Known producer |
| - About the storage |  |  |
| Has all of waste storage table been completed? | Y | See storage section below |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y |  |
| 18. Supporting documents |  |  |
| Has checklist been completed? | Y |  |
| 20. Confidentiality and national security |  |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| 21. Declaration |  |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | n/a |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has the operator confirmed they will comply with the NRW pre-notification requirements. | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y |  |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no storage or spreading within 10 meters) (buffer zones needed) | Y | Watercourse south of fields 26-29 buffered Ponds and watercourse fields 41/42 buffered |
| groundwater source protection zones (permit condition: no storage or spreading in SPZ1) (high risk deployment if in SPZ2) | n/a |  |
| location of all storage sites for the waste to be spread | Y | See storage section below |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no storage or spreading within 50 meters) | Y | Spring between field 28/29 buffered Well to east of field 26 on high level OS map but not when zoomed in- RFI Buffer added 29/11/24 |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | Y | See above |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | Y | See above |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | Y | Wallis Moor SSSI Afon Cleddau Dwyreiniol / Eastern Cleddau River SSSI (Llys-y-Fran Reservoir section) Afonydd Cleddau / Cleddau Rivers SAC (Llys-y-Fran Reservoir section) |
| location of any public rights of way (shown on map) | Y |  |
| any buildings or houses within 250m (shown on map) | Y |  |
| Within Groundwater Safeguard Zone | n/a |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Habitats - Section 7 species - Scheduled Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Hedges - Ditches | Y | Ancient Woodlands shown on maps Habs/species outside of spreading areas |
| Flood risk | Y | Low, Medium and High Flood Risk from Surface Water & Small Watercourses. Watercourses in question have appropriate buffers |
| What is the slope of the fields? | 0-7 degrees (transparent)- all fields 7 -12 degrees fields 26-29 12 degrees plus (red)- no fields See section below for assessment |  |

| Waste Storage | Y N n/a | Comments |
| --- | --- | --- |
| Is waste storage occurring at the place where waste is being deployed? | Y | SN0304426372 Lagoon in field on other deployment. Not yet permitted- see application PAN-027280 SN0283225916 Nurse tank in field on recently issued deployment- PAN-025644 SN0160025254 Nurse tank Court Farm 30 SN0177725351 Field heap Court Farm 26 (Preseli sludge cake only) |
| Is only waste as specified in this deployment being stored in the proposed storage locations? i.e. slurry in lagoons Note: There may be multiple deployments using one storage location | Y |  |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in the same place one constructed within the last two years? or in different location? | Different |  |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in a field: - liable to flooding or becoming waterlogged, or - within 50 metres of a borehole, spring or well or - within 10 metres of surface water or a land drain - within 30 meters of a watercourse on steeply sloping land | N |  |
| If the waste is non-stackable is the waste stored within either a vessel, a covered building, or on an impermeable surface | Y | Lagoon and nurse tanks |
| If the waste is high RAN and it is being stored within 200 metres of a European Site (SAC/SPA), Ramsar Site or a Site of Special Scientific Interest (SSSI) is the waste being covered to prevent loss of ammonia? (condition 3.2.4 of the permit) | n/a | Not high RAN |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | N | Cray waste was out of date at time of application (May 2023) In date analysis supplied 29/11/24- all additions within acceptable range |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | n/a |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | Y | 19 09 02 wastes |
| - If yes; have they been tested for? | Y |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a | Not being spread in groundwater safeguard zone |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y | 3 fields have P index of 3, however offtake is greater than addition for all fields |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y |  |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | n/a |  |
| - Soil physical properties (e.g. organic matter and soil structure) | Y |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| Timing of application- if any of the wastes are high RAN, are they being spread OUTSIDE of the closed periods | n/a |  |
| Groundwater Safeguard Zone for nitrate- if any of the wastes are high RAN, are the application rates less than 50 m3/ha, with a minimum 3 week gap between applications | n/a |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | n/a |  |
| - Included the nutrient level in their environmental risk assessment | n/a |  |
| - Stated the source of information that they used to set the nutrient level. | n/a |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y |  |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need, as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If the cropping is any of the following, is the maximum nitrogen limit in line with that given in The Water Resources (Control of Agricultural Pollution) (Wales) Regulations 2021: - 0 kg/ha- Field beans, Peas - 120 kg/ha- Sugar beet - 150 kg/ha- Asparagus, Carrots, Forage maize. Radish, Spring barley, Swedes - 180 kg/ha- Spring-sown wheat, Winter barley - 220 kg/ha- Autumn or early winter sown wheat | n/a |  |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | Y |  |
| - Is it only for operational purposes? | Y |  |
| - Have individual application rates for each waste been given? | Y |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | Y |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients given by the applicant match NRW calculations? | Y |  |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y | Cattle slurry only |
| - How much material was spread? | Y |  |
| - At what application rate material was spread? | Y |  |
| - The nutrients supplied by the material? | Y |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | Y |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | Y |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | Y |  |
| - Pathways | Y |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | Y |  |
| - Measures that will minimise or mitigate the impact will be put in place | Y |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | Y |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | N |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | Y |
| No FORM 1 completed: HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site. No conceivable impact on the Natura 2000 site, by virtue of the scale or location or nature of the project. Afonydd Cleddau / Cleddau Rivers SAC- Immediate sections 1023 (Llys-y-Fran Reservoir) and 1022 (downstream of Llys-y-Fran Reservoir) SAC is south east of deployment fields 26-29, and south of fields 36/37 and 41/42 and is approx. 325 meters away at its closest point. A watercourse flows along the southern end of the fields 26-29 to the SAC- spreading is not permitted within 10 meters of a watercourse and is not being proposed to be spread within 50 meters of the watercourse. The SAC is downslope of fields 36/37 and 41/42 approx. 330 meters distance. Waste is not proposed to be spread onto the SAC or its immediate borders. The nearest spreading activity is 50 meters from the watercourse that leads to the SAC, and is approx. 325 meters from the nearest SAC border. Runoff risk of the wastes given the slope etc. has been assessed and is deemed to be low. |  |

Comments

Consultation responses: Comments indicate over application of waste on fields 35, 36, 41 for several wastes due to P index of 3. Calculations show P offtake is greater than P application on all fields.

MyMap/location and slope assessments: MyMap/location assessment See location section above Slope assessment Field number: fields 26-29 Slope of the land: 7 -12 degrees Soil type: Medium (loamy) Wastes types applied (solid, liquid, sludge etc.): 9 x liquid sludge, 1 x sludge cake Cropping type: grass silage Method of application: liquid sludge spread via dribble bar, sludge cake rear discharge muck spreaders. Weather conditions before during and after spreading: as per permit and COGAP Other risk factors not mentioned above: none Conclusions (is run off risk high?):No

CPH details: LPD1 application form: 55/461/0058 NRW Mapping: - Court Farm 26, 27, 28, 29, 30, 31, 32, 35- 55/461/0058- MRH & MGP & CB Vaughan- MIDDLE WOODSTOCK - Court Farm 36 & 37- None - Court Farm 41 and 42- 55/492/0006- TM and MS MORRIS- TYRIET Clarification and additional numbers given on document “C. Vaughan, Middle Woodstock (2) Consent Letter”.

Protection of habitats and ecosystems See document Middle Woodstock (2) Supporting Evidence

Agricultural benefit calculations: Bolton Hill appears to have low amounts of aluminium and Iron, however these testing was carried out on Sample as Received Basis rather than Dry Matter. Review of additions are in line with other wastes.

Request for information: RFI sent 28/11/2024

Other information to note: None

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date | 03/12/24 |
| ## Effective date | 03/12/24 |
| ## Permitting Officer Date | Louise Bailey 03/12/24 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 03/12/2024 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 650 |
| Total | 710 |