# Natural Resources Wales permitting decisions

# Bespoke permit

We have decided to grant the permit for the package treatment plant serving the 5 properties of: The Shippon, The Hayloft, The Grain Store, The Granary and the Cart House, Lower Farm Road operated by Lower Farm Barns Ltd.

The application number is PAN-026852.

The permit number is EPR/DB3394HH.

This permit regulates a new discharge of domestic sewage from a package treatment plant serving 5 residential properties. The total daily volume is 3.5 m3/day. The discharge passes through seasonally dry ditch that reaches the River Dee. As the discharge is to a seasonally dry ditch it is not eligible for an exemption. A condition will be added to the permit requiring the applicant to install a perforated pipe.

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

Purpose of this document

This decision document:

- explains how the application has been determined

- provides a record of the decision-making process

- shows how all relevant factors have been taken into account

- justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

Structure of this document

- Annex 1 the decision checklist

# Annex 1: decision checklist

This checklist should be read in conjunction with the Duly Making checklist.

| ### Aspect | ### Justification / Detail | Criteria met |
| --- | --- | --- |
|  |  | Yes |
| Consultation |  |  |  |
| Scope of consultation | The consultation requirements were identified and implemented. The decision was taken in accordance with RGN 6 High Profile Sites, our Public Participation Statement and our Working Together Agreements. | ü |
| ### Responses to web publicising | The proprosal was advertised for 20 working days on out website, according to our guidance from 8th October – 5 November. No responses were received from web publilcising. | ü |
| Operator |  |  |  |
| Control of the facility | We are satisfied that the applicant (now the operator) is the person who will have control over the operation of the facility after the grant of the permit. The decision was taken in accordance with EPR RGN 1 Understanding the meaning of operator. | ü |
| European Directives |  |  |  |
| Applicable directives | All applicable European directives have been considered in the determination of the application. | ü |
| WFD | The discharge activity screens out from needing further assessment under WFD as it is not in a water body at high status or high status for morphology, and it is listed in Annex D as a low risk activity plus there is no in combination impact. |  |
| The site |  |  |  |
| Extent of the site of the facility | The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility including discharge points. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary. | ü |
| Habitat Regulation Assessment for a designated site under the EU Habitats Directive | Appropriate assessment: In light of the conclusions of an appropriate assessment, and taking account of the advice received from protected sites advisors, it has been established that the project will not adversely affect the integrity of any Natura 2000/Ramsar site, taking into account any conditions or restrictions as applicable, either alone or in-combination with other plans and projects. (As documented in section 4 of OGN 200 form 1, and section 5 if applicable). | ü |
| Environmental Risk Assessment and operating techniques |  |  |  |
| Environmental risk | We have carried out a risk assessment on behalf of the operator. The operator considers this risk assessment is satisfactory. The assessment shows that, applying the conservative criteria in our guidance on Environmental Risk Assessment, all emissions may be categorised as environmentally insignificant. | ü |
| Operating techniques | We have reviewed the techniques used by the operator and compared these with the relevant guidance notes. The proposed techniques / emission levels for priorities for control are in line with the benchmark levels contained in the TGN and we consider them to represent appropriate techniques for the facility. | ü |
| The permit conditions |  |  |  |
| Emission limits | We have decided that emission limits should be set for the parameters listed in the permit. It is considered that the numeric limits described below will prevent significant deterioration of receiving waters. We have imposed numeric limits because either a relevant environmental quality or operational standard requires this. - BOD: 8mg/l - Ammona: 3.6 mg/l - Phosphorus: 4.5 mg/l - Suspended Solids: 8 mg/l We have imposed numeric limits because either a relevant environmental quality or operational standard requires this. A Phosphorus limit has been set to comply with current planning regulations for developments within SACs. Additional limits have been set to ensure additional protection of the River Dee SAC. | ü |
| Considerations of foul sewer | We agree with the operators justification for not connecting to foul sewer. | ü |
| Operator Competence |  |  |  |
| Environment Management System | There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. | ü |
| Sustainable Management of Natural Resources (SMNR) |  |  |
| Considerations of SMNR - Compliance with our General Purpose | We are satisfied that this decision is compatible with our general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources” | ü |
| Environment Wales Act 2016 |  |  |
| Biodiversity and resilience of ecosystems duty | Section 6 of the Environment Wales Act 2016 requires that we seek to maintain and enhance biodiversity in the exercise of our functions, and in so doing promote the resilience of ecosystems, in a manner that is consistent with the proper exercise of our functions. NRW is satisfied that in this case, we have taken into account and had due regard to this duty in so far as it is consistent with the function of determining an application for a stand alone water discharge activity permit under EPR2016. | ü |