# Natural Resources Wales permitting decisions

# Bespoke permit

We have decided to grant the permit for Glanarafon Uchaf Caravan Park operated by Bryn Awel Parks Limited Liability Partnership.

The application number is PAN-026400

The permit number is EPR-DB3397HV

The permit regulates an existing discharge from Glanarafon Uchaf Caravan Park. The discharge from two package plants serves 51 caravans, one holiday lodge, one residential property, a toilet block and laundrette. The discharge is currently entering an existing infiltration system that does not meet British Standards and is located within close proximity to an unnamed stream. Improvement conditions have been included in the permit to reduce ammonia to 10mg/l and to install a pipe to discharge directly to the unnamed stream by 1 March 2025.

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

Purpose of this document

This decision document:

- explains how the application has been determined

- provides a record of the decision-making process

- shows how all relevant factors have been taken into account

- justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

Structure of this document

- Annex 1 the decision checklist

# Annex 1: decision checklist

This checklist should be read in conjunction with the Duly Making checklist.

| ### Aspect | ### Justification / Detail | Criteria met |
| --- | --- | --- |
|  |  | Yes |
| Consultation |  |  |  |
| Scope of consultation | The consultation requirements were identified and implemented. The decision was taken in accordance with RGN 6 High Profile Sites, our Public Participation Statement and our Working Together Agreements. | ü |
| ### Responses to web publicising | The web publicising responses (Annex 2) were taken into account in the decision. No responses were received. The decision was taken in accordance with our guidance. | ü |
| Operator |  |  |  |
| Control of the facility | We are satisfied that the applicant (now the operator) is the person who will have control over the operation of the facility after the grant of the permit. The decision was taken in accordance with EPR RGN 1 Understanding the meaning of operator. | ü |
| European Directives |  |  |  |
| Applicable directives | All applicable European directives have been considered in the determination of the application. | ü |
| WFD | In light of the conclusions of a detailed compliance assessment (Stage 3), and taking account of the advice received from technical specialist advisors, it has been established that the activity/project has no potential to cause deterioration of any water body or prevent a water body or WFD Protected Area from meeting its objectives, taking into account any legally binding avoidance measures or restrictions as applicable, either alone or in-combination with other activities. | ü |
| The site |  |  |  |
| Extent of the site of the facility | The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility including discharge points. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary. | ü |
| Biodiversity, Heritage, Landscape and Nature Conservation and a designated site under the Wildlife and Countryside Act. | The application is within the relevant distance criteria of 1000m of a site of heritage, landscape or nature conservation, and/or protected species or habitat . Y Bonc SSSI was screened in. However, the SSSI lies approximately 924m to the south west of the discharge point and is not hydrologically connected. Therefore no Appendix 4 was undertaken. | ü |
| Habitat Regulation Assessment for a designated site under the EU Habitats Directive | No FORM 1 completed: HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site Note: From OGN 200 - no conceivable impact on any Natura 2000 site, by virtue of the scale or location or nature of the project. The screening distance of 1000m did not screen in any designated sites. | ü |
| Environmental Risk Assessment and operating techniques |  |  |  |
| Environmental risk | We have carried out a risk assessment on behalf of the operator. The operator considers this risk assessment is satisfactory. | ü |
| Operating techniques | We have reviewed the techniques used by the operator and compared these with the relevant guidance notes. The proposed techniques / emission levels for priorities for control are in line with the benchmark levels contained in the TGN and we consider them to represent appropriate techniques for the facility. | ü |
| Improvement conditions | Based on the information on the application, we consider that we need to impose improvement conditions. We have imposed improvement conditions to ensure that: The concentration of ammonia in the discharge is reduced from 20mg/l to 10mg/l by 10 January 2026. A solid pipe is installed to convey the discharge directly to the water course by 10 January 2026. | ü |
| Emission limits | We have decided that emission limits should be set for the parameters listed in the permit. It is considered that the numeric limits described below will prevent significant deterioration of receiving waters. We have imposed numeric limits because either a relevant environmental quality or operational standard requires this. BOD – 20mg/l Suspended solids – 30mg/l Ammonia – 10mg/l Following water quality modelling using the Monte Carlo software and using inputs including WFD data, the above limits for BOD and ammoniar are required to protect the receiving watercourse and ensure it maintains a Good status. Suspended solids is set at the capacity of the package treatment plants. | ü |
| The permit conditions |  |  |  |
| Considerations of foul sewer | We agree with the operators justification for not connecting to foul sewer. | ü |
| Operator Competence |  |  |  |
| Environment Management System | There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. | ü |
| Sustainable Management of Natural Resources (SMNR) |  |  |
| Considerations of SMNR - Compliance with our General Purpose | We are satisfied that this decision is compatible with our general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources” | ü |
| Environment Wales Act 2016 |  |  |
| Biodiversity and resilience of ecosystems duty | Section 6 of the Environment Wales Act 2016 requires that we seek to maintain and enhance biodiversity in the exercise of our functions, and in so doing promote the resilience of ecosystems, in a manner that is consistent with the proper exercise of our functions. NRW is satisfied that in this case, we have taken into account and had due regard to this duty in so far as it is consistent with the function of determining an application for a standalone water discharge activity permit under EPR2016. | ü |