# Natural Resources Wales permitting decisions.

# Bespoke permit

We have decided to grant the permit for Broad Oak Farm operated by Funky Furniture Hire (Properties) Limited.

The application number is PAN-026858

The permit number is EPR/DB3399ZN

The discharge is for a new bespoke discharge of 6.5m3 secondary treated domestic effluent to surface water. The discharge will be derived from a housing development of 6x2 bed, 1x3 bed, 3x4 bed and 1x5 bed properties. The proposed system replaces an existing unregistered discharge of 1.5m3 from the main farmhouse (5 bed property that will remain and form part of the new development) and associated toilet facilities in the outbuildings. The existing treatment is via a septic tank to surface water.

The applicant is proposing to install a MI Nutra-Lite treatment plant. The discharge from the package treatment plant would then pass through the MI Phos-Lite unit which would reduce phosphate levels to 0.2mg/l. The system treats phosphate using a natural, non-chemical dosing solution.

The existing and proposed discharge is into an unnamed stream that is a tributary of Pulford Brook. The proposed discharge is within the River Dee phosphate sensitive SAC catchment.

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

Purpose of this document

This decision document:

- explains how the application has been determined.

- provides a record of the decision-making process.

- shows how all relevant factors have been taken into account.

- justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise, we have accepted the applicant’s proposals.

Structure of this document

- Annex 1 the decision checklist

# Annex 1: decision checklist

This checklist should be read in conjunction with the Duly Making checklist.

| ### Aspect | ### Justification / Detail | Criteria met |
| --- | --- | --- |
|  |  | Yes |
| Consultation |  |  |  |
| Scope of consultation | The consultation requirements were identified and implemented. The decision was taken in accordance with RGN 6 High Profile Sites, our Public Participation Statement and our Working Together Agreements. | ü |
| ### Responses to web publicising | The web publicising responses. None were received. The decision was taken in accordance with our guidance. | ü |
| Operator |  |  |  |
| Control of the facility | We are satisfied that the applicant (now the operator) is the person who will have control over the operation of the facility after the grant of the permit. The decision was taken in accordance with EPR RGN 1 Understanding the meaning of operator. | ü |
| European Directives |  |  |  |
| Applicable directives | All applicable European directives have been considered in the determination of the application. | ü |
| WFD | WFD stage 2 scoping has been completed and the activity/project is considered as having no risk of causing deterioration or preventing any water body or WFD Protected Area from reaching its objectives and taking account of the advice from technical officers - is considered compliant with the WFD Regulations 2017. See WFD form here |  |
| The site |  |  |  |
| Extent of the site of the facility | The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility including discharge points. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary. | ü |
| Habitat Regulation Assessment for a designated site under the EU Habitats Directive | Appropriate assessment: In light of the conclusions of an appropriate assessment, and taking account of the advice received from protected sites advisors, it has been established that the project will not adversely affect the integrity of any Natura 2000/Ramsar site, taking into account any conditions or restrictions as applicable, either alone or in-combination with other plans and projects. As documented in section 4 of OGN 200 form 1. See Form 1 here |  |
| Environmental Risk Assessment and operating techniques |  |  |  |
| Environmental risk | We have carried out a risk assessment on behalf of the operator. The operator considers this risk assessment is satisfactory. |  |
| Operating techniques | We have reviewed the techniques used by the operator and compared these with the relevant guidance notes. The proposed techniques / emission levels for priorities for control are in line with the benchmark levels contained in the TGN 7.01 and we consider them to represent appropriate techniques for the facility. | ü |
| The permit conditions |  |  |  |
| Emission limits | We have decided that emission limits should be set for the parameters listed in the permit. Ammoniacal nitrogen - 3mg/l ATU-BOD as O2 - 6 mg/l Total phosphorus as P – 0.2 mg/l Suspended solids (measured after drying at 105o C) – 30 mg/l Visible oil or grease no significant trace present. These limits have been based on the performance of the proposed treatment system. Modelling of the effluent has been undertaken to ensure that no deterioration to the WFD classification and against SAC targets. These are tight limits and will ensure that the treatment systems are well maintained and discharges the same quality of effluent throughout their lifecycle. | ü |
| Considerations of foul sewer | We agree with the operator’s justification for not connecting to foul sewer. | ü |
| Operator Competence |  |  |  |
| Environment Management System | There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. | ü |
| Sustainable Management of Natural Resources (SMNR) |  |  |
| Considerations of SMNR - Compliance with our General Purpose | We are satisfied that this decision is compatible with our general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources”. | ü |
| Environment Wales Act 2016 |  |  |
| Biodiversity and resilience of ecosystems duty | Section 6 of the Environment Wales Act 2016 requires that we seek to maintain and enhance biodiversity in the exercise of our functions, and in so doing promote the resilience of ecosystems, in a manner that is consistent with the proper exercise of our functions. NRW is satisfied that in this case, we have taken into account and had due regard to this duty in so far as it is consistent with the function of determining an application for a stand-alone water discharge activity permit under EPR2016. | ü |