[image: image1.emf]
Record of a Habitats Regulations Assessment of a project or spatially-specific plan
OGN 200 Form 1A (with free text boxes for LSE screening and appropriate assessment)
Document owner: Head of Business for Natural Resource Management
Version History:
	Document Version
	Date Published
	Summary of Changes

	2.0
	*month* 2024
	Following 2023/24 review of HRA OGNs

	
	
	

	
	
	

	
	
	


Next review date:  April 2025
OGN 200 Form 1A 

Record of a Habitats Regulations Assessment

	Plan or project name, brief description or application reference number
	PAN-025629 MCP at Johnsons Hotel Linen Limited

The MCP is a single 5.1 MWth industrial boiler operated by Johnsons Hotel Linen Limited




	HRA iteration/version
	N/A



1.
Plan or Project Details
	Information about the plan or project  

	1
	Date application received
	15/03/2024

	2
	Applicant details
	Johnsons Hotel Linen Limited

	3
	NRW team responsible for carrying out, or requiring to be carried out, the plan or project, and name of lead officer
	N/A

	4
	Activity/ies proposed


	The MCPis a single 5.1 MWth industrial boiler operated by Johnsons Hotel Linen Limited, and providing heat to JOHNSONS Hotel Lined facility.  It is fuelled using Liquid Petroleum Gas. 
Waste gases are discharged through a 10m high stack, and it is the only MCP / permitted facility within the production site.



	5
	Relevant legislation or statutory basis
	Environmental Permitting Regulations (2016)

	6
	Location
	Johnsons Hotel Linen Limited
MCP gas fired steam boiler at Johnsons Hotel Linen site in Pwllheli

Johnsons Hotel Linen, 

Ffordd Cae Gwyn, Chwilog, 

Pwllheli, Gwynedd, 

LL53 6SHA
Grid ref: 243664,337835

	7
	Plan or project documents, including any application documents
	N/A


	8
	Environmental Statement
	N/A


	9
	Pre-application correspondence 
	
 N/A



	10
	NRW team responsible for preparing this HRA report, and lead officer
	Decarbonisation and Standards team on behalf of Regulated Industry Permitting Team


	11
	Team or person responsible for approving the plan or project (competent authority role)
	As row 10



2.
Determining the need for a Habitats Regulations Assessment

	2.1 Is there any possibility that the plan or project could negatively affect any European sites?
	YES


	2.2 Is the whole of the plan or project directly and only connected with or necessary to the management of one or more European sites, for the purposes of conserving the habitats or species for which the European site(s) is/are designated?
	NO


	2.3 Is there a possibility that the plan or project could affect any other feature of the European site(s) concerned, or of another European site, in a way that would undermine that feature’s conservation objectives?
	NO


3.
Considering the likelihood of a significant effect (LSE) 
3.1
Renewal of a project authorisation on the same or more restrictive terms as an extant authorisation
	Is this a renewal of an extant authorisation which complies with NRW approved criteria for ruling out significant effects of renewals (see Part 2 of OGN 200) without conducting a project-specific LSE test?
	NO




3.2
Likelihood of significant effects (LSE) test 
.

3.2.1 Which European sites might be affected by the plan or project?

	(a)
	Based on the plan or project specification, or information provided in the application, it is considered that these European sites have features which could be negatively affected by the plan or project
	· Pen Llyn a`r Sarnau / Lleyn Peninsula and the Sarnau Special Area of Conservation (SAC); and, 

· 
· 

	(b)
	The potential for the plan or project to negatively affect these European sites was also initially considered, but can be ruled out without further consideration
	N/A


3.2.2 Screening for likelihood of significant effect
	TABLE 3.2.2  Screening assessment

	· European site name: Pen Llyn a`r Sarnau / Lleyn Peninsula and the Sarnau Special Area of Conservation (SAC); and, 

· Glanllynnau a Glannau Pen-Ychain i Gricieth Site of Special Scientific Interest (SSSI



	The nine habitat features of the SAC are:

1. Reefs

2. Large shallow inlets and bays

3. Sandbanks which are slightly covered by seawater all the time

4. Estuaries

5. Coastal lagoons

6. Mudflats and sandflats not covered by seawater at low tide

7. Atlantic salt meadows

8. Salicornia and other annuals colonising mud and sand

9. Submerged or partially submerged sea caves

All screen out for NOx, SO2, NDep and Acid Dep using the SCAIL combustion screening tool.  The size of the emission impact is so small that there is no credible impact on the site or its features, no Likely significant effect, and no further assessment required.


	TABLE 3.2.3 Screening decision of the plan or project ‘alone’



	(a) If the screening conclusion for all features for all sites in Table 3.2.2 is ‘SCREEN OUT’
	The plan or project is not likely to have a significant effect on any European site, and no further consideration under the Habitats Regulations is required in order to determine the approval/application.


	(b) If the conclusion for any features in Table 3.2.2 is ‘SCREEN IN’ 
	The plan or project is likely to have a significant effect on one or more European sites and therefore an appropriate assessment is required.


	(c) If there are no features in Table 3.2.2 that are ‘SCREEN IN’ and any features that are ‘IN COMB’
	The plan or project is not likely to have a significant effect on any European sites when considered alone, but the possibility of significant effects in combination with other plans and projects needs to be considered.



�Natural Gas?





www.naturalresources.wales








Page 20 of 20

