# Natural Resources Wales permitting decisions

# Bespoke permit

We have decided to grant the permit for Great Milton Park operated by Redrow Homes Limited.

The application number is PAN-026982.

The permit number is EPR/DB3496HV.

This permit regulates a temporary site drainage rainfall related discharge from a construction site. The site will consist of a large residential development of 850 residential properties, currently being built.

The discharge will be directed into site swales and 3 attenuation ponds to allow the settlement of silt. Gel Flocculants which contain anionic polyacrylamide and Aluminium will also be used to allow the agglomeration of silt to form larger “flocs” of silt which then settle. The discharges will then pass by from: Pond 1 into Monk’s ditch, Pond 2 into a higways drain at Station Road leading to Llanwern ditch and Pond 3 will through a Dŵr Cymru surface water sewer leading to an unnamed tributary of the Usk.

- The applicant is currently discharging from Pond 1 but has not started to discharge from Ponds 2 or 3. Pond 2 will have a pre-operational condition requiring that the highways drain is improved prior to discharge from Pond 2.

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

Purpose of this document

This decision document:

- explains how the application has been determined

- provides a record of the decision-making process

- shows how all relevant factors have been taken into account

- justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

Structure of this document

- Annex 1 the decision checklist

- Annex 2 the web publicising responses

# Annex 1: decision checklist

This checklist should be read in conjunction with the Duly Making checklist.

| ### Aspect | ### Justification / Detail | Criteria met |
| --- | --- | --- |
|  |  | Yes |
| Consultation |  |  |  |
| Scope of consultation | The consultation requirements were identified and implemented. The decision was taken in accordance with RGN 6 High Profile Sites, our Public Participation Statement and our Working Together Agreements. | ü |
| ### Responses to web publicising | The web publicising responses (Annex 2) were taken into account in the decision. The decision was taken in accordance with our guidance. | ü |
| Operator |  |  |  |
| Control of the facility | We are satisfied that the applicant (now the operator) is the person who will have control over the operation of the facility after the grant of the permit. The decision was taken in accordance with EPR RGN 1 Understanding the meaning of operator. | ü |
| European Directives |  |  |  |
| Applicable directives | All applicable European directives have been considered in the determination of the application. | ü |
| WFD | WFD stage 2 scoping has been completed and the activity/project is considered as having no risk of causing deterioration or preventing any water body or WFD Protected Area from reaching its objectives and taking account of the advice from technical officers - is considered compliant with the WFD Regulations 2017. |  |
| The site |  |  |  |
| Extent of the site of the facility | The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility including discharge points. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary. | ü |
| Biodiversity, Heritage, Landscape and Nature Conservation and a designated site under the Wildlife and Countryside Act. | The application is within the relevant distance criteria of a site of heritage, landscape or nature conservation, and/or protected species or habitat . A full assessment of the application and its potential to affect the sites has been carried out as part of the permitting process. We consider that the application will not affect the features of the sites. Formal consultation has been carried out with Newport Council Biodiversity Team because the discharge is within screening distance of Monk’s ditch and Dockwell Wood local wildlife sites. No responses were received from Newport council. | ü |
| Habitat Regulation Assessment for a designated site under the EU Habitats Directive | Appropriate assessment: In light of the conclusions of an appropriate assessment, and taking account of the advice received from protected sites advisors, it has been established that the project will not adversely affect the integrity of any Natura 2000/Ramsar site, taking into account any conditions or restrictions as applicable, either alone or in-combination with other plans and projects. (As documented in section 4 of OGN 200 form 1, and section 5 if applicable) |  |
| Environmental Risk Assessment and operating techniques |  |  |  |
| Environmental risk | We have carried out a risk assessment on behalf of the operator. The operator considers this risk assessment is satisfactory. The assessment shows that, applying the conservative criteria in our guidance on Environmental Risk Assessment, all emissions may be categorised as environmentally insignificant. | ü |
| Operating techniques | We have reviewed the techniques used by the operator and compared these with the relevant guidance notes. The proposed techniques / emission levels for priorities for control are in line with the benchmark levels contained in the TGN and we consider them to represent appropriate techniques for the facility. | ü |
| The permit conditions |  |  |  |
| Pre-operational conditions | Based on the information on the application, we consider that we need to impose pre-operational conditions. A condition has been put on the permit that states that no discharge from Pond 2 shall be permitted into the Station Road highway drainage system until it is upgraded. Currently the highways drain does not have capacity to accommodate the discharge without risk of flooding. This condition has been agreed with NRW’s Flood Risk Management Team. | ü |
| Emission limits | We have decided that emission limits should be set for the parameters listed in the permit. It is considered that the numeric limits described below will prevent significant deterioration of receiving waters. We have imposed numeric limits because either a relevant environmental quality or operational standard requires this. - Suspended Solids (measured after drying at 105oC) – 60 mg/l - Total Aluminium as Al – 1,000 µg/l - pH between 6 -9 The suspended solids limits has been set at the Water Quality Standard of 60mg/l in order to protect the receiving watercourse. The total Aluminium limit has been set as the MAC. The pH limit has been set in order to safeguard the receiving environment. | ü |
| Considerations of foul sewer | We agree with the operators justification for not connecting to foul sewer. | ü |
| Operator Competence |  |  |  |
| Environment Management System | There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. | ü |
| Sustainable Management of Natural Resources (SMNR) |  |  |
| Considerations of SMNR - Compliance with our General Purpose | We are satisfied that this decision is compatible with our general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources” | ü |
| Environment Wales Act 2016 |  |  |
| Biodiversity and resilience of ecosystems duty | Section 6 of the Environment Wales Act 2016 requires that we seek to maintain and enhance biodiversity in the exercise of our functions, and in so doing promote the resilience of ecosystems, in a manner that is consistent with the proper exercise of our functions. NRW is satisfied that in this case, we have taken into account and had due regard to this duty in so far as it is consistent with the function of determining an application for a stand alone water discharge activity permit under EPR2016. | ü |

# Annex 2: Consultation responses

Response received from

Member of the public following web advertising: 27/11/2024.

Brief summary of issues raised

Concerns raised about potential flooding of Monk’s ditch due to this proposed discharge and concerns about flooding from Pond 2 into Station Road highways drain.

Summary of actions taken or show how this has been covered

The permit will have a limit with regards to amount of flow that can be discharged into Monk’s ditch from Pond 1. The applicant is currently discharging from Pond 1 so this discharge needs to be regulated. Regulating this discharge will mean that should the flow rate entering Pond 1 be exceeded, enforcement action can take place. A pre-operational condition will be included in the permit stating that there is to be no discharge into Station Road highways drain until it is improved.