ENVIRONMENT AGENCY

	Form WR – 38
	Ref: prince's gate spring water - bh1 in top field (edwards)
	Agency No.


BOREHOLE RECORD



BH 4 in Top Field
A.  SITE DETAILS 

	Borehole drilled for:
 
	Endaf Edwards, Prince’s Gate Spring Water

	Location:
 

	Prince’s Gate, Narberth, SA67 8JD

	N.G.R.:
 


	SM159114

	Ground Level (if known):
	SURFACE

	Drilling Company:  

	W.B. & A.D. MORGAN LTD., PRESTEIGNE, POWYS.  LD8 2UF

	Date of Drilling:

	Commenced:
	9/10/14
	Completed:
	24/10/14


B.  CONSTRUCTION DETAILS

	Borehole datum (if not ground level)
GROUNDLEVEL



(Point from which all measurements of depth are taken e.g. flange, edge of chamber, etc.)

	Borehole drilled diameter...………….                                       
	460
	mm from
	Surface
	to
	4.5
	m/depth

	
	380
	mm from
	4.5
	to
	79
	m/depth

	
	
	mm from
	
	to
	
	m/depth

	Casing material: Stainless Steel
	diameter
	200
	mm from
	Surface
	to
	79
	m/depth

	and type (e.g. plain steel, plastic slotted)

	Plain
	diameter
	200
	mm from
	Surface
	to
	25
	m/depth

	1mm Slotted
	diameter
	200
	mm from
	25
	to
	31
	m/depth

	Plain
	diameter
	200
	mm from
	31
	to
	37
	m/depth

	1mm Slotted
	diameter
	200
	mm from
	37
	to
	49
	m/depth

	Plain
	diameter
	200
	mm from
	49
	to
	55
	m/depth

	1mm Slotted
	diameter
	200
	mm from
	55
	to
	61
	m/depth

	Plain
	diameter
	200
	mm from
	61
	to
	67
	m/depth

	1mm Slotted
	diameter
	200
	mm from
	67
	to
	79
	m/depth

	
	
	
	
	
	
	

	Grouting details: 
	Pump Bentonite 18m
	to surface

	Water struck at: 
	2.5m, 8m, 22m, 27
	m (depth below datum – mbd)

	Rest water level on completion:
	9
	m (depth below datum – mbd)

	Estimated blowout yield: 
	5,400
	Gallons per hour


C.  STRATA LOG

	Description of Strata
	Thickness (m)
	Depth (m)

	Soft brown subsoil
Soft brown sandstone mudstone

Medium red mudstone marl

Brown sandstone

Medium red mudstone marl with sandstone bands

Brown/grey fractured sandstone

Hard grey limestone (mudstone)

Red mudstone marls

Medium red mudstone with grey limestone bands

Red sandstone mudstone


	0.5
3.5

7

8

21

4

6

10

11

8


	0.5
4

11

19

40

44

50

60

71

79

	Other Comments 

(e.g. gas encountered, saline water intercepted, etc.)
	

	Gravel Pack Quantity: 9,000kg
	Bentonite: 50kg
	Temp Steel Casing:

Depth and Diameter
	4m x 506mm

	Cement: 2,550kg
	
	
	

	Rig & Crew:
	Knebel, D. Morris, R. Davies


