Part A

| Permit Number | VP3935AT |
| --- | --- |
| Name of Operator | Enovert South Ltd |
| Location of Installation | Tir John Landfill, Wern Terrace, Off Fabian Way, Swansea, SA1 8PA |
| Time and date of the detection | May 2025 |
| (b) Notification requirements for the breach of a limit (within 24 hours unless otherwise agreed) |  |
| Emission point reference/ source | Phase 2 LEW Leachate Wells Leachate Head above Cell Base >1m See Table 1 Below |
| Parameter(s) |  |
| Limit |  |
| Measured value and uncertainty |  |
| Date and time of monitoring |  |
| Measures taken, or intended to be taken, to stop the emission | Leachate remains contained within the engineered containment system, so no emission from the site is occurring. |

Part B to be supplied as soon as practicable

| Any more accurate information on the matters for notification under Part A. | Leachate removal to sewer is ongoing. |
| --- | --- |
| Measures taken, or intended to be taken, to prevent a recurrence of the incident | Landfill capping works are almost complete, and we may experience new fluctuations across leachate levels as the site settles. It is unlikely that all the dipped levels are representative of the actual leachate levels in the waste mass. It is likely that as the waste ages and compresses, the pore spaces are reduced, and liquid is effectively squeezed out into the wells. Pumping, capping and drainage methods have impacted average leachate heads across the site, which appear to be following a downward trend year on year. |
| Measures taken, or intended to be taken, to rectify, limit or prevent any pollution of the environment which has been or may be caused by the emission | The elevated levels in wells LEW15a and LEW16 are due to well or pump clogging issues. Solutions have been under discussion within Enovert and with NRW. Initial dosing with bleach failed to break up the blockage, we have trialled dosing material with citric acid which does soften it to some extent, but the process is extremely slow and not practical for site application. We continue to use physical methods to maximise extraction from Cell 16. The substance will be sampled for further laboratory analysis and testing of treatment methods. We have commissioned consultants to support us with this work. |
| The dates of any unauthorised emissions from the installation in the preceding 24 months. | See previous notifications. |
| Name* | Elizabeth Forrest |
| Post | Environmental Data Analyst |
| Date | 19/06/2025 |

* authorised to sign on behalf of Enovert South Ltd

| Sample Point | Date | Leachate Head (m) | Comment |
| --- | --- | --- | --- |
| LEW07 | 14/05/2025 | 1.30 |  |
| LEW12a | 14/05/2025 | 1.07 |  |
| LEW15a | 14/05/2025 |  | Affected by solidifying substance |
| LEW16 | 14/05/2025 |  | Affected by solidifying substance |
| LEW21 | 14/05/2025 | 3.49 |  |
| LEW22 | 14/05/2025 | 3.01 |  |
| LEW23 | 14/05/2025 | 3.32 |  |
| LEW24 | 14/05/2025 | 3.41 |  |
| LEW25 | 14/05/2025 | 2.18 |  |
| LEW26 | 14/05/2025 | 1.71 |  |
| LEW28 | 14/05/2025 | 4.54 |  |
| LEW29 | 14/05/2025 | 7.22 |  |
| LEW30 | 14/05/2025 | 3.68 |  |
| LEW31 | 14/05/2025 | 4.79 |  |
| LEW32 | 14/05/2025 | 3.24 |  |

Table 1: Breach of Compliance Limit for Leachate Boreholes Phase 2 area May 2025