Site Condition Report evaluation form

	Applicant
	Neath Port Talbot Borough Council 

	Permit reference
	EPR-BJ5775IF V020 
PAN 024031 / PAN 024032

	Address
	Materials Recovery and Energy Centre Baldwins Crescent Crymlyn Burrows Swansea SA1 8PZ

	Grid reference
	SS 69838 93372

	Activity Type
	The site is changing from a Energy From Waste Facility (energy from waste activity never properly began) with Waste Treatment  to a 
Waste Transfer Station with Treatment 


	Surrender Type
	Part surrender of activities only  - No land was surrendered during this surrender. 
SCR evaluation template has been completed to record the changes on site only. 

This surrender was agreed to be a low risk surrender. 

	Site Condition Report document reference, date and version number
	SLR REF: 402.02718.0005
Version 2

September 2024


	1.0 Site details (Source)


	Has the applicant provided the following information as required by the application SCR template?

Site plans showing site layout, drainage, surfacing, receptors, sources of emissions/releases and monitoring points
	The permitted facility predates the requirement for them to include a site condition report with their initial application. 
A retrospective site condition report has been provided for this surrender application which  provides the relevant information for this section based with some of the information being from a ‘Ground Conditions and Contamination’ report from the initial application (Gibbs, 1998). 


	2.0 Condition of the land at permit issue (Receptor)



	Has the applicant provided the following information as required by the application SCR template?

a) Environmental setting including geology, hydrogeology and surface waters

b) Pollution history including:

· pollution incidents that may have affected land

· historical land-uses and associated contaminants

· visual/olfactory evidence of existing contamination

· evidence of damage to existing pollution prevention measures

c) Evidence of historic contamination (i.e. historical site investigation, assessment, remediation and verification reports (where available)

d) Has the applicant chosen to collect baseline reference data?
	The permitted facility predates the requirement for them to include a site condition report with their initial application. 

A retrospective site condition report has been provided for this surrender application which  provides the relevant information for this section based with some of the information being from a ‘Ground Conditions and Contamination’ report from the initial application (Gibbs, 1998).


	3.0 Permitted activities (Source)



	Has the applicant provided the following information as required by the application SCR template?

a) Permitted activities

b) Non-permitted activities undertaken at the site
	The currently permitted activities on site are detailed in Schedule 1 of the EP, Table S1.1 (included as Appendix SCR2); 

· AR1: Section 5.1 A (1) (b): Incineration of non-hazardous waste in an incineration plant with a capacity exceeding 3 tonnes per hour; and 

· AR2: Section 5.4 A (1) (a) (ii): Disposal of non-hazardous waste with a capacity exceeding 50 tonnes per day by physico-chemical treatment. The associated 

· Directly Associated Activities (DAAs) are detailed in Schedule 1 of the EP, Table S1.1.
It is worth noting at the time this partial surrender was submitted activities had already changed on site and the above activities had stopped. Alongside this application the operator also submitted a variation to vary the site to a waste operation with the following R&D codes: 

· R3: Recycling/reclamation of organic substances which are not used as solvents; 
· R4: Recycling/reclamation of metals and metal compounds; 
· R5: Recycling/reclamation of other inorganic materials; 
· R13: Storage of wastes pending any of the operations numbered R1 to R12 (excluding temporary storage, pending collection, on the site where it is produced); 
· D9 – physico-chemical treatment; 
· D14 – repackaging prior to submission to any of the operations numbered D1 to 13; and 
· D15 – storage pending any of the operations numbered D1 to D14.
Which are the activities actually being undertaken on site. 




	3.0(a) Environmental Risk Assessment (Source)



	The environmental risk assessment should identify elements that could impact on land and waters, cross- referenced back to documents and plans provided as part of the wider permit application.
	The permitted facility predates the requirement for them to include a site condition report with their initial application. 



	3.0(b) Will the pollution prevention measures protect land and groundwater? (Conceptual model)



	Are the activities likely to result in pollution of land? 


	The permitted facility predates the requirement for them to include a site condition report with their initial application. 

	For dangerous and/or hazardous substances only, are the pollution prevention measures for the relevant activities to a standard that is likely to prevent pollution of land?

	The permitted facility predates the requirement for them to include a site condition report with their initial application. 


	4.0 Changes to the activities (Source)

	Have there been any changes to the following during the operation of the site?

a) Activity boundaries

b) Permitted activities

c) “Dangerous substances” used or produced
	The permitted facility predates the requirement for them to include a site condition report with their initial application. 

A retrospective site condition report has been provided for this surrender application which  provides the relevant information for this section based with some of the information being from a ‘Ground Conditions and Contamination’ report from the initial application (Gibbs, 1998). 


	5.0 Measures taken to protect land (Pathway)


	Has the applicant provided evidence from records collated during the lifetime of the permit, to show that the pollution prevention measures have worked?
	The permitted facility predates the requirement for them to include a site condition report with their initial application. 

A retrospective site condition report has been provided for this surrender application which  provides the relevant information for this section based with some of the information being from a ‘Ground Conditions and Contamination’ report from the initial application (Gibbs, 1998). 


	6.0 Pollution incidents that may have impacted on land and their remediation (Sources)


	Has the applicant provided evidence to show that any pollution incidents which have taken place during the life of the permit, and which may have impacted on land or water have been investigated and remediated (where necessary)?


	The permitted facility predates the requirement for them to include a site condition report with their initial application. 

A retrospective site condition report has been provided for this surrender application which  provides the relevant information for this section based with some of the information being from a ‘Ground Conditions and Contamination’ report from the initial application (Gibbs, 1998).


	7.0 Soil gas and water quality monitoring 


	Where soil gas and/or water quality monitoring has been undertaken, does this demonstrate that there has been no change in the condition of the land? Has any change that has occurred been investigated and remediated?
	The permitted facility predates the requirement for them to include a site condition report with their initial application. 

A retrospective site condition report has been provided for this surrender application which  provides the relevant information for this section based with some of the information being from a ‘Ground Conditions and Contamination’ report from the initial application (Gibbs, 1998). 


	8.0 Decommissioning and removal of pollution risk



	Has the applicant demonstrated that decommissioning works have been undertaken and that all pollution risks associated with the site have been removed? Has any contamination of land that has occurred during these activities been investigated and remediated?
	REMINDER – This surrender is for the removal of activities only no land was surrendered as part of this application. 

Section 9 of the operator SCR confirms that Waste to Energy operation was decommissioned in 2023 and this was confirmed in a CAR Form. NRW0042762. The decommissioning included the removal of: 
· Waste to energy plant and associated feed and discharge plant, turbine generator, and air handling equipment including 40m stack; 

· Emission points A1 and A2 (incinerator line 1 and 2 via 40m stack); 

· The nine in-vessel bio-drying tunnels together with the associated leachate management, bio-filter, and feed, discharge and refining plant (conveyors, shredders, over band magnets, eddy current separators, baler);

·  Point source emission A4 (compost system air vent);

· Leachate sump and tank; 

· Fuel prep extraction stack (stack remains in place but is scheduled for removal under future works. Following 2022-23 works there is no point source emissions from the stack); 

· Re-duster system; and 

· Emission point A3 (de-duster system vents via 18m stack)

The CAR confirms that the flooring in the building was “in good condition and fully impermeable displaying no signs of fracture of weakness”. In addition, it was confirmed that the incineration stack and associated plant “have been appropriately cleared with no signs of contamination or pollution in the immediate area”. The CAR includes photographs evidencing the decommissioned areas and is included as Appendix SCR3.7.




	9.0 Reference data and remediation 


	Has the applicant provided details of any surrender reference data that they have collected and any remediation that they have undertaken?

If the surrender reference data shows that the condition of the land has changed as a result of the permitted activities, the applicant will need to undertake remediation to return the condition of the land back to that at permit issue. You should not require remediation of historic contamination or contamination arising from non-permitted activities as part of the permit surrender.
	N/A


	10.0 Statement of site condition

	Has the applicant provided a statement, backed up with evidence, confirming that the permitted activities have ceased, decommissioning works are complete, and that pollution risk has been removed and that the land and waters at the site are in a satisfactory state? 
	As per the above the site has been appropriately decommissioned and the activities can be surrendered. The Site inspection following the removal of the plant, equipment and machinery used in conjunction with the relevant activities above confirmed the site was clear and in good condition. 


	Surrender SCR decision summary



	The following information must be obtained from the applicant before the permit is determined:
As the Operator has not requested to surrender land we have not assessed whether the site condition has been returned to suitable condition this will be undertaken should the Operator surrender the site (including land). However, we have been mindful to consider whether the installation activities could have affected the site condition. As site activities under the installation activity were never operated we consider there is no way in which the site condition could have been affected by Section 5.1 A (1) (c) or  Section 5.3 A (1) (c) (ii) activities. Whlist the site was an installation,  it has  and will continue to operate in line with a waste operation activites as listed in this varied permit. There have been no pollutions incidents. We therefore deem the surrender of these installations activities suitable.
The plant equipment, and machinery associated with the above listed activities have been decommissioned and the site cleared accordingly in a manner to allow surrender of the activities. 



	Date and name of reviewer:
Matthew Fryer
Senior Permitting Officer 

06/03/2025




1
1

