Complying with the WFD Regulations 2017: detailed assessment template

Guidance note

Reference number: GN078

Document Owner: Integrated Water Planning Team, NRW

What is this document about?

Detailed assessment template for compliance with the WFD Regulations 2017. Please refer to ‘Complying with the WFD Regulations detailed assessment guidance for further information.

Who is this document for?

This guidance is for anyone designing projects and activities that may impact on the water environment.

You must provide WFD compliance information to support any application for activities that may cause deterioration to a WFD water body or prevent it (or WFD Protected Areas) from achieving the objectives set out in the River Basin Management Plan.

Be aware that you are likely to need to employ a suitably experienced and qualified consultant to do a WFD assessment. You may also need specialist technical input depending on the activity and potential impacts.

Contact for queries and feedback

Integrated Water Planning Team, Evidence, Policy and Permitting Directorate

WFDWales@cyfoethnaturiolcymru.gov.uk

Version History

| Document version | Date published | Summary of changes |
| --- | --- | --- |
| 1.0 | 26/06/24 | Document published |

Review Date: Dec 2024

To report issues or problems with this guidance contact: guidance.development@cyfoethnaturiolcymru.gov.uk

# WFD Compliance Assessment of Expansion of Carn Ar Wig & Porthlysgi IMTA Seaweed and Shellfish Farms

## Stage 3:  Detailed Assessment

Include all water bodies that could not be scoped out.

Stage 3 summary table.

| Water body name & ID | WFD element/s scoped in | Description of impacts; include a list of all evidence documents to inform the detailed assessment |
| --- | --- | --- |
| Pembrokeshire South GB611008590003 | Potential introduction of INNS | There is the potential for INNS to be introduced onto the farms when shellfish cultivated in other water bodies are transferred here. NEW INNS Biosecurity Risk Assessment and Management Plan has been used and will be implemented. |

Assessing for deterioration

Is there a risk the proposal may cause deterioration?

The farms are both IMTA sites with seaweed, mussels, scallops and oysters. The seaweed comes from a sterile hatchery environment when it is outplanted to the farms so there is no risk of INNS contamination.

There is potential for any of the 3 shellfish species to be bought in and grown on at both farm sites. This presents a potential risk for the introduction of INNS which could cause deterioration of the water body.

The infrastructure used for the new farm sites will either be new or reused from the current farms which are in the same water body and therefore construction or decommissioning of the farm infrastructure itself would not present any risks of INNS contamination.

Preventing the achievement of good status/potential

Pembrokeshire Marine SAC - full HRA and Baseline Benthic Habitat Survey Report are being included in this application to ensure any potential impacts are identified, assessed and mitigated. This will ensure the proposal will not compromise the SAC from achieving good status (or potential) now or in the future.

Mitigating for potential impacts

List any conditions, restrictions, or other measures, which could remove the risk of deterioration or prevent the water body achieving its objectives. Include details of how such measures would be applied, and who would be responsible for applying them.

If required, further details can be provided in separate clearly referenced documents.

| List avoidance measures | How will the measure be applied | Who will be responsible for applying them? | Will this remove the risk of deterioration or prevent the achievement of water body objectives. |
| --- | --- | --- | --- |
| Follow NRW INNS Biosecurity Risk Assessment and Management Plan | All farm staff will be trained on correct procedures listed in the Plan prior to any external shellfish being deployed on site | Bethany Marshall | Yes |
| All work vessels to be anti fouled | Once a year all work vessels removed from water and covered with anti fouling paint | Fergus Evans | Yes |
| Thorough inspection protocol to be followed | All shellfish to be cleaned and inspected on land before deployment to the farms | Fergus Evans | Yes |
| Ensure INNS protocols are followed and updated | Work closely with NRW Marine INNS Team and appointment of Biosecurity Officer | Bethany Marshall | Yes |

### Conclusion

Can the potential risk that your proposal may cause deterioration or prevent the water body and/or Protected Area from meeting its objectives be ruled out either alone or in combination?

Yes it can be ruled out. There are comprehensive and detailed mitigation measures put in place to ensure the spread of INNS is controlled as much as possible and correct protocols will be followed if any INNS are discovered on the farm.

Stage 3 decision summary table.

| Water body name & ID | WFD element/s scoped in | Is there a risk the proposal may cause deterioration?* ‘YES’ or ‘NO’ | Is there risk of the activity preventing the water body/ PA from meeting its objectives?* ‘YES’ or ‘NO’ |
| --- | --- | --- | --- |
| Pembrokeshire South GB611008590003 | Introduction of INNS | No | No |

* If yes or where there is uncertainty if the proposed activity may prevent a water body or PA from meeting its objectives or may cause deterioration then follow a precautionary approach and select yes.

| Detailed Assessment Completed by | Bethany Marshall, Marine Biologist | Date 07/06/2025 |
| --- | --- | --- |
| Detailed Assessment Reviewed by If applicable |  | Date |

Consultation with NRW and any other regulatory/advisory organisation

List all consultation and advice received

| Officer name and job title | Organisation | Date | Advice received |
| --- | --- | --- | --- |
| Colin Charman- Marine Fisheries Advisor | NRW | Ongoing | Ongoing advice on Baseline Benthic Habitat Survey Report |
| Alexander Scorey- Lead Specialist Advisor | NRW | Ongoing | Ongoing advice on Baseline Benthic Habitat Survey Report |

### NRW contact details

Call us on 0300 065 3000 (Mon-Fri, 9am-5pm). We welcome calls in Welsh or English.

Send us a message on enquiries@naturalresourceswales.gov.uk

Or complete the contact us form