| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-028923 |
| Permit Reference Number: | EPR- AB3891CX |
| Permit Holder: | Mr Daniel James and Mrs Carys James |
| Site Address: | Hafod Farm, Ferwig, Cardigan, Ceredigion, SA43 1PU |
| Grid Reference: | Farm location: SN 18094 50342 Fields: 4-B SN 17789 49959 5-B SN 17844 50084 6-B SN 17604 50225 7-B SN 17468 49980 8-B SN 17883 50437 9-B SN 18008 50535 8222-B SN 17826 50229 7233-B SN 17727 50338 5641-B SN 17563 50418 4956-B SN 17493 50566 HAFOD B/FRON 2849 SN 17301 50496 HAFOD B/FRON 3836 SN 17366 50337 BOLAFRON 4718 SN 17470 50166 BOLAFRON 3107 SN 17319 50053 BOLAFRON 6609 SN 17653 50093 Storage: SN 17505 49867 SN 17940 50095 SN 17956 50061 SN 18220 50263 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: GwastraffaGorfodi.Canolbarth@cyfoethnaturiolcymru.gov.uk Environment team: CeredigionEnvironmentTeam@cyfoethnaturiolcymru.gov.uk NLO: Malcolm.Dines@cyfoethnaturiolcymru.gov.uk |
| Date due: | 23/04/2025 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating B- 100% |

| Fundamental checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y |  |
| Correct fee? | Y | See permit compliance multiplier above and deployment fee- Charging Scheme Transaction ref: 2143097 Date: 03/04/2025 Ref: BS/030425/21/STEPSIDE AGRICULTUEPDEPSTEPS0136FP/WAA0035 Amount: 1,640.00 Transaction ref: 2143146 Date: 04/04/2025 Ref: BS/040425/20 STEPSIDE AGRICULTUEPDEPSTEPS0 Amount: 140.00 |
| Agricultural Benefit Statement | Y |  |
| Waste Analysis Report | Y |  |
| Soil Analysis Report | Y |  |
| Location Map | Y |  |
| Environmental Impact Assessment- Agricultural Regulations Evidence | Y |  |
| Risk assessment (if applicable) | n/a | Only if in high risk location |
| Landowner/occupier consent | Y |  |
| If confidentiality box has been ticked, no evidence provided | n/a | Only if confidentiality requested |
| Declaration authorisation | Y | Only if not signed by a relevant person |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| - About the Permit |  |  |
| ## Has there been any pre-application discussions? | N |  |
| ## Has the permit number been provided? | Y | EPR/AB3891CX |
| ## Has the permit holder name been provided? | Y | Mr Daniel James and Mrs Carys James |
| ## What permit is being deployed under? | SR2010 No4 |  |
| - Contact details |  |  |
| ## Has contact information been provided? | Y | Mr Robert Tucker |
| ## Has the technically competent manager details been provided? | Y | Mr Robert Tucker |
| ## Has the nominated competent person details been provided? | Y | Mr Daniel James |
| - About the land you want to treat |  |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | 55/226/0027 & 55/226/0026 4-B – 55/226/0026 Elizabeth Rachel James 5-B 55/226/0026 6-B 55/226/0026 7-B 55/226/0026 8-B 55/226/0027 Mrs J C Davies and Son 9-B 55/226/0027 8222-B 55/226/0026 7233-B 55/226/0026 5641-B 55/226/0026 4956-B 55/226/0026 HAFOD B/FRON 2849 55/226/0026 HAFOD B/FRON 3836 55/226/0026 BOLAFRON 4718 55/226/0026 BOLAFRON 3107 55/226/0026 BOLAFRON 6609 55/226/0026 |
| ## Has all of parcels of land table been completed? | Y |  |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y |  |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N | Single deployment |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | Y |  |
| ## Has written consent of all owners(s) or occupiers(s) been given? | Y |  |
| - Previous land treatment |  |  |
| Has all of the previous land treatment table been completed? | Y | PAN-025239 Live until 14/05/2025 |
| - Which risk band does the activity fall within? - SR2010No4 List A wastes (Lower risk) - Location risk - Low risk deployment - Medium risk (1) deployment - Medium risk (2) deployment - High risk deployment |  |  |
| ## Which risk band does the activity fall within? | Medium (2) |  |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) or High risk deployment has a SSRA been provided? | n/a |  |
| - Additional information on sensitive receptors |  |  |
| ## What type of risk assessment has been submitted? | site‐specific risk assessment benefit statement bespoke permit- site‐specific risk assessment |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | N/A |  |
| If no have details of the assessment evidence to support this conclusion been provided? | Y |  |
| - About the waste |  |  |
| Have all parts of the waste-types table been completed? - List of waste code (6 digit) - Description of waste (this must be an accurate description specific to this waste that refers to the List of Waste written description) - Physical form of waste/Is it stackable or non-stackable - Is the waste high in readily available nitrogen? - Producer of waste & permit number (if applicable) - Total amount being spread/used (tonnes) | Y |  |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y | Confirmed by own knowledge from previous deployments |
| - About the storage |  |  |
| Has all of waste storage table been completed? | Y |  |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y |  |
| 18. Supporting documents |  |  |
| Has checklist been completed? | Y |  |
| 20. Confidentiality and national security |  |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| 21. Declaration |  |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | n/a |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has the operator confirmed they will comply with the NRW pre-notification requirements. | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y |  |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no storage or spreading within 10 meters) (buffer zones needed) | Y | Water courses all noted by applicant and buffered |
| groundwater source protection zones (permit condition: no storage or spreading in SPZ1) (high risk deployment if in SPZ2) | n/a |  |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no storage or spreading within 50 meters) | n/a |  |
| location of all storage sites for the waste to be spread | Y |  |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | n/a |  |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | n/a |  |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | n/a | Nearest protected site over 600m away, no concern. |
| location of any public rights of way (shown on map) | n/a | None across site |
| any buildings or houses within 250m (shown on map) | Y |  |
| Within Groundwater Safeguard Zone | n/a |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Habitats - Section 7 species - Scheduled Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Hedges - Ditches | n/a | Purple moor grass and rush pastures bordering 4956-B – no concern. No protected species on site. |
| Flood risk | n/a | No flood risk to any of site from sea, rivers, or small water courses |
| What is the slope of the fields? . | 0-7 degrees (transparent)- all fields 7 -12 degrees (Yellow)- very small sections of fields #Hafod B/Fron 2849, 4956-B, Hafod B/From 3826, 9-B and the join between Bolafrom 3107 and 7-B. The areas of yellow are very small and are of no concern in regard to spreading. Please see mapping report for more specific information. 12 degrees plus (Red)- none |  |

| Waste Storage | Y N n/a | Comments |
| --- | --- | --- |
| Is waste storage occurring at the place where waste is being deployed? | Y |  |
| Is only waste as specified in this deployment being stored in the proposed storage locations? i.e. slurry in lagoons Note: There may be multiple deployments using one storage location | Y |  |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in the same place one constructed within the last two years? or in different location? | n/a |  |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in a field: - liable to flooding or becoming waterlogged, or - within 50 metres of a borehole, spring or well or - within 10 metres of surface water or a land drain - within 30 meters of a watercourse on steeply sloping land | n/a |  |
| If the waste is non-stackable is the waste stored within either a vessel, a covered building, or on an impermeable surface | Y |  |
| If the waste is high RAN and it is being stored within 200 metres of a European Site (SAC/SPA), Ramsar Site or a Site of Special Scientific Interest (SSSI) is the waste being covered to prevent loss of ammonia? (condition 3.2.4 of the permit) | n/a | Not high RAN, Not stored within 200m |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y | Potash requirements are over the requirement for the soils based on RB209, but there is an overall reduction when you take offtake into account. Confirm applicant has considered this based on the indices. |
| - Potassium (K) | Y | Potassium on DCWW ones -%w/w or mg/kg? RFI |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | Y | On two analyses |
| - Sulphur | Y | Query high sulphur additions from Bolton Hill and Preseli |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | Y |  |
| - If yes; have they been tested for? | Y | Volac and First Milk tested positive for e-coli – query operational techniques with operator as should leave 3 weeks between spreading and cutting. Sludges are effluent and not milk so additional TGN requirements for the code not needed. |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | N | Only on a few fields that show indices of 1. Please see RFI. |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y | Mr Robert Tucker |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y | To be queried, as above |
| - Timing of application | Y |  |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | n/a |  |
| - Soil physical properties (e.g. organic matter and soil structure) | n/a |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| Timing of application- if any of the wastes are high RAN, are they being spread OUTSIDE of the closed periods | n/a |  |
| Groundwater Safeguard Zone for nitrate- if any of the wastes are high RAN, are the application rates less than 50 m3/ha, with a minimum 3 week gap between applications | n/a |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | Y | Following RFI |
| - Included the nutrient level in their environmental risk assessment | Y |  |
| - Stated the source of information that they used to set the nutrient level. | Y |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y |  |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need, as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y N |  |
| - Waste analysis | Y N |  |
| - Crop nutrient requirements | Y N |  |
| If the cropping is any of the following, check if that the available Nitrogen applied is less than the below amount: - Field beans, Peas- 0 kg/ha - Sugar beet- 120 kg/ha - Asparagus, Carrots, Forage maize. Radish, Spring barley, Swedes- 150 kg/ha - Spring-sown wheat, Winter barley- 180 kg/ha - Autumn or early winter sown wheat- 220 kg/ha *Regulation 10 The Water Resources (Control of Agricultural Pollution) (Wales) Regulations 2021 | n/a | Only grass |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | n/a |  |
| - Is it only for operational purposes? | n/a |  |
| - Have individual application rates for each waste been given? | n/a |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | n/a |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y | April to May before first cut, then following silage cuts May-September |
| - Have they stated how often the waste will be spread? | Y | Up to 5 applications in total. |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients given by the applicant match NRW calculations? | Y |  |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y |  |
| - How much material was spread? | Y |  |
| - At what application rate material was spread? | N | Please seen PAN-025239 for further information |
| - The nutrients supplied by the material? |  |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | n/a |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | n/a |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | n/a |  |
| - Pathways | n/a |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | n/a |  |
| - Measures that will minimise or mitigate the impact will be put in place | n/a |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | N |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant Environment team. - If no, save the Appendix 4 to the DMS as information only | n/a |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site (project) within 500 meters of a European Site (SPA, pSPA, SAC or cSAC) or a Ramsar site? | No- there is no conceivable impact pathway between the project and any European site features, due to the location of the project. |

Comments

Consultation responses: - No comments that require additional assessment. Please see above sections of decision document/

MyMap/location and slope assessments: MyMap/location assessment Please see location section of decision document and maps/plans saved on DMS. Slope assessment All fields are completely flat minus a few very very small areas (please see saved maps). Slope is of no concern for this particular deployment.

CPH details: Please see CPH section above.

Protection of habitats and ecosystems Please see mapping section above. No concerns.

Agricultural benefit calculations: Please see above, RFI and nutrient calculator.

Request for information: 1. The analyses completed by DCWW show potassium values as mg/kg but we believe this is a typo, and they could be %w/w. Confirmed, no concern. 2. Wastes for Bolton Hill and Preseli have significantly more than the advisable amount of sulphur additions. Section 3 of RB209 recommends that soils at risk of sulphur deficiency require 40kg/ha of SO3 before each cut of silage, but the Bolton Hill waste stream will be adding 226.25kg/ha of sulphur, and the Preseli waste stream will be adding 148.82kg/ha, which is significantly higher than the requirement for these silage fields. RB209 works off available sulphur (10% availability of the total), therefore no concern. 3. Fields 4-B and 8-B are at index 3 for phosphorus, potassium and magnesium, and as such do not require any additional key nutrients as are already above the recommended levels for each nutrient. Spreading additional nutrients on these fields may be classed as disposal. Phosphate levels are not expected to increase or decrease from the application (slight increase on nutrient calculator due to rounding differences). Potash will be reduced overall from the application. Increase in magnesium levels not anticipated as will largely be removed from the waste by the crops. Sulphur, sodium, organic matter and trace elements should benefit the soils. Well explained and no further concern. 4. Fields 5-B, 6-B, 7-B, 9-B, 8222-B. 7233-B and 5641-B all have indices of 2 or above, and as such do not require additional key nutrients. Phosphate at 80% availability and total in waste doesn’t increase total in soils, as above. Same regarding potash and magnesium. No concerns. 5. Waste streams from Volac/Sensient and First Milk have both tested positive for e-coli. Your ABS does not refer to this or give any information on how you intend to manage the associated risks. Levels are very low and fields won’t be cut for at least 3 weeks rollowing application, as per guidance. No concern. 6. Current deployment hasn’t expired. Confirmed.

Other information to note: N/A

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date |  |
| ## Effective date | 15/05/2025 |
| ## Permitting Officer Date | Huw Davies 06/05/2025 |
| ## Team leader or Transfer of Powers (ToP) approval Date |  |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 600 |
| Total | 660 |