| Permit Number: | PAN-006496 |
| --- | --- |
| Applicant name: | Mr Paul Lloyd |
| Site Address: | Numbers 1-3 St Cadocs Close, Aberdare Road, Glyneath, SA11 5AQ |
| Discharge Point NGR: | SN 88905 06777 |
| Respond by date: | 18/11/2019 (10 working days) |
| Permitting Officer: | Elliot Burge |

Brief Description of proposal:

The application is for a ‘new’ discharge (began in 2014) to ground of secondary treated sewage effluent. The treatment system comprises of a package treatment plant and infiltration system, both conform to the relevant British Standards. The system serves 3 domestic properties and the maximum daily discharge volume is 2.25m3/day

Habitats Screening Report – Designations or protected species/habitats:

The risk screening report can be found on the DMS here.

There are no Habitats designations or protected species/habitats within the relevant screening distances. The only designation relevant to permitting identified is the Welsh Water Sewer catchment, more information can be found regarding this in the ‘Miscellaneous’ section found below.

Risk Assessment Results (modelling or L1 Risk Assessment):

A L1 Risk Assessment has been completed and saved onto the DMS here.

The score is 33 despite the fast drainage rate. Looking at the sensitivity analysis tab the low score is due to the secondary treatment (package treatment plant) and favourable geological conditions at the site.

The percolation value is <15, this means the infiltration system is undersized as any value under 15 means the field should be constructed to a vp of 15. The infiltration system should therefore be 45m3. The following condition will be in the permit:

‘The drainage field shall be designed in accordance with the British Standard BS 6297:2007+A1:2008 ‘Code of practice for the design and installation of drainage fields for use in wastewater treatment’. (All following references to ‘the British Standard’ are references to this document).

Where the minimum British Standard percolation test value (Vp) of 15 as referred in section 6.2 of the British Standard cannot be complied with the discharge is permissible only if the following additional requirements apply:

i.	the minimum floor area of the drainage field shall be calculated using a Vp equal to 15; and

ii.   a minimum 700 mm deep layer of medium or coarse washed sand shall be laid on a geotextile membrane, below the granular fill distribution layer as referred to in section 6.6, figures 4 and 5 of the British Standard.’

There will also be an improvement condition to upgrade the size of the infiltration system.

Miscellaneous:

The site is within distance of a Welsh Water foul sewer. However, the applicant has demonstrated that it is unreasonable to connect to the foul sewer based on cost, hence we are proceeding with the permit application. A cost benefit analysis has been undertaken and saved on the DMS here. The analysis has been undertaken in accordance with OI 538_06.

NRM Responses:

Environment Management comments:

Historically we have had problems with permit compliance for permits issued in the name of the developer, especially for multiple properties.  I am not sure who Paul Lloyd (the applicant) is, but it would be preferable for the permit to be issued in the name of an entity (eg a business) that can become the responsibility of the people who own the houses

Officers name: Selby Le Roux
Date:5/11/19

Biodiversity comments:

N/A

Officers name:
Date:

Fisheries comments:

N/A

Officers name:
Date:

Conservation/Ecology Comments:

N/A

Officers name:
Date:

Internal Consultation Responses from other NRW Teams:

Water Quality Planning:

N/A

Officers name:

Date:

Geoscience Response:

N/A

Officers name:
Date:

Flood Risk Management:

N/A

Officers name:
Date:

| External Consultations sent to: | Yes/No |
| --- | --- |
| National Parks | N |
| Water Company regarding Drinking Water Protected Zone / SPZ1 | N |
| Environmental Heath | N |

Any Permitting Responses:

The developer will not be the operator of the permit, I have spoken to Paul Lloyd (who owns one of the houses) who is happy to assume sole responsibility of the maintenance of the PTP, whilst the other houses are unoccupied. The permit will be transferred to a management company (inclusive of all homeowners), once the properties have been purchased.

Date: 05/12/2019