## Annex (iv) – SSSI Assessment Form

| # SSSI Assessment for permit/licence and deployment applications |  |
| --- | --- |
| To be completed by Permitting Officers for any applications for a permission which Natural Resources Wales has considered under S28I duty to notify SNCB and take their advice into account. This applies to all proposed permissions within a SSSI, and to operations outside the SSSI boundary which are likely to damage its special features. NRW as a Section 28G authority has, when exercising its functions, a general duty to take reasonable steps, consistent with the proper exercise of its functions, to further the conservation and enhancement of the flora, fauna or geological or physiological features by reason of which the site is of special interest’ Part 1 – SSSI Assessment Part 2 – Formal notification to SNCB (to be completed if part 1 assessment concludes likely damage, not likely to damage because of conditions or you cannot conclude no likely damage) Part 3 – Decision |  |
| Pink text – drafting notes, to be deleted before completion/consultation. Blue text – examples, to be replaced with permission-specific information and text turned to black. |  |

Part 1 – SSSI Assessment

| - Permitting officer/team | Megan Tucker Waste permitting team |  |
| --- | --- | --- |
| - Permit application reference and site name |  |  |
| - a. SSSI name(s) b. location c. NRW Operational Area/Environment Team | a. Cors Bodwrog SSSI b. SH405771 c. Mon ac Arfon |  |
| - Brief description of proposal | A land spreading application for 2 waste streams, waste codes, 19 05 99 at The Bodwina farm. Wastes are to be spread on fields planted for grass crop. The application rates range from 54 and 250 tonnes per ha, with a total tonnage to spread 11100 tonnes. The waste will be spread over 50 hectares of land within the listed fields at Bodwina Farm. The liquid sludges are delivered by HGV road tankers which are discharged into a lagoon or nurse tank prior to spreading or spread on delivery. Liquid wastes are spread onto the fields by shallow disc injections. The fields will be spread in February-August on grass and prior to grazing in the summer. No more than 50t/ha of liquid sludge will be spread on a field in any 3-week period in accordance with CoGAP, and no more than 250t/ha will be spread within any 12-month period. Wastes will be applied when ground and weather conditions are suitable, following CoGAP to avoid soil damage including wheel ruts, compaction, structural damage, erosion and run-off. The SSSI is over 260m from the nearest spreading area. |  |
| - What aspects of the proposed permission are likely to damage the SSSI features of special interest? The following activities are likely to cause damage: Application of manure, fertilisers and lime. Dumping, spreading or discharging of any materials. Storage of materials. Use of vehicles likely to damage the fen vegetation or craft likely to disturb birds The following SSSI(s) features and potential impacts have been considered to assess the likelihood of damage: Cors Bodwrog has been selected for its biological interest as an example of mesotrophic wetland with small areas of alkaline (lime-rich) fen. Impacts The nearest field proposed to be spread is 260m away from the SSSI boundary and as such there are very limited impact pathways that the proposal could have on the SSSI. The sludges are being applied via precise shallow disk injection, with minimal sloping within the fields so run off risk is low. All watercourse have buffer zones implemented to minimise risk of any waste entering a water stream and having potential to effect the SSSI. |  |  |
| - Summary of any informal advice received from internal experts (if required and including pre-app advice) | N/A |  |
| - Recommendation | The proposed permission is not likely to damage any of the flora, fauna or geological or physiological features which are of special interest - The distance from the nearest spreading area to the closest field (field 5) is 260m. Risk is very minimal due to the physical barriers between the site and spreading areas. - All watercourse which may be hydrologically connected to the site have had buffer zones imposed to reduce run off risk. - Wastes will not be spread in high winds to avoid and dust or particle matter from reaching SSSI although wastes have low potential to produce dust. - Spreading will follow CoGAP - No more than 50t/ha of waste will be spread in any 3 week period. |  |
| - Signature and date assessment made | M Tucker 30/09/2025 |  |
| - Officers name and job title | Megan Tucker Waste permitting officer |  |