# Natural Resources Wales permitting decisions

# Bespoke permit

We have decided to grant the permit for Properties 1 -11, Rhos-y-Brenin operated by Rhos YB Management Ltd.

The application number is PAN-027521.

The permit number is EPR/DB3598HJ.

This permit regulates a new discharge of a total 10.5 m3/day domestic sewage effluent from 11 houses. Each property is serviced with its own package treatment plant and additional tertiary treatment, performing better than British Standard BS 12566. The treatment systems at 1 to 6 are connected via a communal sewage pipe that discharges through to Effluent sample point 1 and properties 7 to 11 join are connected by a pipe that passes through Effluent Sample Point 2. Both these pipes then join a single communal sewage discharge pipe whereby the discharges are pumped across a field before discharging via a single discharge point to the Nant Lletys, a tributary of the Afon Rheidol.

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

Purpose of this document

This decision document:

- explains how the application has been determined

- provides a record of the decision-making process

- shows how all relevant factors have been taken into account

- justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

Structure of this document

- Key issues

- Annex 1 the decision checklist

# Key issues of the decision

This permit has been granted against the advice of ANCB following legal advice based upon best available data that there is no evidence of a direct causative link between nutrient enrichment in the stream from the proposed discharge (as the levels indicated by the tertiary treatment) and an impact on site integrity as informed by the Conservation Objectives for the SAC.

With the agreement of the Service Manager of local ANCB team, it was agreed that a permit would be granted for this application without the need for further escalation in accordance with OGN200 and OGN231. See link to email here >> PAN-027521 Confirmation that Escalation is no longer needed

The permit has been issued with tight emission limits for Ammonia and Phosphorus in accordance with the performance of the Tertiary Treatment Plant (TTP). In addition, a pre-operational condition will be added to ensure the TTP is in place before discharging commences in accordance with the permit.

# Annex 1: decision checklist

This checklist should be read in conjunction with the Duly Making checklist.

| ### Aspect | ### Justification / Detail | Criteria met |
| --- | --- | --- |
|  |  | Yes |
| Consultation |  |  |  |
| Scope of consultation | The consultation requirements were identified and implemented. The decision was taken in accordance with RGN 6 High Profile Sites, our Public Participation Statement and our Working Together Agreements. | ü |
| ### Responses to web publicising | There were no responses received from web publicising. The decision was taken in accordance with our guidance. | ü |
| Operator |  |  |  |
| Control of the facility | We are satisfied that the applicant (now the operator) is the person who will have control over the operation of the facility after the grant of the permit. The decision was taken in accordance with EPR RGN 1 Understanding the meaning of operator. | ü |
| European Directives |  |  |  |
| Applicable directives | All applicable European directives have been considered in the determination of the application. | ü |
| WFD | In light of the conclusions of a detailed compliance assessment (Stage 3), and taking account of the advice received from technical specialist advisors, it has been established that the activity/project has no potential to cause deterioration of any water body or prevent a water body or WFD Protected Area from meeting its objectives, taking into account any legally binding avoidance measures or restrictions as applicable, either alone or in-combination with other activities. | ü |
| The site |  |  |  |
| Extent of the site of the facility | The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility including discharge points. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary. | ü |
| Biodiversity, Heritage, Landscape and Nature Conservation and a designated site under the Wildlife and Countryside Act. | The application is within the relevant distance criteria of a site of heritage, landscape or nature conservation, and/or protected species or habitat . A full assessment of the application and its potential to affect the site/species/habitat has been carried out as part of the permitting process. We consider that the application will affect the features of the site/species/habitat. Operational controls/emission limits have been placed on the permit to mitigate against these effects : - Operational controls :- Before a property is brought online and a discharge made, a tertiary treatment system shall be installed at each property which achieve the limits as specified below. Emission limits: - - ATU-BOD as 02 – 20 mg/l (max) • Ammoniacal Nitrogen - 1.1 mg/l (Max) • Phosphorus - 0.2 mg/l (mean) • Suspended Solids (measure after drying at 105oC) – 20mg/l (max) We have not formally consulted externally on the application. The decision was taken in accordance with our guidance. Consultation with the internal local Environment Team for the SSSI has been conducted under Section 28I of the Wildlife and Countryside Act, as inline with OGN 231. The consultation response disagreed with permitting’s position of no likely significant effect upon the special features of the Coedydd a Cheunant Rheidol SSSI: the Bryophytes and objected to issuing the permit based on revised discharge limits to be imposed on the applicant. Following OGN231, the case was escalated through the appropriate management levels with final decision passed to senior service level management. PAN-027521_Escalation_paper_final Notification of intent to issue the permit against ANCB advice was sent to the ANCB on 26/08/2025, meeting the requirement of notification stipulated in Wildlife and Countryside Act, Section 28I(6). PAN-027521 Escalation in accordance with OGN231 and OGN 200 Following legal advice and based upon best available data, there is no evidence of a direct causative link between nutrient enrichment in the stream from the proposed discharge (as the levels indicated by the tertiary treatment) and an impact on site integrity. PAN-027521 legal response re likelihood of appeal After extensive discussion involving senior level management and following internal legal advice the need to escalate final decision to head of Evidence, Policy and Permitting was not required (PAN-027521_confirmation_escalation_not_needed). The decision was made to progress with issuing of the permit, as the permitting processes lead to a clear and positive determination, although the ANCB advice remains the same. The decision to progress is based on NRW’s current guidance/process and approaches using established assessment and tools (e.g. RQP modelling/WFD compliance) to conclude no likely significant effect maintaining existing water quality status in the receiving water course at point of discharge, 330m upstream of the designated SSSI site boundary. The permit will have bespoke limits on the discharge restricting levels of Ammonia, Biological Oxygen Demand, Phosphorus and Suspended Solids. For clarity, this determination was made in compliance with existing NRW policy/guidance and positions at time of determination. | ü |
| Habitat Regulation Assessment for a designated site under the EU Habitats Directive | Appropriate assessment: In light of the conclusions of an appropriate assessment and taking account of the advice received from protected sites advisors, it has been established that the project will not adversely affect the integrity of any Natura 2000/Ramsar site, taking into account any conditions or restrictions as applicable, either alone or in-combination with other plans and projects. (As documented in section 4 of OGN 200 form 1, and section 5 if applicable) The consultation response disagreed with permitting’s position of no likely significant effect upon the special features of the Coedydd a Cheunant Rheidol SAC and objected to issuing the permit based on revised discharge limits to be imposed on the applicant. Following OGN200, the case was escalated through the appropriate management levels with final decision passed to senior service level management. PAN-027521_Escalation_paper_final Notification of intent to issue the permit against ANCB advice was sent to the ANCB on 26/08/2025, meeting the requirement of notification stipulated in OGN 200, PAN-027521 Escalation in accordance with OGN231 and OGN 200 Following legal advice and based upon best available data, there is no evidence of a direct causative link between nutrient enrichment in the stream from the proposed discharge (as the levels indicated by the tertiary treatment) and an impact on site integrity. PAN-027521 legal response re likelihood of appeal After extensive discussion involving senior level management and following internal legal advice the need to escalate final decision to head of Evidence, Policy an Permitting was not required (PAN-027521_confirmation_escalation_not_needed). The decision was made to progress with issuing of the permit, as the permitting processes lead to a clear and positive determination, although the ANCB advice remains the same. The decision to progress is based on NRW’s current guidance/process and approaches using established assessment and tools (e.g. RQP modelling/WFD compliance) to conclude no likely significant effect maintaining existing water quality status in the receiving water course at point of discharge. The permit will have bespoke limits on the discharge restricting levels of Ammonia, Biological Oxygen Demand, Phosphorus and Suspended Solids. For clarity, all permitting decisions are made in compliance with existing NRW policy/guidance and positions at time of determination. Should/when amendments be formally agreed via appropriate governance, future permitting determinations will be made in line with the revised policy/guidance and/or positions at that time. | ü |
| Environmental Risk Assessment and operating techniques |  |  |  |
| Environmental risk | We have carried out a risk assessment on behalf of the operator. The operator considers this risk assessment is satisfactory. The assessment shows that, applying the conservative criteria in our guidance on Environmental Risk Assessment, all emissions may be categorised as environmentally insignificant with the exception of Ammonia and Phosphorous within the discharge. These will be controlled via a maximum emission limit for Ammonia and mean for Phosphorus set within the permit, as described emissions limit section of this document. | ü |
| Operating techniques | We have reviewed the techniques used by the operator and compared these with the relevant guidance notes “How to Comply with Your Environmental Permit, TGN 7.01”. The proposed techniques / emission levels for priorities for control are in line with the benchmark levels contained in the TGN and we consider them to represent appropriate techniques for the facility. | ü |
| The permit conditions |  |  |  |
| Pre-operational conditions | Based on the information on the application, we consider that we need to impose pre-operational conditions. We have imposed a condition for the applicant to install tertiary treatment plants to each treatment system before discharges commence. This is to ensure that permit limits for Ammonia and Phosphorus are not exceeded in order to protect the receiving watercourse. | ü |
| Emission limits | We have decided that emission limits should be set for the parameters listed in the permit. It is considered that the numeric limits described below will prevent significant deterioration of receiving waters. • ATU-BOD as 02 – 20 mg/l (max) • Ammoniacal Nitrogen - 1.1 mg/l (Max) • Phosphorus - 0.2 mg/l (mean) • Suspended Solids (measure after drying at 105oC) – 20mg/l (max) We have imposed numeric limits because either a relevant environmental quality or operational standard requires this. Ammonia and Phosphorus limits have been imposed to ensure no deterioration of the receiving watercourse: the Nant Lletys. These limits along with a limit for BOD limit have been set in order to maintain the current WFD class and in-line with HAB Regs, ensuring there is no deterioration of the special features of the SAC and SSSI. A limit of Suspended Solids has been set as additional protection of the receiving watercourse. | ü |
| Considerations of foul sewer | We agree with the operator’s justification for not connecting to foul sewer. A cost benefit analysis has been completed in line with current operational guidance notes. A connection to foul sewer is not considered technically achievable. The applicant provided an email from DCWW dated 21/10/2024 with their application >> PAN-027521 letter from DCWW. The email stated that connection to foul sewer was over 100m away from the property boundary and technically unfeasible to connect due to available space, cost and topography. This decision has considered all aspects as detailed in the operational instruction 538_06. | ü |
| Operator Competence |  |  |  |
| Environment Management System | There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. | ü |
| Sustainable Management of Natural Resources (SMNR) |  |  |
| Considerations of SMNR - Compliance with our General Purpose | We are satisfied that this decision is compatible with our general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources.” Although the discharge is within a sewered area the quality of effluent produced by the tertiary treatment system is of higher quality than that from existing Dŵr Cymru Welsh Water Treatment Works (DCWW WwTW). The DCWW discharge is regulated under permit: BP0230101 which is descriptive. The DCWW WwTW permit allows higher levels of Ammonia and BOD to be discharged as it is descriptive and it does not have any Phosphorus control. Therefore, by issuing this permit we can actively reduce the levels of Ammonia, BOD and Phosphorus discharged, ensuring that our duty to safeguard the watercourse and its status is met. | ü |
| Environment Wales Act 2016 |  |  |
| Biodiversity and resilience of ecosystems duty | Section 6 of the Environment Wales Act 2016 requires that we seek to maintain and enhance biodiversity in the exercise of our functions, and in so doing promote the resilience of ecosystems, in a manner that is consistent with the proper exercise of our functions. NRW is satisfied that in this case, we have taken into account and had due regard to this duty in so far as it is consistent with the function of determining an application for a stand alone water discharge activity permit under EPR2016. | ü |