PAN-030537  Mapping report:

Application maps

Deployment Boundaries and storage

Detailed river network 10m

Watercourses between 7849, 2 and 0252, also to the S boundary of 1561. Buffered on application map.

Pond within 1. Buffered on application map.

Pond and watercourse to the SE of 3, buffered on application map

None at other fields

Groundwater source protection zones

none

Any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies within 50m.

Issue NW of 6210 buffered on application map

Issue between 7849 and 2, buffered on application map

None at other fields.

Any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies within 250m.

Issue E of 6210, shown on application map

Spring S of 0252, shown on application map

None at other feilds

Within Groundwater Safeguard Zone

none

European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m

none

Other protected sites within 500m:

- Protected Species, section 7-

Several within range mainly hedgehogs recorded in 2021/22

- Protected Habitats-

Reedbeds adjacent pond within 1, buffered on application map

Lowland dry acid grassland and Lowland calcareous grassland and Purple moor grass and rush pastures within 500metres. Nil risk due to waste type and delivery method

- Scheduled Ancient Monument- none

- Local Nature Reserve- none

- National Nature Reserve- none

- Ancient Woodlands- none

- Local Wildlife Sites (SINC)- none

- National Park- none

- Nitrate Vulnerable Zone (NVZ)- ALL Wales

- Groundwater Safeguard Zone- none

Flood risk

none

Public rights of way

All footpaths within fields are shown on application maps

Distance from Main Farm Address

Adjacent

Slope Map

All fields 0-7

END