| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-030538 |
| Permit Reference Number: | EPR-JB3533RJ |
| Permit Holder: | Simon Jones |
| Site Address: | Sarnfadog, Llanerchymedd, LL71 8ER |
| Grid Reference: | Farm location: SH 42073 81090 Fields: 528 SH 42069 81299 2928 SH 42285 81286 2952 SH 42288 81523 3171 SH 42301 81724 2949 SH 42509 81491 4726 SH 42470 81296 5111 SH 46485 91134 4300 SH 46438 90997 7291 SH 46735 90924 8570 SH 46856 90710 9685 SH 46967 90865 909 SH 47087 91083 1794 SH 47175 90933 Storage: SH 42073 81013 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: IndustryRegulation&Waste.NW@cyfoethnaturiolcymru.gov.uk Environment team: TimAmgylcheddMonacArfon@cyfoethnaturiolcymru.gov.uk NLO: IndustryRegulation&Waste.NW@cyfoethnaturiolcymru.gov.uk |
| Date due: | 14/10/2025 |
| Are fields on Anglesey? If yes, then send application. | Yes: ajxht@anglesey.gov.uk pem@anglesey.gov.uk |
| Permit compliance rating and multiplier | Rating A- 100% |

| Fundamental checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | Welsh- PaLS |
| Correct fee? | N | See permit compliance multiplier above and deployment fee- Charging Scheme Transaction number: 2148873 Date: 01/10/2025 Ref on MyNRW: BS/01102025/36 GBP FASTER PAYMENTSL- 58558583 -2909 WPCC16056 SR JO Amount: £2,093 Applicant has paid a high risk fee a refund is required as this is a medium risk deployment. Should have paid: £1,780 Refund = £313 |
| Agricultural Benefit Statement | Y |  |
| Waste Analysis Report | Y |  |
| Soil Analysis Report | Y |  |
| Location Map | Y |  |
| Environmental Impact Assessment- Agricultural Regulations Evidence | Y | Within spreading maps |
| Risk assessment (if applicable) | Y | Only if in high risk location |
| Landowner/occupier consent | Y |  |
| If confidentiality box has been ticked, no evidence provided | n/a | Only if confidentiality requested |
| Declaration authorisation | Y | Only if not signed by a relevant person |

| Saving documents- by determining permitting officer | Y N |
| --- | --- |
| Save embedded application documents from application form | Y |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| - About the Permit |  |  |
| ## Has there been any pre-application discussions? | N |  |
| ## Has the permit number been provided? | Y |  |
| ## Has the permit holder name been provided? | Y |  |
| ## What permit is being deployed under? | SR2010 No4 |  |
| - Contact details |  |  |
| ## Has contact information been provided? | Y |  |
| ## Has the technically competent manager details been provided? | Y |  |
| ## Has the nominated competent person details been provided? | Y |  |
| - About the land you want to treat |  |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | See comments in CPH details at the bottom of the decision document. Consent from 2022. Does not include wastes to be spread on each field or storage locations. |
| ## Has all of parcels of land table been completed? | Y |  |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y |  |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | Y | See comments in CPH |
| ## Has written consent of all owners(s) or occupiers(s) been given? | Y | See comments in CPH |
| - Previous land treatment |  |  |
| Has all of the previous land treatment table been completed? | n/a |  |
| - Which risk band does the activity fall within? - Medium risk deployment |  |  |
| ## Which risk band does the activity fall within? | Medium |  |
| Is the risk banding correct? | N | Not high risk deployment. This is a medium risk. |
| ## If Medium risk (2) or High risk deployment has a SSRA been provided? | n/a |  |
| - Additional information on sensitive receptors |  |  |
| ## What type of risk assessment has been submitted? | Benefit statement |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | n/a |  |
| If no have details of the assessment evidence to support this conclusion been provided? | Y |  |
| - About the waste |  |  |
| Have all parts of the waste-types table been completed? - List of waste code (6 digit) - Description of waste (this must be an accurate description specific to this waste that refers to the List of Waste written description) - Physical form of waste/Is it stackable or non-stackable Is the waste high in readily available nitrogen? - Producer of waste & permit number (if applicable) - Total amount being spread/used (tonnes) | N | Wash water high RAN and online form states this as no. |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y |  |
| - About the storage |  |  |
| Has all of waste storage table been completed? | Y |  |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y |  |
| 18. Supporting documents |  |  |
| Has checklist been completed? | Y |  |
| 20. Confidentiality and national security |  |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| 21. Declaration |  |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | Y |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has the operator confirmed they will comply with the NRW pre-notification requirements. | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y |  |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no storage or spreading within 10 meters) (buffer zones needed) | Y |  |
| groundwater source protection zones (permit condition: no storage or spreading in SPZ1) (high risk deployment if in SPZ2) | n/a |  |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no storage or spreading within 50 meters) | N | Buffered yes, size of buffer unknown , rfi |
| location of all storage sites for the waste to be spread | Y |  |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | N | See above comment |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | n/a |  |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | n/a |  |
| location of any public rights of way (shown on map) | Y |  |
| any buildings or houses within 250m (shown on map) | Y |  |
| Within Groundwater Safeguard Zone | n/a |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Habitats - Section 7 species - Scheduled Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Hedges - Ditches | Y |  |
| Flood risk | Y | Minor flood areas in fields 2928, 2952 & 3171 has been addressed via non spread buffer zones along the watercourse which borders these fields. |
| What is the slope of the fields? | 0-7 degrees (transparent)- Fields 5111, 8570, 1794, 0909, 0528, 2928, 4726, 2949, 2952 & 3171 7 -12 degrees (Yellow)- Fields 4300, and 9685 12 degrees plus (Red)- Field 7291 |  |

| Waste Storage | Y N n/a | Comments |
| --- | --- | --- |
| Is waste storage occurring at the place where waste is being deployed? | Y |  |
| Is only waste as specified in this deployment being stored in the proposed storage locations? i.e. slurry in lagoons Note: There may be multiple deployments using one storage location | N |  |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in the same place one constructed within the last two years? or in different location? | n/a | Non stackable wastes. |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in a field: - liable to flooding or becoming waterlogged, or - within 50 metres of a borehole, spring or well or - within 10 metres of surface water or a land drain - within 30 meters of a watercourse on steeply sloping land | n/a |  |
| If the waste is non-stackable is the waste stored within either a vessel, a covered building, or on an impermeable surface | N |  |
| If the waste is high RAN and it is being stored within 200 metres of a European Site (SAC/SPA), Ramsar Site or a Site of Special Scientific Interest (SSSI) is the waste being covered to prevent loss of ammonia? (condition 3.2.4 of the permit) | n/a | Not stored within 200m. |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | Y |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | Y |  |
| - If yes; have they been tested for? | Y | Due to the odorous nature of this waste it is likely that a site specific risk assessment will be required for deployments of this waste |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y | Maintenance dressing for the majority of the fields. Mg justification is needed for fields with a mg indices at 3. |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y |  |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | Y |  |
| - Soil physical properties (e.g. organic matter and soil structure) | Y |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| Timing of application- if any of the wastes are high RAN, are they being spread OUTSIDE of the closed periods | N | ABS has not identified high ran wastes not clear therefore not site spefic. |
| Groundwater Safeguard Zone for nitrate- if any of the wastes are high RAN, are the application rates less than 50 m3/ha, with a minimum 3 week gap between applications | n/a | Not in groundwater safeguard zone |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | n/a | Nutrients are being applied for with rb209 guidelines for each soil indices value for P and K. However Mg justification is needed for fields with a mg indices at 3. This is missing from the application. |
| - Included the nutrient level in their environmental risk assessment | n/a |  |
| - Stated the source of information that they used to set the nutrient level. | n/a |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | N | More assessment needed on odour. |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need, as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If the cropping is any of the following, check if that the available Nitrogen applied is less than the below amount: - Field beans, Peas- 0 kg/ha - Sugar beet- 120 kg/ha - Asparagus, Carrots, Forage maize. Radish, Spring barley, Swedes- 150 kg/ha - Spring-sown wheat, Winter barley- 180 kg/ha - Autumn or early winter sown wheat- 220 kg/ha *Regulation 10 The Water Resources (Control of Agricultural Pollution) (Wales) Regulations 2021 | n/a | grass |
| Nutrient assessments for P- If land is above target index (3+ for arable, grassland, and forage crops. 4+ for vegetables) has a net reduction in P levels across the crop rotation been demonstrated? | N | This is missing from the application. |
| Nutrient assessments for P- if the above has been agreed for a previous deployment, is it still correct for this this application? | n/a | no previous deployment within last 12 months. |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | Y |  |
| - Is it only for operational purposes? | Y |  |
| - Have individual application rates for each waste been given? | Y |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | N |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients given by the applicant match NRW calculations? | Y |  |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | n/a |  |
| - How much material was spread? | n/a |  |
| - At what application rate material was spread? | n/a |  |
| - The nutrients supplied by the material? | n/a |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | n/a |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | n/a |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | n/a |  |
| - Pathways | n/a |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | n/a |  |
| - Measures that will minimise or mitigate the impact will be put in place | n/a |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | N |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant Environment team. - If no, save the Appendix 4 to the DMS as information only | n/a |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site (project) within 500 meters of a European Site (SPA, pSPA, SAC or cSAC) or a Ramsar site? | No |

Comments

Consultation responses: The comments left on the form were taken into account for the determination of this deployment as general standard practise of complying with SR2010no4.

MyMap/location and slope assessments: MyMap/location assessment For details see the Mymap report on the DMS. Slope assessment Field number: 7291 Slope of the land: Partially highly sloping land 12o+ Soil type: Medium clay Waste types applied: Liquid non stackable Cropping type: Grass Method of application: Trailing shoe Weather conditions before during and after spreading: In compliance with the weather restrictions outlined in CoGAP. Conclusions (is run off risk high?): Considering compliance with SR210no4 by with the correct buffer zones in place and suitable application method of a trailing shoe that applies the waste directly onto the grass the runoff risk of applying this wastes has been determined as low to surrounding water courses. Only small areas are highly sloping in field 7291 and is contained within the field. Field number: 4300, 7291 and 9685 Slope of the land: moderately sloping land (7o-12o) Soil type: Medium clay Wastes types applied: Liquid non stackable Cropping type: Grass Method of application: Trailing shoe Weather conditions before during and after spreading: In compliance with the weather restrictions outlined in CoGAP. Conclusions (is run off risk high?): Considering compliance with SR210no4 by with the correct buffer zones in place and suitable application method of a trailing shoe that applies the waste directly onto the grass the runoff risk of applying this wastes has been determined as low to surrounding water courses. Fields 5111, 8570, 1794, 0909, 0528, 2928, 4726, 2949, 2952 & 3171 are located on flat to gently sloping land where the runoff risk has been determined as low.

CPH details: CPH provided LPD1: 53/166/0005 CPH record Mymap: All fields – 53/011/0226 M AND R PRITCARD + SONS The Mymap CPH numbers differ from the CPH provided in the submitted deployment application. The provided consent form is dated to 2022 and not 2025 and is therefore not specific to this deployment. The consent form is missing the waste to be spread on each field along with storage locations. As this would have required a new signed CPH consent form this would be classified as reassessment with third party input. For this reason the deployment was returned to the applicant.

Protection of habitats and ecosystems For details see the Mymap report on the DMS.

Agricultural benefit calculations:

Request for information: N/A

Other information to note: This deployment was returned for the following reasons: - CPH consent documents that were provided would require third party input and are incorrect. (dated to 2022 and 2023 and not the current year) & waste to be spread under the proposed deployment and the storage locations were not provided. A new CPH consent form would need to be provided and would constitute reassessment. - An odour management plan was not provided where the deployment proposed to spread EWC 02 05 01 & 02 05 02 which are odorous wastes, while undertaking the location screening numerous sensitive receptors were identified close to the spreading fields including a school.

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | No |
| ## Issue date | n/a |
| ## Effective date | n/a |
| ## Permitting Officer Date | Owen Clee 13/11/2025 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 17/11/2025 |

| Time recording |  |
| --- | --- |
| Admin/consultations (Permit support time) | 60 |
| Determination (waste permitting team time) | 1,800 |
| Total (to be added to PALS) | 1,860 |